Oregon Address Point

Data Standard

Version 0.3

December 2024

Revision History:

Original 0.1 Draft Written by: Tom Elder

Version 0.2 revised based on the Address Point Data Standard workgroup

Version 0.3 revised based on GIS community comment

Version 0.x revised based on Advisory Group comment

Endorsed by the Oregon Geographic Information Council [Month] [Year]

For questions about this data standard, contact:

Name:	Tom Elder

Email:	tom.elder@das.oregon.gov

Table of Contents

- Introduction

1.1		Mission and Goals of the Standard

1.2		Relationship to Existing Standards

1.3		Description of the Standard

1.4		Applicability and Intended Use of the Standard

1.5		Standard Development Procedures

1.5.1		Participants

1.5.2		Comment Opportunities and Reviews

1.6		Maintenance of the Standard

2.0	Body of the Standard

2.1		Scope and Content of the Standard

2.2		Need for the Standard

2.3		Participation in the Standard Development

2.4		Integration with Other Standards

2.5		Technical and Operational Context

2.5.1		Data Environment

2.5.2		Reference System

2.5.3		Integration of Themes

2.5.4		Encoding

2.5.5		Resolution

2.5.6		Accuracy

2.5.7		Edge Matching

2.5.8		Feature Identifier

2.5.9		Attributes

2.5.10		Transactional Updating

2.5.11		Records Management

2.5.12		Metadata

3.0	Data Characteristics

3.1		Spatial Data Elements

3.1.1		Address Point

3.2		Minimum Attribute or Non-Graphic Data Elements

3.2.1		Point

3.3		Other Graphic Data Elements

3.3.1		Point

3.4		Optional Attribute of Non-Graphic Data Elements

3.4.1		Point

References

Appendices

A.	Terms and Acronyms

B.	Workgroup Participants

C.	Implementation

1.0	Introduction

Under the direction of the Oregon Geographic Information Council (OGIC) and with the guidance of the Oregon Framework Program, the Addresses and Buildings Framework Implementation Team (FIT) was convened to create statewide data standards for address points and building footprints. This document is the result of collaboration and cooperation between many address point data providers (local jurisdictions and 9-1-1 Public Safety Answering Points), major address data consumers, and other stakeholders. The goal was to define a standard to help facilitate the gathering, combining, and distributing of address point data for the entire state that is reliable, accurate, complete, and timely.

There are approximately two million address points across Oregon. An address point usually refers to the location of a building or other structure, typically described using street names, house or apartment numbers, and other identifiers. Address points show the geographic location of individual houses, apartments, condominiums, mobile homes, offices, shops, schools, factories, farms, and  other places where people live, work, and gather.  Address point locations are indispensable for the efficient delivery of government services and the equitable allocation of public resources, helping to ensure representation and fairness and a better understanding of Oregon’s population at the highest level of spatial detail.

1.1	Mission and Goals of the Standard

Establishing an Oregon Address Point Data Standard is the first step to building and distributing a seamless statewide address point dataset.  Address points originate from, and are maintained by, local governments for many different purposes and in many different data formats. The Oregon Address Point Data Standard is designed to accommodate and align different formats in order to assemble a complete, accurate, and comprehensive dataset of all address points for the State of Oregon.

The primary goals of the Address Points Standard are -

- To ensure consistency and compatibility between address point data maintained by different jurisdictions for multiple uses within the state by providing common definitions and value ranges for address information to enhance data sharing.

- To ensure that all address points in the state can be accommodated down to the unit level of detail.

- To ensure that each address point is unique and not duplicated or confused with another address.

- To ensure that each address point is complete and includes all address elements that make each one unique.

- To support the broadest range of stakeholders and use cases.

The Oregon Address Point Data Standard is not intended to replace any jurisdiction’s local schema, internal data workflow, or storage specifications required for their operational needs.

The Oregon Address Point Standard establishes a unified set of attributes and domains for creating and maintaining an enterprise-level statewide address dataset. These attributes promote consistency in locally maintained address data and compatibility between local and statewide address data.

1.2	Relationship to Existing Standards

1.2.1	Existing National Address Standards

Four major national address standards were reviewed during the development of Oregon’s Address Points standard.

- FGDC	Federal Geospatial Data Committee, Numbered Thoroughfare address classification.

- NENA	National Emergency Number Association, Site Structure Address Point.

- NAD	National Address Database.

- USPS	United States Postal Service.

FGDC was the former Oregon standard adopted by the Oregon Geographic Information Council in 2014. However, it was never implemented and after review by the Oregon Address Point Workgroup was determined to no longer be the best fit for Oregon.

The NENA standard was selected as the core of the Oregon Address Point Data Standard for the following reasons:

- NENA aligns with existing FGDC and NAD standards.

- NENA is simpler to implement than FGDC.

- Many other states already use NENA.

- Most address point data is already available from OEM in NENA format.

- Coordination with the ongoing NG9-1-1 Technical Advisory Committee would reduce redundancies.

- NENA, although intended for public safety use cases, could accommodate other use cases and business requirements with supplemental fields.

The Oregon Address Points Data Standard closely follows the NENA Standard. 9-1-1 Public Safety Answering Points are currently providing address points to the Oregon Department of Emergency Management (OEM) in the NENA standard for public safety purposes.  The Oregon Address Point workgroup determined modifications to the NENA standard could transform a single-use data set into an enterprise address point database that could support a variety of use-cases. The Oregon Address Point workgroup has determined that with a few modifications, the nationally recognized and locally adopted NENA Next Generation 9-1-1 Data Model Standard can not only support a variety of use cases but also expedite the creation of an enterprise address dataset.

1.2.2	Existing Oregon Framework Standards

Address points are mostly man-made structures that are generally be located within existing parcel and building footprint polygons. The attributes for these polygons (parcel identifier and building footprint identifier) can be added to each address point which helps tie the three themes together.  A parcel could have one or more building footprints and each building footprint could have one or more address points.

The Oregon Address Point Standard is closely related to the Cadastral Data Exchange Standard and the Statewide Buildings Footprint Standard. The Statewide Buildings Footprint Data Standard is currently in development and expected to be endorsed in 2025.  The Cadastral Data Exchange Standards, Building Footprint Standard, and Oregon Address Point Standard will require coordination through development and future updates as they will result in overlapping datasets with closely related attributes.

1.3	Description of the Standard

The intent of the Oregon Address Point Data Standard is to support data sharing and streamline the aggregation of data created and maintained by local jurisdictions into an enterprise Address Point dataset for the benefit of all Oregonians.

This Address Point Data Standard requires every address point in the state is

- complete

- consistent

- unique

Complete

Every complete street address point can have up to 12 basic elements (shown in Table 1 below). Some of the basic elements are further parsed into sub-elements in the FGDC, NENA, and NAD standards. This standard accommodates the possible elements and sub-elements to ensure that any address in Oregon has the fields to make it complete. A complete address is vital to making each address unique. Missing elements may create duplicate addresses.

Table 1 – Address Point Basic Elements

|  | ELEMENT GROUP | BASIC ELEMENT | FGDC/NENA/NAD SUB-ELEMENT | EXAMPLE |
| --- | --- | --- | --- | --- |
| 1 | Street Number | Street Number | Street Number Prefix | 123 |
|  |  |  | Street Number |  |
|  |  |  | Street Number Suffix |  |
| 2 | Street Name | Street Name Pre Direction |  |  |
| 3 |  | Street Name | Street Name Pre Modifier | Maple |
|  |  |  | Street Name Pre Type |  |
|  |  |  | Street Name Pre Separator |  |
|  |  |  | Street Name |  |
|  |  |  | Street Name Post Modifier |  |
| 4 |  | Street Name Post Type |  | Street |
| 5 |  | Street Name Post Direction |  | Northeast |
| 6 | Subaddress | Subaddress Type | Unit Building Floor Room Seat | Apartment 101 |
| 7 |  | Subaddress Number |  |  |
| 8 | Locality | City Name |  | Salem |
| 9 |  | State |  | OR |
| 10 |  | ZIP Code |  | 97301 |
| 11 | Location | Longitude |  | -123.005432 |
| 12 |  | Latitude |  | 45.809563 |

Consistent

The values used in the address elements should be unambiguous and consistent from any source. Fully spelled street names, with no abbreviations, are used as well as fully spelled street directions and street types. Domains are used to reinforce consistency by providing a complete set of acceptable values for most of the street elements.

Unique

In order for an address to be unique every address must include all of the address elements that apply, have a ZIP Code or city name, and the unit number must be included for addresses that have units. One of the cornerstones of this standard is that every address is unique within a ZIP Code or city. The same address may be found in different ZIP Codes or cities but will never be duplicated within a ZIP Code or city. The combination of the full address including the primary street address plus any secondary unit address if present plus the correct ZIP Code or city is unique and can be used as an intrinsic primary key in a dataset of all addresses. Having the correct ZIP Code or city name be assigned to each address is imperative to prevent duplicates or confusing one address for another.

1.4	Applicability and Intended Use of the Standard

The Address Point Standard is intended to support a broad range of important uses.  Table 2 describes a variety of use cases the Oregon Address Point Data Standard is intended to support.

Table 2- Identified use cases for the Oregon Address Point Data Standard.

| Category | Use-Case |
| --- | --- |
| Census | Local Update of Census Addresses Redistricting Voter Registration Elections |
| Public Safety | Next Generation 9-1-1: Location Verification (LVF) , Call Routing (ECRF) 9-1-1 Dispatchable Locations |
| Emergency Management | Risk Assessment Event Notification Evacuation Shelter |
| Disaster Response | Damage Assessment Search and Rescue |
| Property | Building Permitting Tax Assessment |
| Utilities | Broadband Mail |
| Service Delivery | Application Verification and Qualification Billing Record Keeping and Compliance |
| Planning and Development | Population Estimates Transportation Housing Public Health Business and Economic Development |

1.5	Standard Development Procedures

1.5.1	Participants

The Oregon Address Point Workgroup was led by Oregon Geospatial Enterprise Operations in close cooperation with the Oregon Department of Emergency Management, Next Generation 911 Technical Advisory Committee (NGTAC), and the Oregon Framework Coordinator.  The workgroup also consisted of participants from private sector, local jurisdictions, state agency and federal government. The Oregon Address Point Workgroup participants included data providers, data consumers, and other stakeholders with national experience. A full list of participants is provided in Appendix B.

1.5.2	Comment Opportunities and Reviews

The Oregon Address Point Data Standard was circulated throughout Oregon’s GIS Community for review and comment. Table 3 describes the review period and refinement throughout the development process.

| Date | Review | Result |
| --- | --- | --- |
| Spring/Summer 2024 | Workgroup Development | Draft Standard |
| October 29, 2024 | Proto Standard Presentation | Presentation to GIS Community at Framework Forum. |
| November 2024 | Public Review Period | Proto Standard published on GEO Website and circulated through Listservs (FIT, Address & Buildings FIT, Cadastral FIT, GPL, TAC, OGIC, etc.) |
| Winter 2024 – 2025 | Formal Peer Review | Draft Standard underwent formal peer review by Advisory Group comprised of active Framework members from multiple themes. |
| Spring 2025 | Technical Review of Preliminary Final Draft | Technical Review by members of OGIC TAC. Recommendations provided for OGIC’s Review. |
| Spring 2025 | OGIC Request for Endorsement | Presentation to OGIC of Final Draft. Request for Endorsement. |
| Spring 2025 | Endorsement & Promulgation | Published to OGIC’s Hub site and communicated through listservs and upcoming forum |

1.6	Maintenance of the Standard

The Oregon Address Point Data Standard will be reviewed regularly or as needed. As time progresses, unforeseen needs or requirements may arise, necessitating ongoing improvements to the standard. The Framework Implementation Team (FIT) plans to conduct reviews annually.

2.0	Body of the Standard

2.1	Scope and Content of the Standard

This standard is primarily intended to be used for the most commonly found type of Address Points which are –

- Physical address locations that can be seen on the ground or on aerial photography and mapped. They are usually manmade sites or structures where people live, work, or gather.

- Street addresses that have both a street number and street name that fall within the address range of a street segment.

- Any kind of residential or non-residential address whether they are a private, commercial or public place. Group quarters, such as nursing homes, are also included.

In Scope:

2.1.1 Address Points within the Scope

The scope of this standard includes every numbered street address point in the state of Oregon including residential, non-residential, group quarters. Landmarks, building names and business names associated with street address location can also be included. Table 4 further describes types of address points to be expected in a statewide address point dataset.

Table 4.  Types of Oregon Street Address Points

| Address Point or Attributes | Description of Address Point or Attributes |
| --- | --- |
| Residential | Single Family, Multi Family |
| Non-Residential | Private (commercial) or public (government), including building and business names. |
| Group Quarters | Non-Residential addresses that have residents including institutions like nursing homes, hospitals, dormitories, jails, other facilities |
| Landmarks | Landmark names and locations |

2.1.2  Address Points (or Attributes) Outside of Scope

The Oregon Address Point Data Standard will not include personal information attributes or points for  critical infrastructure locations.

Table 5.  Oregon Address Points or Attributes that are not considered within the scope of the Oregon Address Point Data Standard

| Address Point or Attributes | Description of Address Point or Attributes |
| --- | --- |
| Personal information | Names of any individual occupants, residents, or tenants of residential addresses, as well as, telephone numbers, email addresses, web addresses or any other personal information associated with residential addresses will not be included in statewide address point data set. |
| Critical infrastructure | Critical infrastructure as defined in ORS 276A.509. This excludes sites and structures that are mostly industrial machinery and where people do not typically reside or work regularly except for temporary maintenance. Electric power substations, water well sites, sewage lift stations, telecommunications switch stations, natural gas compression plants, fuel storage sites, and other automated or unmanned mechanical facilities will not be included in a statewide address point data set. |

2.2	Need for the Standard

As of 2024 there is no comprehensive statewide seamless address point dataset available for Oregon. All address points originate with local governments (cities, counties, tribal, regional) where the address authorities are usually found in the planning, development, or building permit departments of each jurisdiction. There are 241 incorporated cities, 36 counties, 9 tribes, and 6 regional councils of governments in Oregon and each maintains their own list of addresses and locations. Local jurisdictions provide the address points to other local government agencies, public utility companies, telecommunications companies, law enforcement, and the US Postal Service. The US Postal Service does not create addresses.

Generally, each jurisdiction has its own format and method for storing and distributing address information. Because the format of each jurisdiction may be different from another, a single comprehensive standard is needed to combine addresses into a seamless statewide dataset. Assembling combined address databases from multiple sources on an ad-hoc basis is extremely inefficient, time-consuming, and costly and a statewide standard will help avoid this uncoordinated duplication of effort.

2.3	Participation in the Standard Development

The Oregon Address Points Workgroup was comprised of local jurisdictions, state agencies, academic partners and federal partners. Participation in the workgroup was open to all entities interested in the production, use and exchange of address points. The workgroup was intentionally composed to include participation throughout various levels of stakeholder interest including data originators, data aggregators, and data consumers. Listed below are all local, state, academic, and federal organizations that participated in development or review of the address point standard.

Local Jurisdictions:

- City of Portland

- City of Salem

- Baker County

- Lane County Council of Governments

- Yamhill County

State Agencies:

- Oregon Department of Emergency Management

- Department of Geology and Mineral Industries

- Department of Land Conservation and Development

- Geospatial Enterprise Operations

- Secretary of State’s Office

- Oregon Department of Forestry

- Business Oregon

Academic Partners:

- Portland State University, Population Research Center

Federal Partners:

- US Department of Transportation, National Address Database

- GEOComm

2.4	Integration with Other Standards

The Oregon Address Points Data Standard will coordinate closely with the Cadastral Data Exchange Standard, Road Centerline Data Standard, and Statewide Building Footprints Data Standard.

Cadastral Data Exchange Standard: Address points generally are located within parcel polygons and a parcel identifier can be added to the address point to group them by parcel.

Road Centerline Data Standard:  Address points are located along road centerlines and will contain similar attributes.

Statewide Buildings Footprint Data Standard: The Statewide Buildings Footprint Data Standard is currently in development and the Oregon Address Point Standards workgroup will coordinate closely with the Statewide Buildings Footprint workgroup. Address points generally are located within building footprint polygons and a building identifier can be added to the address point to group them by building.

2.5	Technical and Operational Context

This standard can be implemented in several ways.

- Basic – As a single text file format including CSV, JSON, XML without domains.

- Intermediate – As a file geodatabase with domains.

- Advanced – As a full relational database model with several lookup tables.

2.5.1	Data Environment

The spatial data environment for the address standard is a vector model comprised exclusively of point geometry and associated attribute tables.

2.5.2	Reference System

The NENA standard specifies the spatial reference system will be in longitude (X) and latitude (Y) stored as decimal degrees in WGS84 (well-known spatial reference ID 4326).

2.5.3	Integration of Themes

Addresses and Buildings Theme

The address points are very closely related to building footprints. Address points will usually fall within a building footprint and, in the case of multi-family or multi-business addresses, there could be multiple address points within one building footprint. Each address point will typically have a building footprint identifier as an attribute based on the location. Exceptions can occur where the address point was created before the actual building has been constructed (future/planned status) or where building has been demolished (past/historical status).

Cadastral Framework Theme

Address points and building footprints are usually found within parcel boundaries. Each address point will typically have a parcel identifier as an attribute. There can be one or more address points within a building footprint, and one or more building footprints within a parcel. For example, a parcel may have an apartment complex with several buildings and each building could have several address points for the units.

Transportation Framework Theme

Address points are more loosely related to street centerlines. The full street name (including street type and direction) should have a corresponding street segment in the road centerlines. Likewise, the street number of an address point should be within range of street numbers for the correct street segment and on the correct side (right or left) of the street segment.

Preparedness Framework Theme

Subsets of address points are used for elements in the Preparedness theme such as hospitals, schools, police stations, fire stations, and many others. These often contain additional detail fields that are not included in the Oregon Address Point Standard to provide much more information about each subset. Each address point found in a Preparedness element should also have a corresponding address point in the Address Point theme.

2.5.4	Encoding

Proper encoding greatly improves data consistency and cuts down on data cleanup. This is very important when sorting and comparing addresses to identify duplicates.

All address text attribute data will be stored using only upper case letters (A-Z), lower case letters (a-z), numbers (0-9), and spaces in ASCII 8-bit characters in ranges 32, 47–57, 65-90, 97-122. Do not use other special characters except those noted below, non-printing ASCII characters (in range 0 – 31), or extended ASCII characters (in range 128 – 255). No extended ASCII characters are stored for diacritical symbols in foreign language spellings in a street address. Unicode data types (NVARCHAR) are not used are not necessary. Invalid characters (most special characters, non-printing or extended characters) found embedded in the source data should be converted to standard ASCII characters or removed.

Only single spaces between words are stored. Multiple spaces between words should be converted to single spaces. Leading and trailing spaces should be removed.

All text values are stored using Title Case where all principal words are capitalized and others are not capitalized, like articles (a, an, the, others), prepositions (on, in, of, de, de la, others) and a few others (as, to, and). This includes any foreign language (Spanish, others) words as well as English words. All words are fully spelled and no abbreviations are used except as noted below.

Examples:	7700 Avenue of the Sun, White City, OR, 97503

15980 Camino de Oro, La Pine, OR, 97739

Full Spelling Exceptions:

Numbered streets are not fully spelled (1st, 2nd, 3rd, 4th, 5th, etc.). Also State (OR) and Country (US) are always abbreviated.

Examples:	2500 Southeast 157th Avenue Apartment 19, Portland, OR, 97236

Special Character Exceptions

A hyphen (-) may be found in sub-address numbers.

Examples:	3407 S Hemlock Street Unit C-2, Cannon Beach, OR, 97110

A forward slash (/) is used to store half-addresses in the street number suffix field. One half is stored as one-forward slash-two “1/2” and not ½ (extended ASCII 189).

Examples:	1210 1/2 River Road, Eugene, OR, 97404

A pound sign (#) in the Unit field may be used to separate a sub-address (unit) number from the street address where the unit type is not otherwise indicated. This is especially important for numbered highway addresses that have sub-addresses to avoid having the sub-address number confused with the highway number.

Examples:	21255 Highway 20 # 20, Bend, OR, 97701 (21255 Highway 20 Apartment 20)

2.5.5	Resolution

The resolution of the data will be determined by the local jurisdictions creating and maintaining address point data.

2.5.6	Accuracy

There are two definitions of accuracy for address data in this standard.

- Address Attribute Accuracy – The completeness and correctness of all address elements.

- Address Location Accuracy – The positional accuracy of the location coordinates of the address.

Address Attribute Accuracy

Each address must include all its components, such as directionals, street type, unit numbers, and any other essential elements; none of these can be missing or excluded if they are part of the address.

One way to check the accuracy of the address is to use the free USPS ZIP Code Lookup tool.

Enter an incomplete or suspect address along with the city and state. The tool will complete the address by filling in the missing elements (street type and post-direction in this example) and provide the correct ZIP Code. US Postal Service CASS-certified software and services can do the same thing, and add more information, for large address lists but are not free. The only limitation with these tools is that they work best for addresses that receive mail (which most do).

Address Location Accuracy

The position of an address point is relative, not absolute, because an address point generally represents large features on the ground, such as a house, not a precise point, such as a survey monument. Address points are typically placed manually based on the interpretation of one or a combination of the following: maps, site diagrams, aerial photographs, street view photos, and/or other reference data. Various levels of accuracy will be defined for the location including unit level (best), rooftop or building level (good), parcel level (fair) or street level (estimated).

| RANK | NENA VALUE | OREGON VALUE | QUALITY |
| --- | --- | --- | --- |
| 0 | Unknown | Unknown | None |
| 1 |  | Unit | Best |
| 2 | Structure | Rooftop | Good |
| 3 | Access Point |  | Good |
| 4 | Site |  | Good |
| 5 | Parcel | Parcel | Fair |
| 6 | Geocode | Street | Estimated |

At a minimum, the address point(s) should be located within the building footprint if a structure exists on the property. If no structure exists, the address points(s) must be located within the parcel for the property. Address points that are estimated along a street centerline should be repositioned to either the correct parcel or building footprint.

Buildings

Parcels

1

2

3

4

5

6

Street Centerline

Placement Examples

1 – 	One rooftop address, one building, one parcel (single family home)

2 – 	Multiple rooftop addresses, multiple buildings, one parcel (home with ADU)

3 – 	One parcel address, no building, one parcel (vacant lot, future construction or past historical)

4 –	 One parcel address, one building, one parcel (misplaced address point, move to building)

5 – 	Multiple unit addresses, multiple buildings, one parcel (apartment complex)

6 – 	One street address, no parcel (geocode location, in right-of-way, move to parcels or buildings)

2.5.7	Edge Matching

Many public safety agencies also map address points in neighboring jurisdictions that are near their boundaries to aid in emergency response. However, duplicate address points in overlapping neighboring datasets must be removed when building any aggregated dataset. Often these will be easy to identify because they will have the same addresses and location coordinates. Less obvious duplicates may require additional standardization and comparison steps to identify and eliminate.

2.5.8	Feature Identifier

Every feature identifier must be assigned by the provider, unique and permanently assigned to the address point for the entire duration of its lifecycle. There can be multiple feature identifiers in an address points dataset.

- The NENA standard specifies a specific format for the NENA Globally Unique ID field.

2.5.9	Attributes

The Oregon Address Points Data start has divided attributes into two categories, NENA Core Attributes, and supplemental Oregon attributes.

2.5.10	Transactional Updating

Address point data is created and maintained by local jurisdictions then aggregated into a statewide dataset. Transactional updates to the aggregated dataset relies on maintaining a unique primary key to ensure the same address is comparable in both the old and new updates. While most address points will have no change in each update some may have changes including -

New address (added)

Changed address or location (modified)

Old address (deleted)

2.5.11	Records Management

The Oregon Address Points Data Standard will be available to the public on OGIC’s Hub site with all other Oregon Framework standards. The geospatial data created using this standard will be made available to the public through standard means such as online data services or data downloads through GEOHub.

2.5.12	Metadata

The standard follows the Oregon Framework Metadata Standard for geospatial data, which is integrated with the Federal Geographic Data Committee, Content Standard for Digital Geospatial Metadata.

3.0	Data Characteristics

3.1	Minimum Graphic Data Elements

The Oregon Address Point Data Standard only includes point features.

Table 3 – Address Point Data Schema

IMPORTANT: The core of the Oregon Address Point Standard is the NENA Site Structure Address Point (SSAP) schema. Refer to the NENA Standard for NG9-1-1 GIS Data Model document on pages 31 – 33 with detailed descriptions of each field starting on page 51.

Required – Red indicates a change in the Oregon standard from the national standard.

"Y" = The data element is required to be present in all addresses.

"N" = The data field is optional in an address.

"C" = The data field is conditionally required if a value is present in a complete address.

Used – Indicates how often a field is used.

"1" = Always used

"2" = Commonly used

"3" = Occasionally used

"4" = Rarely used

Values –

"Y" - Domain or lookup table of acceptable values is available for the field.

Source – Indicates if the field comes from the NENA core or has been added for the Oregon standard.

"NENA"	   = Core fields (45 fields)

"ADDED" = Supplemental fields added to the Oregon standard (14 fields)

| GROUP | SUBGROUP | FIELD NAME | FIELD DESCRIPTION | DATA TYPE | WIDTH | REQUIRED | USED | VALUES | SOURCE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDRESS |  | ADDRESS_ID | Unique Address Identifier | TEXT | 36 | Y | 1 |  | ADDED |
| ADDRESS |  | ADDRESS_FULL | Complete Street Address | TEXT | 255 | Y | 1 |  | ADDED |
| ADDRESS | STREET NUMBER | STREET_NUMBER_FULL | Complete Street Number | TEXT | 10 | Y | 1 |  | ADDED |
| ADDRESS | STREET NUMBER | Add_Number | Address Number | INTEGER | 6 | Y | 1 |  | NENA |
| ADDRESS | STREET NUMBER | AddNum_Pre | Address Number Prefix | TEXT | 15 | C | 4 |  | NENA |
| ADDRESS | STREET NUMBER | AddNum_Suf | Address Number Suffix | TEXT | 15 | C | 2 |  | NENA |
| ADDRESS | STREET NAME | STREET_NAME_FULL | Complete Street Name | TEXT | 254 | Y | 1 |  | ADDED |
| ADDRESS | STREET NAME | St_PosTyp | Street Name Post Type | TEXT | 50 | C | 2 | Y | NENA |
| ADDRESS | STREET NAME | St_PreDir | Street Name Pre Directional | TEXT | 10 | C | 2 | Y | NENA |
| ADDRESS | STREET NAME | St_PosDir | Street Name Post Directional | TEXT | 10 | C | 2 | Y | NENA |
| ADDRESS | STREET NAME | St_Name | Street Name | TEXT | 254 | Y | 1 | Y | NENA |
| ADDRESS | STREET NAME | St_PreTyp | Street Name Pre Type | TEXT | 50 | C | 3 | Y | NENA |
| ADDRESS | STREET NAME | St_PreSep | Street Name Pre Type Separator | TEXT | 20 | C | 4 | Y | NENA |
| ADDRESS | STREET NAME | St_PreMod | Street Name Pre Modifier | TEXT | 15 | C | 4 |  | NENA |
| ADDRESS | STREET NAME | St_PosMod | Street Name Post Modifier | TEXT | 25 | C | 4 |  | NENA |
| ADDRESS | SUBADDRESS | SUBADDRESS_FULL | Complete Unit Address | TEXT | 20 | C | 2 |  | ADDED |
| ADDRESS | SUBADDRESS | SUBADDRESS_TYPE | Unit Type | TEXT | 10 | C | 2 | Y | ADDED |
| ADDRESS | SUBADDRESS | SUBADDRESS_NUMBER | Unit Number | TEXT | 10 | C | 2 |  | ADDED |
| ADDRESS | SUBADDRESS | Unit | Unit | TEXT | 75 | C | 2 |  | NENA |
| ADDRESS | SUBADDRESS | Building | Building | TEXT | 75 | N | 3 |  | NENA |
| ADDRESS | SUBADDRESS | Floor | Floor | TEXT | 75 | N | 3 |  | NENA |
| ADDRESS | SUBADDRESS | Room | Room | TEXT | 75 | N | 3 |  | NENA |
| ADDRESS | SUBADDRESS | Seat | Seat | TEXT | 75 | N | 4 |  | NENA |
| ADDRESS | SUBADDRESS | Addtl_Loc | Additional Location Information | TEXT | 225 | N | 4 |  | NENA |
| LOCALITY | PROPERTY | BUILDING_ID | Building Footprint Identifier | TEXT | 20 | N | 2 |  | ADDED |
| LOCALITY | PROPERTY | PARCEL_ID | Parcel Identifier | TEXT | 20 | N | 2 |  | ADDED |
| LOCALITY | COMMUNITY | Post_Comm | Postal Community Name | TEXT | 40 | Y | 1 | Y | NENA |
| LOCALITY | COMMUNITY | Post_Code | Postal Code | TEXT | 7 | Y | 1 | Y | NENA |
| LOCALITY | COMMUNITY | PostCodeEx | Postal Code Extension | TEXT | 4 | N | 2 |  | NENA |
| LOCALITY | COMMUNITY | Inc_Muni | Incorporated Municipality (A3) | TEXT | 100 | Y | 1 | Y | NENA |
| LOCALITY | COMMUNITY | Uninc_Comm | Unincorporated Community (A4) | TEXT | 100 | N | 3 | Y | NENA |
| GROUP | SUBGROUP | FIELD NAME | FIELD DESCRIPTION | DATA TYPE | WIDTH | REQUIRED | USED | VALUES | SOURCE |
| LOCALITY | COMMUNITY | Nbrhd_Comm | Neighborhood Community (A5) | TEXT | 100 | N | 4 |  | NENA |
| LOCALITY | REGION | County | County or Equivalent (A2) | TEXT | 100 | Y | 1 | Y | NENA |
| LOCALITY | REGION | State | State or Equivalent (A1) | TEXT | 2 | Y | 1 | OR | NENA |
| LOCALITY | REGION | Country | Country | TEXT | 2 | Y | 1 | US | NENA |
| LOCATION |  | SHAPE | SHAPE | GEOMETRY |  | Y | 1 |  | ADDED |
| LOCATION |  | OBJECTID | ArcGIS Record Identifier | INTEGER |  | Y | 1 |  | ADDED |
| LOCATION |  | Longitude | Longitude (X) | DECIMAL |  | Y | 1 |  | NENA |
| LOCATION |  | Latitude | Latitude (Y) | DECIMAL |  | Y | 1 |  | NENA |
| LOCATION |  | Elevation | Elevation (Z) | INTEGER | 6 | N | 4 |  | NENA |
| LOCATION |  | Placement | Placement Method | TEXT | 25 | N | 2 | Y | NENA |
| EXTRA | CASS | MAIL | USPS Delivery Point Validation | TEXT | 20 | N | 2 | Y | ADDED |
| EXTRA | CASS | RESIDENTIAL | USPS Residential Delivery Indicator | TEXT | 20 | N | 2 | Y | ADDED |
| EXTRA | TIME | STAGE | Address Lifecycle Stage | TEXT | 20 | N | 2 | Y | ADDED |
| EXTRA |  | Place_Type | Place Type | TEXT | 50 | N | 3 | Y | NENA |
| EXTRA |  | LandmkName | Complete Landmark Name | TEXT | 150 | C | 3 |  | NENA |
| EXTRA |  | AddDataURI | Additional Data URI | TEXT | 254 | C | 3 |  | NENA |
| METADATA | TIME | DateUpdate | Date Updated | DATE |  | Y | 1 |  | NENA |
| METADATA | TIME | Effective | Effective Date | DATE |  | N | 3 |  | NENA |
| METADATA | TIME | Expire | Expiration Date | DATE |  | N | 3 |  | NENA |
| 911 | 911 IDENTIFIER | DiscrpAgID | Discrepancy Agency ID | TEXT | 100 | Y | 1 |  | NENA |
| 911 | 911 IDENTIFIER | NGUID | NENA Globally Unique ID | TEXT | 254 | Y | 1 |  | NENA |
| 911 | 911 LEGACY ADDRESS | LSt_PreDir | Legacy Street Name Pre Directional | TEXT | 2 | C | 4 | Y | NENA |
| 911 | 911 LEGACY ADDRESS | LSt_Name | Legacy Street Name | TEXT | 75 | C | 4 |  | NENA |
| 911 | 911 LEGACY ADDRESS | LSt_Typ | Legacy Street Name Type | TEXT | 4 | C | 4 | Y | NENA |
| 911 | 911 LEGACY ADDRESS | LSt_PosDir | Legacy Street Name Post Directional | TEXT | 2 | C | 4 | Y | NENA |
| 911 | 911 REGION | MSAGComm | Master Street Address Guide Community Name | TEXT | 30 | C | 4 | Y | NENA |
| 911 | 911 REGION | ESN | Emergency Service Number | TEXT | 5 | C | 4 | Y | NENA |
| 911 | 911 OTHER | Milepost | Milepost | TEXT | 150 | C | 4 |  | NENA |

Address Point Data Dictionary – Alphabetical by Field Name

Format

| Field Name | Field Description | NENA Core or Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| 5.n - NENA standard section and description. |  |  |  |  |
| Oregon additional description, notes, tips, and examples. |  |  |  |  |

| AddDataURI | Additional Data URI | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.4 - Uniform Resource Identifier(s) for additional data associated with the address point. This attribute is contained in the Site Structure Address Point layer and will define the Service URI of additional information about a location, including building information (blueprints, contact info, floor plans, etc.). |  |  |  |  |
| A web link that adds context to, or additional specific information about, a street address. Examples: 1705 Main Street, Baker City, OR 97814 https://www.bakertower.com |  |  |  |  |

| Addtl_Loc | Additional Location Information | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.5 - A part of a sub-address that is not a Building, Floor, Unit, Room, or Seat. |  |  |  |  |

| Add_Number | Address Number | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.6 - The numeric identifier of a location along a thoroughfare or within a defined community. |  |  |  |  |
| The street number, occurring within the range of block numbers along a street segment, that is usually assigned to each house or separate structure. The street number is the integer-only portion and does not include any leading zeros or trailing characters. Even and odd numbers are usually found on opposite sides of the street. |  |  |  |  |

| AddNum_Pre | Address Number Prefix | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.7 - An extension of the address number that precedes it and further identifies a location along a thoroughfare or within a defined area. |  |  |  |  |
| There are no known addresses in Oregon with an address number prefix. |  |  |  |  |

| AddNum_Suf | Address Number Suffix | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.8 - An extension of the address number that follows it and further identifies a location along a thoroughfare or within a defined area. |  |  |  |  |
| The vast majority of Oregon street numbers are numeric only. However, a few street numbers also contain trailing characters with the most common being half addresses (1/2) separated from the street number by a space. Examples: 510 1/2 East 12th Avenue Apartment B, Eugene, OR 97401 Much less common are single letters (usually A, B, C, D) sometimes used in mailing addresses for units in duplexes, triplexes, or Auxiliary Dwelling Units (ADU). These characters are usually not separated from the street number by a space. For added confusion, these letters could also be in the Unit field instead. Examples: 2560A Columbia Boulevard, Saint Helens, OR 97051 2560B Columbia Boulevard, Saint Helens, OR 97051 2560C Columbia Boulevard, Saint Helens, OR 97051 2560D Columbia Boulevard, Saint Helens, OR 97051 |  |  |  |  |

| ADDRESS_FULL | Complete Street Address | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| The complete address concatenated from all the address elements present for an address including the full street number, full street name, and full subaddress. Can be part of a composite intrinsic primary key along with the ZIP Code. |  |  |  |  |

| ADDRESS_ID | Address Identifier | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Unique extrinsic address primary key. |  |  |  |  |

| Building | Building | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.19 - One among a group of buildings that have the same address number and complete street name. |  |  |  |  |
| The name, number, or letter identifying a single building among a group of buildings associated with the same street address. Buildings are often identified in apartment complexes, university campuses, business parks, and other multi-building areas. Examples: 415 Toliver Road Building F1, Molalla, OR 97038 415 Toliver Road Building F2, Molalla, OR 97038 Tip – Sometimes the building is part of the unit number. Examples: 1605 Oak Street Southeast Apartment A101, Albany, OR 97322 (Building A, Unit 101) |  |  |  |  |

| BUILDING_ID | Building Footprint Identifier | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Placeholder for a building footprint identifier that the address point is located within. Still to be determined from a future building footprint data standard. |  |  |  |  |

| Country | Country | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.24 - The name of a country represented by its two-letter ISO 3166-1 English country alpha-2 code elements in UPPER CASE letters. |  |  |  |  |
| “US” only. No blanks or nulls. |  |  |  |  |

| County | County or Equivalent (A2) | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.27 - The name of a County or County-equivalent where the address is located. A county (or its equivalent) is the primary legal division of a state or territory. |  |  |  |  |
| Values are the county names for the 36 counties in Oregon listed in a domain. No blanks or nulls. |  |  |  |  |

| DateUpdate | Date Updated | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.30 - The date and time that the record was created or last modified. This value MUST be populated upon modifications to attributes, geometry, or both. |  |  |  |  |

| DiscrpAgID | Discrepancy Agency Identifier | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.31 - Agency that receives a Discrepancy Report (DR), should a discrepancy be discovered, and will take responsibility for ensuring discrepancy resolution. This may or may not be the same as the 9-1-1 Authority. |  |  |  |  |

| Effective | Date Effective | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.33 - The date and time that the record is scheduled to take effect. |  |  |  |  |

| Elevation | Elevation | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.34 - The elevation, given in meters above a reference surface defined by the coordinate system, associated with the site/structure address. |  |  |  |  |

| ESN | Emergency Service Number | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.35 - A 3-5 character numeric string that represents one or more 9-1-1 Emergency Service Zones (ESZ). |  |  |  |  |

| Expire | Date Expired | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.39 - The date and time when the information in the record is no longer considered valid. |  |  |  |  |

| Floor | Floor | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.40 - A floor, story, or level within a building. |  |  |  |  |
| Examples: 1300 Southwest 5th Avenue Floor 10, Portland, OR 97201 1300 Southwest 5th Avenue Floor 11, Portland, OR 97201 1300 Southwest 5th Avenue Floor 12, Portland, OR 97201 Tip – Sometimes the floor is part of the unit number. Examples: 1605 Oak Street Southeast Apartment A101, Albany, OR 97322 (Building A, Floor 1, Unit 101) 1605 Oak Street Southeast Apartment A201, Albany, OR 97322 (Building A, Floor 2, Unit 201) |  |  |  |  |

| Inc_Muni | Incorporated Municipality | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.45 - The name of the Incorporated Municipality or other general-purpose local governmental unit (if any) where the address is located. Use "Unincorporated" if the address is not within an incorporated local government. |  |  |  |  |
| The name of the actual incorporated jurisdiction the address point is in. Values of all 241 incorporated city names in Oregon are listed in a domain. IMPORTANT – The postal city name may not match the incorporated city name of the actual jurisdiction the address point is located in because ZIP Codes frequently do not follow incorporated city limits. |  |  |  |  |

| LandmkName | Complete Landmark Name | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.21 - The name by which a prominent site/structure is publicly known. Landmarks may or may not be associated with a civic address. |  |  |  |  |
| The landmark name can be used to attach a familiar, prominent, or famous place name to a street address. Examples: 900 Court Street Northeast, Salem, OR 97301 Landmark Name: Oregon State Capitol |  |  |  |  |

| Latitude | Latitude (Y) | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.51 - The angular distance of a location north or south of the equator as defined by the coordinate system, expressed in decimal degrees. |  |  |  |  |
| In Oregon the latitude (Y) coordinate is north of the Equator in positive decimal degrees between 42.0 and 46.25. Accuracy to six decimal places is required. |  |  |  |  |

| Longitude | Longitude (X) | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.65 - The angular distance of a location east or west of the prime meridian of the coordinate system, expressed in decimal degrees. |  |  |  |  |
| In Oregon the longitude (X) coordinate is west of the Prime Meridian in negative decimal degrees from -116.5 to -124.5. Accuracy to six decimal places is required. |  |  |  |  |

| LSt_Name | Legacy Street Name | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.55 - The street name as it currently exists in the MSAG. Ideally this is the name as assigned by the local addressing authority. |  |  |  |  |

| LSt_PosDir | Legacy Street Post Direction | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.56 - The street direction suffix as it currently exists in the MSAG. Ideally this is the name as assigned by the local addressing authority. |  |  |  |  |

| LSt_PreDir | Legacy Street Pre Direction | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.57 - The leading street direction prefix as it currently exists in the MSAG. Ideally this is the street name pre directional as assigned by the local addressing authority. |  |  |  |  |

| LSt_Typ | Legacy Street Type | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.58 - The valid street abbreviation as it currently exists in the MSAG. Ideally this is the street name type as assigned by the local addressing authority. |  |  |  |  |

| MAIL | USPS Delivery Point Validation | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| DPV indicates that the street and unit addresses matches the US Postal Service Delivery Point File (DPF) and receives mail. The USPS ZIP Code Lookup tool will provide the DPV for an individual address. CASS tools will also return the DPV for address lists. Y = Matches both street and unit address (if present). Receives mail. N = Does not match a street number for the street address range. Does not receive mail. D = Matches street address but a unit address is missing. Does not receive mail. S = Matches street address but the unit address is incorrect. Does not receive mail. Null = No match found in the DPF. Does not receive mail. NOTE – Many addresses actually exist on the ground that do not receive mail. These addresses may have a Post Office box for a mailing address. For example, addresses in Sunriver do not receive mail. |  |  |  |  |

| Milepost | Milepost | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.67 - A distance travelled along a route such as a road or highway, typically indicated by a milepost sign. There is typically a post or other marker indicating the distance in miles/kilometers from or to a given point. |  |  |  |  |
| Not required for Oregon street addresses. |  |  |  |  |

| MSAGComm | Master Street Address Guide Community | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.68 - The Community name associated with an address as given in the MSAG and may or may not be the same as the Community Name used by the postal service. |  |  |  |  |

| Nbrhd_Comm | Neighborhood Community | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.71 - The name of an unincorporated neighborhood, subdivision, or area, either within an incorporated municipality or in an unincorporated portion of a county or both, where the address is located. |  |  |  |  |

| NGUID | NENA Globally Unique Identifier | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.74 - The NENA Globally Unique ID (Primary Key) for each record in a GIS data layer. Each record in the GIS data layer MUST have a globally unique ID. |  |  |  |  |

| OBJECTID | ArcGIS Record Identifier | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| The OBJECTID is a unique, sequential, auto-incrementing row number for each geometry feature that is used and maintained exclusively by ArcGIS software. It is not used as an address primary key because it may change unexpectedly at any time. |  |  |  |  |

| PARCEL_ID | Parcel Identifier | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Placeholder for the parcel or taxlot that the address point is located within. Conforms to the MapTaxlot field in the Oregon Cadastral Exchange Standard. |  |  |  |  |

| Place_Type | Place Type | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.78 - The type of feature identified by the address. See the Location Types Registry. |  |  |  |  |

| Placement | Placement Method | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.79 - The methodology used for placement of the address point. See the Placement Method Registry. |  |  |  |  |

| Post_Code | Postal Code | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.80 - A system of 5-digit (US) codes that identify the individual USPS Post Office or metropolitan area delivery station associated with an address. |  |  |  |  |
| The US Postal Service assigned five-digit Zone Improvement Plan (ZIP) Code for the address. This address element can be used to make identical addresses found in different cities unique. There are 479 ZIP Codes in Oregon all beginning with “97” and all values are listed in a domain. The USPS ZIP Code Lookup tool can be used to find the correct ZIP Code for individual street addresses. USPS CASS certified software or services can be also used to find the correct ZIP Code for address lists. ZIP Codes are always stored as text, not integers, because they can have leading zeros. The first character of a ZIP Code is for the general geographic area within the US from 0 on the east coast to 9 on the west coast. The second and third characters are for regions within Oregon. The last two characters are for specific Post Offices. Examples: 10 North Main Street, Lebanon, OR, 97355 10 North Main Street, Falls City, OR, 97344 |  |  |  |  |

| Post_Comm | Postal Community | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.84 - A city name for the Postal Code of an address. |  |  |  |  |
| The USPS preferred city name associated with the ZIP Code. ZIP Codes do not necessarily follow city limits so the postal city name may not match the actual city jurisdiction the address is located in. The US Postal Service exclusively manages ZIP Codes and the preferred city names. Some ZIP Codes have other acceptable postal city names that can be used for mail instead of the preferred city name. Refer to the ZIP Codes in Oregon to see the preferred, acceptable, and city names to avoid for each ZIP Code. |  |  |  |  |

| PostCodeEx | Postal Code Extension | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.81 - The addition of the Postal Code Extension refines the mail delivery point down to a specific block or building, and may prove useful to validate locations. Postal Code Extensions change more often than US Postal Codes, and this additional data field should make maintaining these optional codes easier |  |  |  |  |
| The four-digit ZIP Code+4 add-on. The USPS ZIP Code Lookup tool can be used to find the correct ZIP Code add-on for any individual address. CASS certified software or services can also be used to find the correct ZIP Code add-on for address lists. The ZIP Code add-on is not used to make an address unique because the five-digit ZIP Code is sufficient to make sure there are no duplicate addresses. Even though they are numerals the ZIP+4 add-on is always stored as text, not integers, because it can have leading zeros. |  |  |  |  |

| RESIDENTIAL | USPS Residential Delivery Indicator | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| The US Postal Service Residential Delivery Indicator (RDI) can be used to distinguish a residential address from a non-residential address. CASS software can provide the RDI. |  |  |  |  |

| Room | Room | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.97 - A single room within a building. |  |  |  |  |

| Seat | Seat | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.98 - A place where a person might sit within a building. |  |  |  |  |

| SHAPE | SHAPE | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| The SHAPE field stores the spatial location of the address point as a binary geometry object. This object is constructed from - 1. The geometry type – single POINT for every address. There will be no multi-point features for address points. 2. The negative longitude (X) coordinate for the east-west direction to six decimal places. 3. The positive latitude (Y) coordinate for the north-south direction to six decimal places. 4. The spatial reference well-known identifier. 4326 = WGS84 (World Geodetic System 1984). Examples: POINT(-123.123456 45.123456),4326 NOTE: The SHAPE field will also have a spatial index that greatly speeds up spatial operations. |  |  |  |  |

| St_Name | Street Name | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.110 - The official name of the road, usually defined by the lowest jurisdictional authority (e.g., city). The street name does not include any street types, directionals, or modifiers. |  |  |  |  |
| The street name may consist of letters or numbers, single or multiple words separated by single spaces. Examples: 124th Avenue Avenue A Highway 101 Summer Street |  |  |  |  |

| St_PosDir | Street Name Post Direction | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.111 - A word following the Street Name element that indicates the direction taken by the road from an arbitrary starting point or line, or the sector where it is located. |  |  |  |  |
| A street may or may not have a post-direction. This usually depends on the jurisdiction. Generally, if a street has a pre-direction it will not have a post-direction. Many streets have neither and may be blank or null. The pre-direction and post-direction values are the same – North, South, East, West, Northeast, Northwest, Southeast, Southwest. Examples: 500 Summer Street Northeast, Salem, OR 97301 |  |  |  |  |

| St_PosMod | Street Name Post Modifier | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.112 - A word or phrase that follows and modifies the Street Name element, but is separated from it by a Street Name Post Type or a Street Name Post Directional or both. |  |  |  |  |
| Very few addresses in Oregon have a street name post-modifier. Examples: Highway 95 Spur Highway 101 Business |  |  |  |  |

| St_PosTyp | Street Name Post Type | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.113 - A word or phrase that follows the Street Name element and identifies a type of thoroughfare in a complete street name. |  |  |  |  |
| The fully spelled street suffix or street type following the street name (Avenue, Boulevard, Road, Street, many others), that identifies a type of thoroughfare in a complete street name. The vast majority of streets will have a street type but this field may be null for the relatively few addresses that do not have a street type (mostly numbered Highway addresses). A list of all street type values are listed in a domain. |  |  |  |  |

| St_PreDir | Street Name Pre Direction | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.114 - A word preceding the Street Name element that indicates the direction taken by the road from an arbitrary starting point or line, or the sector where it is located. |  |  |  |  |
| A street may or may not have a pre-direction. This usually depends on the jurisdiction. Generally, if a street has a post-direction it will not have a pre-direction. Many streets have neither and may be blank or null. The pre-direction and post-direction values are the same – North, South, East, West, Northeast, Northwest, Southeast, Southwest. Examples: 800 Northeast Oregon Street, Portland, OR 97232 |  |  |  |  |

| St_PreMod | Street Name Pre Modifier | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.115 - A word or phrase that precedes and modifies the Street Name element but is separated from it by a Street Name Pre Type or a Street Name Pre Directional or both. |  |  |  |  |
| There are very few addresses in Oregon that have a pre-modifier. Examples: Old Highway 99 |  |  |  |  |

| St_PreSep | Street Name Pre Separator | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.117 - A preposition or prepositional phrase between the Street Name Pre Type and the Street Name. See Street Name Pre Type Separators. |  |  |  |  |
| Very few street names in Oregon have pre separators which are found between the street pre-type and street name for certain addresses. These can have foreign language spellings (Spanish, Italian, French, Portuguese) as well as English spellings and will also have street pre types. Examples: 7700 Avenue of the Sun, White City, OR 97503 15820 Camino de Oro, La Pine, OR 97739 |  |  |  |  |

| St_PreTyp | Street Name Pre Type | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.116 - A word or phrase that precedes the Street Name element and identifies a type of thoroughfare in a complete street name. See Street Name Pre and Post Types. |  |  |  |  |
| The street type appears before the street name for certain addresses. The street pre-type uses the same values as the street type domain. Generally, there will be no street type if there is a street name pre-type. By far, the most common street pre-type in Oregon is for numbered Highway addresses. Examples: 2701 Northwest Highway 101, Lincoln City, OR 97367 1125 Avenue A, Seaside, OR 97138 |  |  |  |  |

| STAGE | Address Lifecycle Stage | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Indicates the lifecycle stage the address is in, especially if there is no structure associated with it. Future - Planned, platted, or pending address that has not been constructed yet. Current - Address is active, constructed, occupied Past - Historical address that has been demolished. |  |  |  |  |

| State | State or Equivalent (A1) | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.107 - The name of a state or state equivalent, represented by the two-letter UPPER CASE abbreviation given in USPS Publication 28, Appendix B. A state is a primary governmental division of the United States. See the Census State List. |  |  |  |  |
| This field has only one value - OR for Oregon. No blanks or nulls are allowed. |  |  |  |  |

| STREET_NAME_FULL | Complete Street Name | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Complete street name concatenated from these separate street name elements separated by blank spaces: Street Name Pre Modifier Street Name Pre Type Street Name Pre Separator Street Name Street Name Post Modifier It does not include these elements: Street Name Pre Direction Street Name Post Type Street Name Post Direction |  |  |  |  |

| STREET_NUMBER_FULL | Complete Street Number | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Complete street number concatenated from these separate address number elements: AddNum_Pre Add_Number AddNum_Suf |  |  |  |  |

| SUBADDRESS_FULL | Complete Subaddress | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| Complete subaddress concatenated from these separate subaddress elements separated by a blank space: SUBADDRESS_TYPE SUBADDRESS_NUMBER |  |  |  |  |

| SUBADDRESS_NUMBER | Subaddress Number | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| The FGDC, NENA, and NAD standards typically concatenate both the unit type and unit number into one field. The USPS standard has separate unit type and unit number fields. |  |  |  |  |

| SUBADDRESS_TYPE | Subaddress Type | Oregon Added |  |  |
| --- | --- | --- | --- | --- |
| none |  |  |  |  |
| The FGDC, NENA, and NAD standards typically concatenate both the unit type and unit number into one field. The USPS standard has separate unit type and unit number fields. The unit type has specific values listed in a domain (apartment, space, suite, unit, etc.). A pound sign (#) can also be used if the unit type is not known. |  |  |  |  |

| Uninc_Comm | Unincorporated Community | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.120 - The name of an Unincorporated Community, either within an incorporated municipality or in an unincorporated portion of a county, or both, where the address is located. |  |  |  |  |
| This may not be the same as the preferred city name for the ZIP Code but might be an acceptable alternate mailing city name. Could also be a Census Designated Place (CDP). Examples: Aloha, OR 97078 Damascus, OR 97009 |  |  |  |  |

| Unit | Unit | NENA Core |  |  |
| --- | --- | --- | --- | --- |
| 5.123 - A group or suite of rooms within a building that are under common ownership or tenancy, typically having a common primary entrance. |  |  |  |  |
| The unit number can be any combination of letters and/or numbers. Examples – Apartment 101, Apartment B-201, Space 18, Suite 3, Unit A. By far, the most common unit numbers are either single numbers or single letters. Various other letter/number patterns are less common. IMPORTANT – If an address has units then they must be included as part of a complete unique address point. Because it is needed to make an address unique, the Oregon standard makes the unit field conditionally required (instead of not required in the national NENA standard). If the unit numbers are omitted from address points that should have them, duplicate street addresses could result. IMPORTANT – To make sure all unit numbers are included for an address use the USPS ZIP Code Lookup tool to get all the unit ranges for an address. Enter just the street address and, if the address has units, the unit ranges will be listed in the results. NOTE – Some mixed-use buildings have both non-residential offices (suites) and residential apartments, usually on different floors. Examples: 1705 Main Street Suite 101, Baker City, OR 97814 1705 Main Street Suite 102, Baker City, OR 97814 1705 Main Street Suite 103, Baker City, OR 97814 1705 Main Street Suite 104, Baker City, OR 97814 1705 Main Street Suite 105, Baker City, OR 97814 1705 Main Street Suite 200, Baker City, OR 97814 1705 Main Street Suite 300, Baker City, OR 97814 1705 Main Street Suite 301, Baker City, OR 97814 1705 Main Street Suite 400, Baker City, OR 97814 1705 Main Street Suite 401, Baker City, OR 97814 1705 Main Street Suite 402, Baker City, OR 97814 1705 Main Street Suite 501, Baker City, OR 97814 1705 Main Street Suite 503, Baker City, OR 97814 1705 Main Street Apartment 600, Baker City, OR 97814 1705 Main Street Apartment 700, Baker City, OR 97814 1705 Main Street Apartment 701, Baker City, OR 97814 1705 Main Street Apartment 800, Baker City, OR 97814 1705 Main Street Apartment 801, Baker City, OR 97814 1705 Main Street Apartment 900, Baker City, OR 97814 NOTE – Some apartments do not have unit numbers but have individual street numbers instead. The Winter Garden Apartments in Salem has twelve units in two buildings, all with individual street numbers instead of unit numbers. |  |  |  |  |

References

Link

Idaho Site Structure Address Point Standard

Minnesota Geospatial Advisory Council Address Point Data Standard

Address Point Standard for Minnesota

NENA Next Generation 9-1-1 United States Civic Location Data Exchange Format Standard

NENA Standards for the Provisioning and Maintenance of GIS data to ECRFs and LVFs

NENA Standard for NG9-1-1 GIS Data Model

NENA GIS Data Transition Information Document

NENA Information Document for Development of Site/Structure Address Point GIS Data for 9-1-1

NENA Information Document for GIS Data Stewardship for Next Generation 9-1-1

Census MAF/TIGER Feature Class Code Definitions

Census TIGER/Line Shapefiles and TIGER/Line Files Technical Documentation

Census MAF/TIGER Feature Class Code (MTFCC) Definitions

Census Feature Name Directions

Census Feature Name Types

Census Feature Name Qualifiers

https://openaddresses.io/

Regrid Matched Address Schema

Regrid Oregon Data

Overture

Appendices

Appendix A - Acronyms

| Acronym | Description |
| --- | --- |
| ADU | Auxiliary Dwelling Unit |
| ALI | 9-1-1 Automatic Location Identification |
| CAD | 9-1-1 Computer Aided Dispatch |
| CASS | US Postal Service Coding Accuracy Support System |
| CDP | Census Designated Place |
| DAS | Oregon Department of Administrative Services |
| DPF | US Postal Service Delivery Point File |
| DPV | US Postal Service Delivery Point Validation |
| ESN | 9-1-1 Emergency Service Number |
| FGDC | Federal Geographic Data Committee |
| FIT | Framework Implementation Team |
| GEO | Geospatial Enterprise Operations |
| LACS | US Postal Service Locatable Address Conversion System |
| LUCA | Local Update of Census Addresses |
| MSAG | 9-1-1 Master Street Address Guide |
| NG9-1-1 | Next Generation 9-1-1 |
| NAD | National Address Database |
| NAD83 | North American Datum of 1983 |
| NENA | National Emergency Number Association |
| OEM | Oregon Department of Emergency Management |
| OGIC | Oregon Geographic Information Council |
| PSAP | 9-1-1 Public Safety Answering Point |
| RDI | US Postal Service Residential Delivery Indicator |
| SSAP | NENA Site Structure Address Point layer |
| URI | Uniform Resource Identifier |
| USPS | United States Postal Service |
| WGS84 | World Geodetic System of 1984 |
| ZIP | US Postal Service Zone Improvement Plan |
| ZCTA | Census ZIP Code Tabulation Area |

Appendix B - Workgroup Participants

| Participant | Organization |
| --- | --- |
| Alex Petzold | Oregon Department of Emergency Management |
| Alicia Wood | Oregon Metro |
| Christina Barrows | Lane Council of Governments |
| Erik Larson | Oregon Department of Forestry |
| Ethan Sharygin | Portland State University, Population Research Center |
| Hilary Leavell | City of Salem, Willamette Valley Communications |
| Jason Ford | National Address Database, US Department of Transportation |
| Jessica Beierman | GeoComm |
| Juliana Wold | Oregon Department of Emergency Management |
| Kurt Abe | City of Portland, Bureau of Development Services |
| Matt Williams | FIT Co-lead, Oregon Department of Geology and Mineral Industries |
| Melissa Foltz | Framework Coordinator, Oregon Department of Administrative Services |
| Paul Cone | City of Portland |
| Tim Esau | Oregon Secretary of State Elections |
| Tim Smothers | Baker County Planning Department |
| Tim VanDeWalle | Yamhill Communications Agency |
| Tom Elder | FIT Co-lead, Oregon Department of Administrative Services |

Appendix C - Implementation

Sample code and lookup tables for implementing the standard.

Oregon Address Point Data Standard – SQL Server data definition language

CREATE TABLE [dbo].[OREGON_ADDRESS_POINT_STANDARD] (

-- ADDRESS

[ADDRESS_ID]         [VARCHAR]( 36)     NULL

,[ADDRESS_FULL]       [VARCHAR](255)     NULL

-- STREET NUMBER

,[STREET_NUMBER_FULL] [VARCHAR]( 10)     NULL

,[Add_Number]         [INT]              NULL

,[AddNum_Pre]         [VARCHAR]( 15)     NULL

,[AddNum_Suf]         [VARCHAR]( 15)     NULL

-- STREET NAME

,[STREET_NAME_FULL]   [VARCHAR](254)     NULL

,[St_PreMod]          [VARCHAR]( 15)     NULL

,[St_PreDir]          [VARCHAR]( 10)     NULL

,[St_PreTyp]          [VARCHAR]( 50)     NULL

,[St_PreSep]          [VARCHAR]( 20)     NULL

,[St_Name]            [VARCHAR](254)     NULL

,[St_PosTyp]          [VARCHAR]( 50)     NULL

,[St_PosDir]          [VARCHAR]( 10)     NULL

,[St_PosMod]          [VARCHAR]( 25)     NULL

--SUBADDRESS

,[SUBADDRESS_FULL]    [VARCHAR]( 20)     NULL

,[SUBADDRESS_TYPE]    [VARCHAR]( 10)     NULL

,[SUBADDRESS_NUMBER]  [VARCHAR]( 10)     NULL

,[Unit]               [VARCHAR]( 75)     NULL

,[Building]           [VARCHAR]( 75)     NULL

,[Floor]              [VARCHAR]( 75)     NULL

,[Room]               [VARCHAR]( 75)     NULL

,[Seat]               [VARCHAR]( 75)     NULL

,[Addtl_Loc]          [VARCHAR](225)     NULL

-- LOCALITY

,[BUILDING_ID]        [VARCHAR]( 20)     NULL

,[PARCEL_ID]          [VARCHAR]( 20)     NULL

,[Post_Comm]          [VARCHAR]( 40)     NULL

,[Post_Code]          [VARCHAR](  7)     NULL

,[PostCodeEx]         [VARCHAR](  4)     NULL

,[Inc_Muni]           [VARCHAR](100) NOT NULL DEFAULT 'Unincorporated'

,[Uninc_Comm]         [VARCHAR](100)     NULL

,[Nbrhd_Comm]         [VARCHAR](100)     NULL

,[County]             [VARCHAR](100) NOT NULL

,[State]              [VARCHAR](  2) NOT NULL DEFAULT 'OR'

,[Country]            [VARCHAR](  2) NOT NULL DEFAULT 'US'

-- LOCATION

,[SHAPE]              [GEOMETRY]         NULL

,[OBJECTID]           [INT]          NOT NULL IDENTITY(1,1)

,[Longitude]          [FLOAT]            NULL

,[Latitude]           [FLOAT]            NULL

,[Elevation]          [INT]              NULL

,[Placement]          [VARCHAR]( 25)     NULL

-- EXTRA

,[MAIL]               [VARCHAR]( 20)     NULL

,[RESIDENTIAL]        [VARCHAR]( 20)     NULL

,[STAGE]              [VARCHAR]( 20)     NULL

,[Place_Type]         [VARCHAR]( 50)     NULL

,[LandmkName]         [VARCHAR](150)     NULL

,[AddDataURI]         [VARCHAR](254)     NULL

-- METADATA

,[DateUpdate]         [DATETIME]     NOT NULL DEFAULT GETDATE()

,[Effective]          [DATETIME]         NULL

,[Expire]             [DATETIME]         NULL

-- 911

,[DiscrpAgID]         [VARCHAR](100) NOT NULL

,[NGUID]              [VARCHAR](254) NOT NULL

,[LSt_PreDir]         [VARCHAR](  2)     NULL

,[LSt_Name]           [VARCHAR]( 75)     NULL

,[LSt_Typ]            [VARCHAR](  4)     NULL

,[LSt_PosDir]         [VARCHAR](  2)     NULL

,[MSAGComm]           [VARCHAR]( 30)     NULL

,[ESN]                [VARCHAR](  5)     NULL

,[Milepost]           [VARCHAR](150)     NULL);

| LOOKUP_COUNTY |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| DOR_ID | Integer |  | Oregon Dept. of Revenue county identifier. |
| STATE_FIPS | Text | 2 | Census state code. |
| COUNTY_FIPS | Text | 3 | Census county code. |
| GEOID | Text | 7 | Census unique combined state and county code. |
| County | Text | 100 | County name. |

| DOR_ID | STATE_FIPS | COUNTY_FIPS | GEOID | County |
| --- | --- | --- | --- | --- |
| 1 | 41 | 001 | 41001 | Baker |
| 2 | 41 | 003 | 41003 | Benton |
| 3 | 41 | 005 | 41005 | Clackamas |
| 4 | 41 | 007 | 41007 | Clatsop |
| 5 | 41 | 009 | 41009 | Columbia |
| 6 | 41 | 011 | 41011 | Coos |
| 7 | 41 | 013 | 41013 | Crook |
| 8 | 41 | 015 | 41015 | Curry |
| 9 | 41 | 017 | 41017 | Deschutes |
| 10 | 41 | 019 | 41019 | Douglas |
| 11 | 41 | 021 | 41021 | Gilliam |
| 12 | 41 | 023 | 41023 | Grant |
| 13 | 41 | 025 | 41025 | Harney |
| 14 | 41 | 027 | 41027 | Hood River |
| 15 | 41 | 029 | 41029 | Jackson |
| 16 | 41 | 031 | 41031 | Jefferson |
| 17 | 41 | 033 | 41033 | Josephine |
| 18 | 41 | 035 | 41035 | Klamath |
| 19 | 41 | 037 | 41037 | Lake |
| 20 | 41 | 039 | 41039 | Lane |
| 21 | 41 | 041 | 41041 | Lincoln |
| 22 | 41 | 043 | 41043 | Linn |
| 23 | 41 | 045 | 41045 | Malheur |
| 24 | 41 | 047 | 41047 | Marion |
| 25 | 41 | 049 | 41049 | Morrow |
| 26 | 41 | 051 | 41051 | Multnomah |
| 27 | 41 | 053 | 41053 | Polk |
| 28 | 41 | 055 | 41055 | Sherman |
| 29 | 41 | 057 | 41057 | Tillamook |
| 30 | 41 | 059 | 41059 | Umatilla |
| 31 | 41 | 061 | 41061 | Union |
| 32 | 41 | 063 | 41063 | Wallowa |
| 33 | 41 | 065 | 41065 | Wasco |
| 34 | 41 | 067 | 41067 | Washington |
| 35 | 41 | 069 | 41069 | Wheeler |
| 36 | 41 | 071 | 41071 | Yamhill |

| LOOKUP_INCORPORATED_COMMUNITY |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| STATE_FIPS | Text | 2 | Census state code. |
| PLACE_FIPS | Text | 3 | Census place code. |
| GEOID | Text | 7 | Census unique combined state and place code. |
| Inc_Muni | Text | 100 | Incorporated city or town name. |

| STATE_FIPS | PLACE_FIPS | GEOID | Inc_Muni |
| --- | --- | --- | --- |
| 41 | 00000 | 4100000 | Unincorporated |
| 41 | 00275 | 4100275 | Adair Village |
| 41 | 00350 | 4100350 | Adams |
| 41 | 00500 | 4100500 | Adrian |
| 41 | 01000 | 4101000 | Albany |
| 41 | 02000 | 4102000 | Amity |
| 41 | 02250 | 4102250 | Antelope |
| 41 | 02800 | 4102800 | Arlington |
| 41 | 03050 | 4103050 | Ashland |
| 41 | 03150 | 4103150 | Astoria |
| 41 | 03200 | 4103200 | Athena |
| 41 | 03250 | 4103250 | Aumsville |
| 41 | 03300 | 4103300 | Aurora |
| 41 | 03650 | 4103650 | Baker City |
| 41 | 03800 | 4103800 | Bandon |
| 41 | 03850 | 4103850 | Banks |
| 41 | 04000 | 4104000 | Barlow |
| 41 | 04800 | 4104800 | Bay City |
| 41 | 05350 | 4105350 | Beaverton |
| 41 | 05800 | 4105800 | Bend |
| 41 | 07200 | 4107200 | Boardman |
| 41 | 07300 | 4107300 | Bonanza |
| 41 | 08650 | 4108650 | Brookings |
| 41 | 09050 | 4109050 | Brownsville |
| 41 | 09800 | 4109800 | Burns |
| 41 | 10050 | 4110050 | Butte Falls |
| 41 | 10750 | 4110750 | Canby |
| 41 | 10850 | 4110850 | Cannon Beach |
| 41 | 10950 | 4110950 | Canyon City |
| 41 | 11000 | 4111000 | Canyonville |
| 41 | 11150 | 4111150 | Carlton |
| 41 | 11600 | 4111600 | Cascade Locks |
| 41 | 11850 | 4111850 | Cave Junction |
| 41 | 12400 | 4112400 | Central Point |
| 41 | 13050 | 4113050 | Chiloquin |
| 41 | 13750 | 4113750 | Clatskanie |
| 41 | 14400 | 4114400 | Coburg |
| 41 | 14750 | 4114750 | Columbia City |
| 41 | 15000 | 4115000 | Condon |
| 41 | 15250 | 4115250 | Coos Bay |
| 41 | 15350 | 4115350 | Coquille |
| 41 | 15550 | 4115550 | Cornelius |
| 41 | 15800 | 4115800 | Corvallis |
| 41 | 15950 | 4115950 | Cottage Grove |
| 41 | 16250 | 4116250 | Cove |
| 41 | 16950 | 4116950 | Creswell |
| 41 | 17300 | 4117300 | Culver |
| 41 | 17700 | 4117700 | Dallas |
| 41 | 18250 | 4118250 | Dayton |
| 41 | 18300 | 4118300 | Dayville |
| 41 | 18850 | 4118850 | Depoe Bay |
| 41 | 19100 | 4119100 | Detroit |
| 41 | 20100 | 4120100 | Donald |
| 41 | 20500 | 4120500 | Drain |
| 41 | 20900 | 4120900 | Dufur |
| 41 | 21050 | 4121050 | Dundee |
| 41 | 21150 | 4121150 | Dunes City |
| 41 | 21250 | 4121250 | Durham |
| 41 | 21550 | 4121550 | Eagle Point |
| 41 | 22200 | 4122200 | Echo |
| 41 | 22550 | 4122550 | Elgin |
| 41 | 22800 | 4122800 | Elkton |
| 41 | 23500 | 4123500 | Enterprise |
| 41 | 23800 | 4123800 | Estacada |
| 41 | 23850 | 4123850 | Eugene |
| 41 | 24250 | 4124250 | Fairview |
| 41 | 24550 | 4124550 | Falls City |
| 41 | 26050 | 4126050 | Florence |
| 41 | 26200 | 4126200 | Forest Grove |
| 41 | 26650 | 4126650 | Fossil |
| 41 | 28000 | 4128000 | Garibaldi |
| 41 | 28100 | 4128100 | Gaston |
| 41 | 28200 | 4128200 | Gates |
| 41 | 28450 | 4128450 | Gearhart |
| 41 | 28650 | 4128650 | Gervais |
| 41 | 29000 | 4129000 | Gladstone |
| 41 | 29350 | 4129350 | Glendale |
| 41 | 29900 | 4129900 | Gold Beach |
| 41 | 29950 | 4129950 | Gold Hill |
| 41 | 30500 | 4130500 | Granite |
| 41 | 30550 | 4130550 | Grants Pass |
| 41 | 30650 | 4130650 | Grass Valley |
| 41 | 31050 | 4131050 | Greenhorn |
| 41 | 31250 | 4131250 | Gresham |
| 41 | 31600 | 4131600 | Haines |
| 41 | 31650 | 4131650 | Halfway |
| 41 | 31750 | 4131750 | Halsey |
| 41 | 32050 | 4132050 | Happy Valley |
| 41 | 32550 | 4132550 | Harrisburg |
| 41 | 33250 | 4133250 | Helix |
| 41 | 33550 | 4133550 | Heppner |
| 41 | 33700 | 4133700 | Hermiston |
| 41 | 34100 | 4134100 | Hillsboro |
| 41 | 34250 | 4134250 | Hines |
| 41 | 34900 | 4134900 | Hood River |
| 41 | 35450 | 4135450 | Hubbard |
| 41 | 35700 | 4135700 | Huntington |
| 41 | 35800 | 4135800 | Idanha |
| 41 | 36050 | 4136050 | Imbler |
| 41 | 36150 | 4136150 | Independence |
| 41 | 36400 | 4136400 | Ione |
| 41 | 36500 | 4136500 | Irrigon |
| 41 | 36750 | 4136750 | Island City |
| 41 | 37000 | 4137000 | Jacksonville |
| 41 | 37250 | 4137250 | Jefferson |
| 41 | 37550 | 4137550 | John Day |
| 41 | 37650 | 4137650 | Johnson City |
| 41 | 37850 | 4137850 | Jordan Valley |
| 41 | 37900 | 4137900 | Joseph |
| 41 | 38000 | 4138000 | Junction City |
| 41 | 38500 | 4138500 | Keizer |
| 41 | 39150 | 4139150 | King City |
| 41 | 39700 | 4139700 | Klamath Falls |
| 41 | 40300 | 4140300 | Lafayette |
| 41 | 40350 | 4140350 | La Grande |
| 41 | 40550 | 4140550 | Lake Oswego |
| 41 | 40650 | 4140650 | Lakeside |
| 41 | 40700 | 4140700 | Lakeview |
| 41 | 41050 | 4141050 | La Pine |
| 41 | 41650 | 4141650 | Lebanon |
| 41 | 42200 | 4142200 | Lexington |
| 41 | 42600 | 4142600 | Lincoln City |
| 41 | 43400 | 4143400 | Lonerock |
| 41 | 43550 | 4143550 | Long Creek |
| 41 | 43900 | 4143900 | Lostine |
| 41 | 44050 | 4144050 | Lowell |
| 41 | 44300 | 4144300 | Lyons |
| 41 | 45000 | 4145000 | McMinnville |
| 41 | 45250 | 4145250 | Madras |
| 41 | 45400 | 4145400 | Malin |
| 41 | 45700 | 4145700 | Manzanita |
| 41 | 46500 | 4146500 | Maupin |
| 41 | 46730 | 4146730 | Maywood Park |
| 41 | 47000 | 4147000 | Medford |
| 41 | 47700 | 4147700 | Merrill |
| 41 | 47750 | 4147750 | Metolius |
| 41 | 48150 | 4148150 | Mill City |
| 41 | 48300 | 4148300 | Millersburg |
| 41 | 48600 | 4148600 | Milton-Freewater |
| 41 | 48650 | 4148650 | Milwaukie |
| 41 | 49150 | 4149150 | Mitchell |
| 41 | 49450 | 4149450 | Molalla |
| 41 | 49550 | 4149550 | Monmouth |
| 41 | 49600 | 4149600 | Monroe |
| 41 | 49750 | 4149750 | Monument |
| 41 | 50000 | 4150000 | Moro |
| 41 | 50050 | 4150050 | Mosier |
| 41 | 50150 | 4150150 | Mount Angel |
| 41 | 50250 | 4150250 | Mount Vernon |
| 41 | 50950 | 4150950 | Myrtle Creek |
| 41 | 51050 | 4151050 | Myrtle Point |
| 41 | 51700 | 4151700 | Nehalem |
| 41 | 52100 | 4152100 | Newberg |
| 41 | 52450 | 4152450 | Newport |
| 41 | 53000 | 4153000 | North Bend |
| 41 | 53150 | 4153150 | North Plains |
| 41 | 53300 | 4153300 | North Powder |
| 41 | 53750 | 4153750 | Nyssa |
| 41 | 54000 | 4154000 | Oakland |
| 41 | 54100 | 4154100 | Oakridge |
| 41 | 54900 | 4154900 | Ontario |
| 41 | 55200 | 4155200 | Oregon City |
| 41 | 56250 | 4156250 | Paisley |
| 41 | 57150 | 4157150 | Pendleton |
| 41 | 57450 | 4157450 | Philomath |
| 41 | 57500 | 4157500 | Phoenix |
| 41 | 57650 | 4157650 | Pilot Rock |
| 41 | 59000 | 4159000 | Portland |
| 41 | 59250 | 4159250 | Port Orford |
| 41 | 59600 | 4159600 | Powers |
| 41 | 59650 | 4159650 | Prairie City |
| 41 | 59750 | 4159750 | Prescott |
| 41 | 59850 | 4159850 | Prineville |
| 41 | 60850 | 4160850 | Rainier |
| 41 | 61200 | 4161200 | Redmond |
| 41 | 61300 | 4161300 | Reedsport |
| 41 | 61700 | 4161700 | Richland |
| 41 | 61850 | 4161850 | Riddle |
| 41 | 62250 | 4162250 | Rivergrove |
| 41 | 62900 | 4162900 | Rockaway Beach |
| 41 | 63450 | 4163450 | Rogue River |
| 41 | 63650 | 4163650 | Roseburg |
| 41 | 64200 | 4164200 | Rufus |
| 41 | 64600 | 4164600 | Saint Helens |
| 41 | 64850 | 4164850 | Saint Paul |
| 41 | 64900 | 4164900 | Salem |
| 41 | 65250 | 4165250 | Sandy |
| 41 | 65500 | 4165500 | Scappoose |
| 41 | 65650 | 4165650 | Scio |
| 41 | 65800 | 4165800 | Scotts Mills |
| 41 | 65950 | 4165950 | Seaside |
| 41 | 66200 | 4166200 | Seneca |
| 41 | 66550 | 4166550 | Shady Cove |
| 41 | 66700 | 4166700 | Shaniko |
| 41 | 67050 | 4167050 | Sheridan |
| 41 | 67100 | 4167100 | Sherwood |
| 41 | 67500 | 4167500 | Siletz |
| 41 | 67650 | 4167650 | Silverton |
| 41 | 67950 | 4167950 | Sisters |
| 41 | 68550 | 4168550 | Sodaville |
| 41 | 69450 | 4169450 | Spray |
| 41 | 69600 | 4169600 | Springfield |
| 41 | 69900 | 4169900 | Stanfield |
| 41 | 70200 | 4170200 | Stayton |
| 41 | 70700 | 4170700 | Sublimity |
| 41 | 70850 | 4170850 | Summerville |
| 41 | 71000 | 4171000 | Sumpter |
| 41 | 71650 | 4171650 | Sutherlin |
| 41 | 71950 | 4171950 | Sweet Home |
| 41 | 72500 | 4172500 | Talent |
| 41 | 72600 | 4172600 | Tangent |
| 41 | 72950 | 4172950 | The Dalles |
| 41 | 73650 | 4173650 | Tigard |
| 41 | 73700 | 4173700 | Tillamook |
| 41 | 74000 | 4174000 | Toledo |
| 41 | 74850 | 4174850 | Troutdale |
| 41 | 74950 | 4174950 | Tualatin |
| 41 | 75150 | 4175150 | Turner |
| 41 | 75550 | 4175550 | Ukiah |
| 41 | 75650 | 4175650 | Umatilla |
| 41 | 75850 | 4175850 | Union |
| 41 | 76250 | 4176250 | Unity |
| 41 | 76600 | 4176600 | Vale |
| 41 | 77050 | 4177050 | Veneta |
| 41 | 77250 | 4177250 | Vernonia |
| 41 | 78000 | 4178000 | Waldport |
| 41 | 78150 | 4178150 | Wallowa |
| 41 | 78900 | 4178900 | Warrenton |
| 41 | 78950 | 4178950 | Wasco |
| 41 | 79050 | 4179050 | Waterloo |
| 41 | 79950 | 4179950 | Westfir |
| 41 | 80150 | 4180150 | West Linn |
| 41 | 80350 | 4180350 | Weston |
| 41 | 81300 | 4181300 | Wheeler |
| 41 | 82350 | 4182350 | Willamina |
| 41 | 82800 | 4182800 | Wilsonville |
| 41 | 83400 | 4183400 | Winston |
| 41 | 83750 | 4183750 | Woodburn |
| 41 | 83950 | 4183950 | Wood Village |
| 41 | 84200 | 4184200 | Yachats |
| 41 | 84250 | 4184250 | Yamhill |
| 41 | 84600 | 4184600 | Yoncalla |

| LOOKUP_MAIL |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| MAIL | Text | 1 | US Postal Service Delivery Point Validation (DPV) code. |
| DESCRIPTION | Text | 255 | Brief description for each code. |

| MAIL | DESCRIPTION |
| --- | --- |
| D | Match street address but unit address is missing. |
| N | No match for the street address. |
| S | Match street address but unit address is incorrect. |
| X | No match for the street address. |
| Y | Match street address and unit address (if present). |

| LOOKUP_PLACEMENT |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| Placement | Text | 1 | Term describing the placement method of the point location. |
| QUALITY | Text | 10 | Relative quality rating of the location accuracy. |
| DESCRIPTION | Text | 255 | Brief description for each method. |

| PLACEMENT | QUALITY | DESCRIPTION |
| --- | --- | --- |
| Geocoding | Low | Road centerline |
| Parcel | Fair | Parcel centroid |
| PropertyAccess | Good | Access point to the property |
| Site | Fair | No defined boundary or building footprint |
| Structure | Good | Building footprint centroid |
| Subaddress | Best | Unit within a building footprint |
| Unknown | None | Unknown |

| LOOKUP_POSTAL_CODE |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| Post_Code | Text | 5 | US Postal Service ZIP Code in Oregon. |
| Post_Comm | Text | 100 | US Postal Service preferred city name for the ZIP Code. |
| Post_Comm2 | Text | 100 | US Postal Service acceptable alternate city name. |
| Post_Comm3 | Text | 100 | US Postal Service another acceptable alternate city name. |
| State | Text | 2 | US Postal Service state abbreviation. |
| ZIP_TYPE | Text | 20 | Type of use for the ZIP Code. |

| Post_Code | Post_Comm | Post_Comm2 | Post_Comm3 | State | ZIP_TYPE |
| --- | --- | --- | --- | --- | --- |
| 97001 | Antelope |  |  | OR | PO BOX ONLY |
| 97002 | Aurora |  |  | OR | STANDARD |
| 97003 | Beaverton | Aloha | Hillsboro | OR | STANDARD |
| 97004 | Beavercreek |  |  | OR | STANDARD |
| 97005 | Beaverton |  |  | OR | STANDARD |
| 97006 | Beaverton | Aloha | Hillsboro | OR | STANDARD |
| 97007 | Beaverton | Aloha |  | OR | STANDARD |
| 97008 | Beaverton |  |  | OR | STANDARD |
| 97009 | Boring | Damascus |  | OR | STANDARD |
| 97010 | Bridal Veil |  |  | OR | PO BOX ONLY |
| 97011 | Brightwood |  |  | OR | STANDARD |
| 97013 | Canby |  |  | OR | STANDARD |
| 97014 | Cascade Locks |  |  | OR | STANDARD |
| 97015 | Clackamas | Damascus | Happy Valley | OR | STANDARD |
| 97016 | Clatskanie | Birkenfeld | Westport | OR | STANDARD |
| 97017 | Colton |  |  | OR | STANDARD |
| 97018 | Columbia City |  |  | OR | STANDARD |
| 97019 | Corbett |  |  | OR | STANDARD |
| 97020 | Donald |  |  | OR | STANDARD |
| 97021 | Dufur | Friend |  | OR | STANDARD |
| 97022 | Eagle Creek |  |  | OR | STANDARD |
| 97023 | Estacada |  |  | OR | STANDARD |
| 97024 | Fairview |  |  | OR | STANDARD |
| 97026 | Gervais |  |  | OR | STANDARD |
| 97027 | Gladstone |  |  | OR | STANDARD |
| 97028 | Government Camp | Timberline Lodge |  | OR | STANDARD |
| 97029 | Grass Valley |  |  | OR | STANDARD |
| 97030 | Gresham |  |  | OR | STANDARD |
| 97031 | Hood River |  |  | OR | STANDARD |
| 97032 | Hubbard |  |  | OR | STANDARD |
| 97033 | Kent |  |  | OR | PO BOX ONLY |
| 97034 | Lake Oswego |  |  | OR | STANDARD |
| 97035 | Lake Oswego | Lake Grove |  | OR | STANDARD |
| 97036 | Marylhurst |  |  | OR | PO BOX ONLY |
| 97037 | Maupin |  |  | OR | STANDARD |
| 97038 | Molalla |  |  | OR | STANDARD |
| 97039 | Moro |  |  | OR | STANDARD |
| 97040 | Mosier |  |  | OR | STANDARD |
| 97041 | Mount Hood Parkdale |  |  | OR | STANDARD |
| 97042 | Mulino |  |  | OR | STANDARD |
| 97044 | Odell |  |  | OR | PO BOX ONLY |
| 97045 | Oregon City |  |  | OR | STANDARD |
| 97048 | Rainier | Prescott |  | OR | STANDARD |
| 97049 | Rhododendron | Zigzag |  | OR | STANDARD |
| 97050 | Rufus |  |  | OR | PO BOX ONLY |
| 97051 | Saint Helens |  |  | OR | STANDARD |
| 97053 | Warren |  |  | OR | STANDARD |
| 97054 | Deer Island |  |  | OR | STANDARD |
| 97055 | Sandy |  |  | OR | STANDARD |
| 97056 | Scappoose |  |  | OR | STANDARD |
| 97057 | Shaniko |  |  | OR | PO BOX ONLY |
| 97058 | The Dalles |  |  | OR | STANDARD |
| 97060 | Troutdale | Wood Village |  | OR | STANDARD |
| 97062 | Tualatin |  |  | OR | STANDARD |
| 97063 | Tygh Valley | Wamic |  | OR | STANDARD |
| 97064 | Vernonia |  |  | OR | STANDARD |
| 97065 | Wasco |  |  | OR | STANDARD |
| 97067 | Welches |  |  | OR | STANDARD |
| 97068 | West Linn |  |  | OR | STANDARD |
| 97070 | Wilsonville |  |  | OR | STANDARD |
| 97071 | Woodburn |  |  | OR | STANDARD |
| 97075 | Beaverton |  |  | OR | PO BOX ONLY |
| 97076 | Beaverton |  |  | OR | PO BOX ONLY |
| 97077 | Beaverton |  |  | OR | UNIQUE |
| 97078 | Beaverton | Aloha |  | OR | STANDARD |
| 97079 | Beaverton |  |  | OR | UNIQUE |
| 97080 | Gresham |  |  | OR | STANDARD |
| 97086 | Happy Valley | Clackamas | Portland | OR | STANDARD |
| 97089 | Damascus | Boring |  | OR | STANDARD |
| 97101 | Amity |  |  | OR | STANDARD |
| 97102 | Arch Cape |  |  | OR | STANDARD |
| 97103 | Astoria |  |  | OR | STANDARD |
| 97106 | Banks |  |  | OR | STANDARD |
| 97107 | Bay City |  |  | OR | STANDARD |
| 97108 | Beaver |  |  | OR | STANDARD |
| 97109 | Buxton | Banks |  | OR | STANDARD |
| 97110 | Cannon Beach |  |  | OR | PO BOX ONLY |
| 97111 | Carlton |  |  | OR | STANDARD |
| 97112 | Cloverdale | Beaver |  | OR | STANDARD |
| 97113 | Cornelius |  |  | OR | STANDARD |
| 97114 | Dayton |  |  | OR | STANDARD |
| 97115 | Dundee |  |  | OR | STANDARD |
| 97116 | Forest Grove | Glenwood |  | OR | STANDARD |
| 97117 | Gales Creek |  |  | OR | STANDARD |
| 97118 | Garibaldi |  |  | OR | PO BOX ONLY |
| 97119 | Gaston |  |  | OR | STANDARD |
| 97121 | Hammond |  |  | OR | STANDARD |
| 97122 | Hebo |  |  | OR | STANDARD |
| 97123 | Hillsboro | Cornelius |  | OR | STANDARD |
| 97124 | Hillsboro | Cornelius |  | OR | STANDARD |
| 97125 | Manning | Banks |  | OR | STANDARD |
| 97127 | Lafayette |  |  | OR | STANDARD |
| 97128 | McMinnville |  |  | OR | STANDARD |
| 97129 | Hillsboro |  |  | OR | UNIQUE |
| 97130 | Manzanita |  |  | OR | PO BOX ONLY |
| 97131 | Nehalem |  |  | OR | STANDARD |
| 97132 | Newberg |  |  | OR | STANDARD |
| 97133 | North Plains |  |  | OR | STANDARD |
| 97134 | Oceanside |  |  | OR | PO BOX ONLY |
| 97135 | Pacific City |  |  | OR | PO BOX ONLY |
| 97136 | Rockaway Beach |  |  | OR | STANDARD |
| 97137 | Saint Paul |  |  | OR | STANDARD |
| 97138 | Seaside | Gearhart |  | OR | STANDARD |
| 97140 | Sherwood |  |  | OR | STANDARD |
| 97141 | Tillamook |  |  | OR | STANDARD |
| 97143 | Netarts | Netarts Bay |  | OR | PO BOX ONLY |
| 97144 | Timber |  |  | OR | STANDARD |
| 97145 | Tolovana Park |  |  | OR | PO BOX ONLY |
| 97146 | Warrenton |  |  | OR | STANDARD |
| 97147 | Wheeler |  |  | OR | PO BOX ONLY |
| 97148 | Yamhill |  |  | OR | STANDARD |
| 97149 | Neskowin |  |  | OR | STANDARD |
| 97201 | Portland |  |  | OR | STANDARD |
| 97202 | Portland |  |  | OR | STANDARD |
| 97203 | Portland |  |  | OR | STANDARD |
| 97204 | Portland |  |  | OR | STANDARD |
| 97205 | Portland |  |  | OR | STANDARD |
| 97206 | Portland |  |  | OR | STANDARD |
| 97207 | Portland |  |  | OR | PO BOX ONLY |
| 97208 | Portland |  |  | OR | PO BOX ONLY |
| 97209 | Portland |  |  | OR | STANDARD |
| 97210 | Portland |  |  | OR | STANDARD |
| 97211 | Portland |  |  | OR | STANDARD |
| 97212 | Portland |  |  | OR | STANDARD |
| 97213 | Portland |  |  | OR | STANDARD |
| 97214 | Portland |  |  | OR | STANDARD |
| 97215 | Portland |  |  | OR | STANDARD |
| 97216 | Portland |  |  | OR | STANDARD |
| 97217 | Portland |  |  | OR | STANDARD |
| 97218 | Portland |  |  | OR | STANDARD |
| 97219 | Portland |  |  | OR | STANDARD |
| 97220 | Portland |  |  | OR | STANDARD |
| 97221 | Portland |  |  | OR | STANDARD |
| 97222 | Portland | Milwaukie | Oak Grove | OR | STANDARD |
| 97223 | Portland | Tigard |  | OR | STANDARD |
| 97224 | Portland | King City | Tigard | OR | STANDARD |
| 97225 | Portland |  |  | OR | STANDARD |
| 97227 | Portland |  |  | OR | STANDARD |
| 97228 | Portland |  |  | OR | PO BOX ONLY |
| 97229 | Portland |  |  | OR | STANDARD |
| 97230 | Portland |  |  | OR | STANDARD |
| 97231 | Portland |  |  | OR | STANDARD |
| 97232 | Portland |  |  | OR | STANDARD |
| 97233 | Portland |  |  | OR | STANDARD |
| 97236 | Portland |  |  | OR | STANDARD |
| 97238 | Portland |  |  | OR | PO BOX ONLY |
| 97239 | Portland |  |  | OR | STANDARD |
| 97240 | Portland |  |  | OR | PO BOX ONLY |
| 97242 | Portland |  |  | OR | PO BOX ONLY |
| 97250 | Portland |  |  | OR | UNIQUE |
| 97251 | Portland |  |  | OR | UNIQUE |
| 97252 | Portland |  |  | OR | UNIQUE |
| 97253 | Portland |  |  | OR | UNIQUE |
| 97254 | Portland |  |  | OR | UNIQUE |
| 97256 | Portland |  |  | OR | UNIQUE |
| 97266 | Portland |  |  | OR | STANDARD |
| 97267 | Portland | Milwaukie | Oak Grove | OR | STANDARD |
| 97268 | Portland | Oak Grove |  | OR | PO BOX ONLY |
| 97269 | Portland | Milwaukie |  | OR | PO BOX ONLY |
| 97280 | Portland |  |  | OR | PO BOX ONLY |
| 97281 | Portland | Tigard |  | OR | PO BOX ONLY |
| 97282 | Portland |  |  | OR | PO BOX ONLY |
| 97283 | Portland |  |  | OR | PO BOX ONLY |
| 97286 | Portland |  |  | OR | PO BOX ONLY |
| 97290 | Portland |  |  | OR | PO BOX ONLY |
| 97291 | Portland |  |  | OR | PO BOX ONLY |
| 97292 | Portland |  |  | OR | PO BOX ONLY |
| 97293 | Portland |  |  | OR | PO BOX ONLY |
| 97294 | Portland |  |  | OR | PO BOX ONLY |
| 97296 | Portland |  |  | OR | PO BOX ONLY |
| 97298 | Portland |  |  | OR | PO BOX ONLY |
| 97301 | Salem |  |  | OR | STANDARD |
| 97302 | Salem |  |  | OR | STANDARD |
| 97303 | Salem | Keizer |  | OR | STANDARD |
| 97304 | Salem |  |  | OR | STANDARD |
| 97305 | Salem | Brooks |  | OR | STANDARD |
| 97306 | Salem |  |  | OR | STANDARD |
| 97307 | Keizer | Salem |  | OR | PO BOX ONLY |
| 97308 | Salem |  |  | OR | PO BOX ONLY |
| 97309 | Salem |  |  | OR | PO BOX ONLY |
| 97310 | Salem |  |  | OR | UNIQUE |
| 97311 | Salem |  |  | OR | UNIQUE |
| 97312 | Salem |  |  | OR | UNIQUE |
| 97314 | Salem |  |  | OR | UNIQUE |
| 97317 | Salem |  |  | OR | STANDARD |
| 97321 | Albany | Millersburg |  | OR | STANDARD |
| 97322 | Albany |  |  | OR | STANDARD |
| 97324 | Alsea |  |  | OR | STANDARD |
| 97325 | Aumsville | West Stayton |  | OR | STANDARD |
| 97326 | Blodgett |  |  | OR | STANDARD |
| 97327 | Brownsville |  |  | OR | STANDARD |
| 97329 | Cascadia |  |  | OR | STANDARD |
| 97330 | Corvallis | Adair Village |  | OR | STANDARD |
| 97331 | Corvallis |  |  | OR | STANDARD |
| 97333 | Corvallis |  |  | OR | STANDARD |
| 97335 | Crabtree |  |  | OR | PO BOX ONLY |
| 97336 | Crawfordsville |  |  | OR | PO BOX ONLY |
| 97338 | Dallas |  |  | OR | STANDARD |
| 97339 | Corvallis |  |  | OR | PO BOX ONLY |
| 97341 | Depoe Bay |  |  | OR | STANDARD |
| 97342 | Detroit |  |  | OR | PO BOX ONLY |
| 97343 | Eddyville |  |  | OR | STANDARD |
| 97344 | Falls City |  |  | OR | STANDARD |
| 97345 | Foster |  |  | OR | STANDARD |
| 97346 | Gates |  |  | OR | STANDARD |
| 97347 | Grand Ronde |  |  | OR | STANDARD |
| 97348 | Halsey |  |  | OR | STANDARD |
| 97350 | Idanha |  |  | OR | STANDARD |
| 97351 | Independence |  |  | OR | STANDARD |
| 97352 | Jefferson |  |  | OR | STANDARD |
| 97355 | Lebanon |  |  | OR | STANDARD |
| 97357 | Logsden |  |  | OR | STANDARD |
| 97358 | Lyons |  |  | OR | STANDARD |
| 97360 | Mill City |  |  | OR | STANDARD |
| 97361 | Monmouth |  |  | OR | STANDARD |
| 97362 | Mount Angel |  |  | OR | STANDARD |
| 97364 | Neotsu |  |  | OR | STANDARD |
| 97365 | Newport |  |  | OR | STANDARD |
| 97366 | South Beach | Newport |  | OR | STANDARD |
| 97367 | Lincoln City | Rose Lodge |  | OR | STANDARD |
| 97368 | Otis |  |  | OR | STANDARD |
| 97369 | Otter Rock |  |  | OR | STANDARD |
| 97370 | Philomath |  |  | OR | STANDARD |
| 97371 | Rickreall |  |  | OR | STANDARD |
| 97373 | Saint Benedict |  |  | OR | PO BOX ONLY |
| 97374 | Scio |  |  | OR | STANDARD |
| 97375 | Scotts Mills |  |  | OR | STANDARD |
| 97376 | Seal Rock |  |  | OR | STANDARD |
| 97377 | Shedd |  |  | OR | STANDARD |
| 97378 | Sheridan |  |  | OR | STANDARD |
| 97380 | Siletz |  |  | OR | STANDARD |
| 97381 | Silverton |  |  | OR | STANDARD |
| 97383 | Stayton |  |  | OR | STANDARD |
| 97384 | Mehama |  |  | OR | PO BOX ONLY |
| 97385 | Sublimity |  |  | OR | STANDARD |
| 97386 | Sweet Home |  |  | OR | STANDARD |
| 97388 | Gleneden Beach |  |  | OR | PO BOX ONLY |
| 97389 | Tangent |  |  | OR | STANDARD |
| 97390 | Tidewater |  |  | OR | STANDARD |
| 97391 | Toledo |  |  | OR | STANDARD |
| 97392 | Turner | Marion |  | OR | STANDARD |
| 97394 | Waldport |  |  | OR | STANDARD |
| 97396 | Willamina |  |  | OR | STANDARD |
| 97401 | Eugene |  |  | OR | STANDARD |
| 97402 | Eugene |  |  | OR | STANDARD |
| 97403 | Eugene |  |  | OR | STANDARD |
| 97404 | Eugene |  |  | OR | STANDARD |
| 97405 | Eugene |  |  | OR | STANDARD |
| 97406 | Agness |  |  | OR | STANDARD |
| 97407 | Allegany |  |  | OR | PO BOX ONLY |
| 97408 | Eugene | Coburg |  | OR | STANDARD |
| 97409 | Alvadore |  |  | OR | PO BOX ONLY |
| 97410 | Azalea |  |  | OR | STANDARD |
| 97411 | Bandon |  |  | OR | STANDARD |
| 97412 | Blachly | Eugene |  | OR | STANDARD |
| 97413 | Blue River | McKenzie Bridge |  | OR | STANDARD |
| 97414 | Broadbent |  |  | OR | STANDARD |
| 97415 | Brookings | Harbor |  | OR | STANDARD |
| 97416 | Camas Valley |  |  | OR | STANDARD |
| 97417 | Canyonville |  |  | OR | STANDARD |
| 97419 | Cheshire |  |  | OR | STANDARD |
| 97420 | Coos Bay | Charleston |  | OR | STANDARD |
| 97423 | Coquille |  |  | OR | STANDARD |
| 97424 | Cottage Grove | Curtin | Saginaw | OR | STANDARD |
| 97426 | Creswell |  |  | OR | STANDARD |
| 97429 | Days Creek |  |  | OR | STANDARD |
| 97430 | Deadwood | Greenleaf |  | OR | STANDARD |
| 97431 | Dexter |  |  | OR | STANDARD |
| 97432 | Dillard |  |  | OR | PO BOX ONLY |
| 97434 | Dorena | Culp Creek |  | OR | STANDARD |
| 97435 | Drain |  |  | OR | STANDARD |
| 97436 | Elkton |  |  | OR | STANDARD |
| 97437 | Elmira |  |  | OR | STANDARD |
| 97438 | Fall Creek | Jasper |  | OR | STANDARD |
| 97439 | Florence | Dunes City |  | OR | STANDARD |
| 97440 | Eugene |  |  | OR | PO BOX ONLY |
| 97441 | Gardiner |  |  | OR | STANDARD |
| 97442 | Glendale |  |  | OR | STANDARD |
| 97443 | Glide |  |  | OR | STANDARD |
| 97444 | Gold Beach |  |  | OR | STANDARD |
| 97446 | Harrisburg |  |  | OR | STANDARD |
| 97447 | Idleyld Park |  |  | OR | STANDARD |
| 97448 | Junction City |  |  | OR | STANDARD |
| 97449 | Lakeside |  |  | OR | STANDARD |
| 97450 | Langlois |  |  | OR | STANDARD |
| 97451 | Lorane |  |  | OR | STANDARD |
| 97452 | Lowell |  |  | OR | STANDARD |
| 97453 | Mapleton |  |  | OR | STANDARD |
| 97454 | Marcola |  |  | OR | STANDARD |
| 97455 | Pleasant Hill | Eugene |  | OR | STANDARD |
| 97456 | Monroe |  |  | OR | STANDARD |
| 97457 | Myrtle Creek |  |  | OR | STANDARD |
| 97458 | Myrtle Point | Norway | Remote | OR | STANDARD |
| 97459 | North Bend |  |  | OR | STANDARD |
| 97461 | Noti |  |  | OR | STANDARD |
| 97462 | Oakland |  |  | OR | STANDARD |
| 97463 | Oakridge |  |  | OR | STANDARD |
| 97464 | Ophir |  |  | OR | PO BOX ONLY |
| 97465 | Port Orford |  |  | OR | STANDARD |
| 97466 | Powers |  |  | OR | STANDARD |
| 97467 | Reedsport | Winchester Bay |  | OR | STANDARD |
| 97469 | Riddle |  |  | OR | STANDARD |
| 97470 | Roseburg |  |  | OR | STANDARD |
| 97471 | Roseburg |  |  | OR | STANDARD |
| 97473 | Scottsburg |  |  | OR | STANDARD |
| 97475 | Springfield |  |  | OR | PO BOX ONLY |
| 97476 | Sixes |  |  | OR | STANDARD |
| 97477 | Springfield |  |  | OR | STANDARD |
| 97478 | Springfield |  |  | OR | STANDARD |
| 97479 | Sutherlin |  |  | OR | STANDARD |
| 97480 | Swisshome |  |  | OR | STANDARD |
| 97481 | Tenmile |  |  | OR | STANDARD |
| 97484 | Tiller |  |  | OR | STANDARD |
| 97486 | Umpqua |  |  | OR | STANDARD |
| 97487 | Veneta |  |  | OR | STANDARD |
| 97488 | Vida |  |  | OR | STANDARD |
| 97489 | Walterville | Leaburg |  | OR | STANDARD |
| 97490 | Walton |  |  | OR | STANDARD |
| 97491 | Wedderburn |  |  | OR | PO BOX ONLY |
| 97492 | Westfir |  |  | OR | STANDARD |
| 97493 | Westlake |  |  | OR | STANDARD |
| 97494 | Wilbur |  |  | OR | PO BOX ONLY |
| 97495 | Winchester |  |  | OR | STANDARD |
| 97496 | Winston |  |  | OR | STANDARD |
| 97497 | Wolf Creek |  |  | OR | STANDARD |
| 97498 | Yachats |  |  | OR | STANDARD |
| 97499 | Yoncalla |  |  | OR | STANDARD |
| 97501 | Medford |  |  | OR | STANDARD |
| 97502 | Central Point | Medford |  | OR | STANDARD |
| 97503 | White City | Medford |  | OR | STANDARD |
| 97504 | Medford |  |  | OR | STANDARD |
| 97520 | Ashland |  |  | OR | STANDARD |
| 97522 | Butte Falls |  |  | OR | STANDARD |
| 97523 | Cave Junction |  |  | OR | STANDARD |
| 97524 | Eagle Point |  |  | OR | STANDARD |
| 97525 | Gold Hill |  |  | OR | STANDARD |
| 97526 | Grants Pass |  |  | OR | STANDARD |
| 97527 | Grants Pass |  |  | OR | STANDARD |
| 97528 | Grants Pass |  |  | OR | PO BOX ONLY |
| 97530 | Jacksonville | Applegate |  | OR | STANDARD |
| 97531 | Kerby | Cave Junction |  | OR | STANDARD |
| 97532 | Merlin |  |  | OR | STANDARD |
| 97533 | Murphy |  |  | OR | PO BOX ONLY |
| 97534 | O Brien |  |  | OR | STANDARD |
| 97535 | Phoenix |  |  | OR | STANDARD |
| 97536 | Prospect |  |  | OR | STANDARD |
| 97537 | Rogue River |  |  | OR | STANDARD |
| 97538 | Selma |  |  | OR | STANDARD |
| 97539 | Shady Cove |  |  | OR | STANDARD |
| 97540 | Talent |  |  | OR | STANDARD |
| 97541 | Trail |  |  | OR | STANDARD |
| 97543 | Wilderville | Grants Pass |  | OR | STANDARD |
| 97544 | Williams |  |  | OR | STANDARD |
| 97601 | Klamath Falls | Oretech |  | OR | STANDARD |
| 97602 | Klamath Falls |  |  | OR | PO BOX ONLY |
| 97603 | Klamath Falls |  |  | OR | STANDARD |
| 97604 | Crater Lake |  |  | OR | PO BOX ONLY |
| 97620 | Adel |  |  | OR | PO BOX ONLY |
| 97621 | Beatty |  |  | OR | STANDARD |
| 97622 | Bly |  |  | OR | PO BOX ONLY |
| 97623 | Bonanza |  |  | OR | STANDARD |
| 97624 | Chiloquin |  |  | OR | STANDARD |
| 97625 | Dairy | Klamath Falls |  | OR | STANDARD |
| 97626 | Fort Klamath |  |  | OR | PO BOX ONLY |
| 97627 | Keno |  |  | OR | STANDARD |
| 97630 | Lakeview |  |  | OR | STANDARD |
| 97632 | Malin |  |  | OR | STANDARD |
| 97633 | Merrill |  |  | OR | STANDARD |
| 97634 | Midland |  |  | OR | PO BOX ONLY |
| 97635 | New Pine Creek |  |  | OR | STANDARD |
| 97636 | Paisley |  |  | OR | STANDARD |
| 97637 | Plush |  |  | OR | STANDARD |
| 97638 | Silver Lake |  |  | OR | STANDARD |
| 97639 | Sprague River | Chiloquin |  | OR | STANDARD |
| 97640 | Summer Lake |  |  | OR | STANDARD |
| 97641 | Christmas Valley |  |  | OR | PO BOX ONLY |
| 97701 | Bend |  |  | OR | STANDARD |
| 97702 | Bend |  |  | OR | STANDARD |
| 97703 | Bend |  |  | OR | STANDARD |
| 97707 | Bend | Sunriver |  | OR | STANDARD |
| 97708 | Bend |  |  | OR | PO BOX ONLY |
| 97709 | Bend |  |  | OR | PO BOX ONLY |
| 97710 | Fields |  |  | OR | STANDARD |
| 97711 | Ashwood |  |  | OR | STANDARD |
| 97712 | Brothers |  |  | OR | STANDARD |
| 97720 | Burns | Lawen |  | OR | STANDARD |
| 97721 | Princeton |  |  | OR | STANDARD |
| 97722 | Diamond |  |  | OR | STANDARD |
| 97730 | Camp Sherman |  |  | OR | STANDARD |
| 97731 | Chemult | Diamond Lake |  | OR | STANDARD |
| 97732 | Crane |  |  | OR | PO BOX ONLY |
| 97733 | Crescent | Crescent Lake |  | OR | STANDARD |
| 97734 | Culver |  |  | OR | STANDARD |
| 97735 | Fort Rock |  |  | OR | PO BOX ONLY |
| 97736 | Frenchglen |  |  | OR | STANDARD |
| 97737 | Gilchrist |  |  | OR | STANDARD |
| 97738 | Hines |  |  | OR | STANDARD |
| 97739 | La Pine |  |  | OR | STANDARD |
| 97741 | Madras | Metolius |  | OR | STANDARD |
| 97750 | Mitchell |  |  | OR | STANDARD |
| 97751 | Paulina |  |  | OR | STANDARD |
| 97752 | Post |  |  | OR | STANDARD |
| 97753 | Powell Butte |  |  | OR | STANDARD |
| 97754 | Prineville |  |  | OR | STANDARD |
| 97756 | Redmond |  |  | OR | STANDARD |
| 97758 | Riley |  |  | OR | STANDARD |
| 97759 | Sisters |  |  | OR | STANDARD |
| 97760 | Terrebonne | Crooked River Ranch |  | OR | STANDARD |
| 97761 | Warm Springs |  |  | OR | PO BOX ONLY |
| 97801 | Pendleton | Cayuse |  | OR | STANDARD |
| 97810 | Adams |  |  | OR | STANDARD |
| 97812 | Arlington |  |  | OR | STANDARD |
| 97813 | Athena |  |  | OR | STANDARD |
| 97814 | Baker City | Medical Springs |  | OR | STANDARD |
| 97817 | Bates |  |  | OR | STANDARD |
| 97818 | Boardman |  |  | OR | STANDARD |
| 97819 | Bridgeport |  |  | OR | STANDARD |
| 97820 | Canyon City |  |  | OR | STANDARD |
| 97823 | Condon | Lonerock |  | OR | STANDARD |
| 97824 | Cove |  |  | OR | STANDARD |
| 97825 | Dayville |  |  | OR | STANDARD |
| 97826 | Echo |  |  | OR | STANDARD |
| 97827 | Elgin |  |  | OR | STANDARD |
| 97828 | Enterprise |  |  | OR | STANDARD |
| 97830 | Fossil | Kinzua | Mayville | OR | STANDARD |
| 97833 | Haines |  |  | OR | STANDARD |
| 97834 | Halfway |  |  | OR | STANDARD |
| 97835 | Helix |  |  | OR | STANDARD |
| 97836 | Heppner |  |  | OR | STANDARD |
| 97837 | Hereford |  |  | OR | STANDARD |
| 97838 | Hermiston |  |  | OR | STANDARD |
| 97839 | Lexington |  |  | OR | STANDARD |
| 97840 | Oxbow |  |  | OR | STANDARD |
| 97841 | Imbler |  |  | OR | STANDARD |
| 97842 | Imnaha |  |  | OR | STANDARD |
| 97843 | Ione |  |  | OR | STANDARD |
| 97844 | Irrigon |  |  | OR | STANDARD |
| 97845 | John Day |  |  | OR | STANDARD |
| 97846 | Joseph |  |  | OR | STANDARD |
| 97848 | Kimberly |  |  | OR | STANDARD |
| 97850 | La Grande | Island City |  | OR | STANDARD |
| 97856 | Long Creek | Fox | Ritter | OR | STANDARD |
| 97857 | Lostine |  |  | OR | STANDARD |
| 97859 | Meacham |  |  | OR | PO BOX ONLY |
| 97861 | Mikkalo | Arlington |  | OR | STANDARD |
| 97862 | Milton Freewater |  |  | OR | STANDARD |
| 97864 | Monument |  |  | OR | STANDARD |
| 97865 | Mount Vernon |  |  | OR | STANDARD |
| 97867 | North Powder |  |  | OR | STANDARD |
| 97868 | Pilot Rock |  |  | OR | STANDARD |
| 97869 | Prairie City |  |  | OR | STANDARD |
| 97870 | Richland |  |  | OR | STANDARD |
| 97873 | Seneca |  |  | OR | STANDARD |
| 97874 | Spray |  |  | OR | STANDARD |
| 97875 | Stanfield |  |  | OR | STANDARD |
| 97876 | Summerville |  |  | OR | STANDARD |
| 97877 | Sumpter | Granite | Greenhorn | OR | STANDARD |
| 97880 | Ukiah | Dale |  | OR | PO BOX ONLY |
| 97882 | Umatilla | McNary |  | OR | STANDARD |
| 97883 | Union |  |  | OR | STANDARD |
| 97884 | Unity |  |  | OR | STANDARD |
| 97885 | Wallowa |  |  | OR | STANDARD |
| 97886 | Weston |  |  | OR | STANDARD |
| 97901 | Adrian |  |  | OR | STANDARD |
| 97902 | Arock |  |  | OR | PO BOX ONLY |
| 97903 | Brogan |  |  | OR | STANDARD |
| 97904 | Drewsey |  |  | OR | STANDARD |
| 97905 | Durkee |  |  | OR | PO BOX ONLY |
| 97906 | Harper |  |  | OR | STANDARD |
| 97907 | Huntington |  |  | OR | STANDARD |
| 97908 | Ironside |  |  | OR | STANDARD |
| 97909 | Jamieson |  |  | OR | STANDARD |
| 97910 | Jordan Valley | South Mountain |  | OR | STANDARD |
| 97911 | Juntura |  |  | OR | STANDARD |
| 97913 | Nyssa |  |  | OR | STANDARD |
| 97914 | Ontario |  |  | OR | STANDARD |
| 97917 | Riverside |  |  | OR | STANDARD |
| 97918 | Vale |  |  | OR | STANDARD |
| 97920 | Westfall |  |  | OR | PO BOX ONLY |

| LOOKUP_POSTAL_COMMUNITY |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| Post_Comm | Text | 100 | US Postal Service preferred and acceptable city names. |

Post_Comm

Adair Village

Adams

Adel

Adrian

Agness

Albany

Allegany

Aloha

Alsea

Alvadore

Amity

Antelope

Applegate

Arch Cape

Arlington

Arock

Ashland

Ashwood

Astoria

Athena

Aumsville

Aurora

Azalea

Baker City

Bandon

Banks

Bates

Bay City

Beatty

Beaver

Beavercreek

Beaverton

Bend

Birkenfeld

Blachly

Blodgett

Blue River

Bly

Boardman

Bonanza

Boring

Bridal Veil

Bridgeport

Brightwood

Broadbent

Brogan

Brookings

Brooks

Brothers

Brownsville

Burns

Butte Falls

Buxton

Camas Valley

Camp Sherman

Canby

Cannon Beach

Canyon City

Canyonville

Carlton

Cascade Locks

Cascadia

Cave Junction

Cayuse

Central Point

Charleston

Chemult

Cheshire

Chiloquin

Christmas Valley

Clackamas

Clatskanie

Cloverdale

Coburg

Colton

Columbia City

Condon

Coos Bay

Coquille

Corbett

Cornelius

Corvallis

Cottage Grove

Cove

Crabtree

Crane

Crater Lake

Crawfordsville

Crescent

Crescent Lake

Creswell

Crooked River Ranch

Culp Creek

Culver

Curtin

Dairy

Dale

Dallas

Damascus

Days Creek

Dayton

Dayville

Deadwood

Deer Island

Depoe Bay

Detroit

Dexter

Diamond

Diamond Lake

Dillard

Donald

Dorena

Drain

Drewsey

Dufur

Dundee

Dunes City

Durkee

Eagle Creek

Eagle Point

Echo

Eddyville

Elgin

Elkton

Elmira

Enterprise

Estacada

Eugene

Fairview

Fall Creek

Falls City

Fields

Florence

Forest Grove

Fort Klamath

Fort Rock

Fossil

Foster

Fox

Frenchglen

Friend

Gales Creek

Gardiner

Garibaldi

Gaston

Gates

Gearhart

Gervais

Gilchrist

Gladstone

Glendale

Gleneden Beach

Glenwood

Glide

Gold Beach

Gold Hill

Government Camp

Grand Ronde

Granite

Grants Pass

Grass Valley

Greenhorn

Greenleaf

Gresham

Haines

Halfway

Halsey

Hammond

Happy Valley

Harbor

Harper

Harrisburg

Hebo

Helix

Heppner

Hereford

Hermiston

Hillsboro

Hines

Hood River

Hubbard

Huntington

Idanha

Idleyld Park

Imbler

Imnaha

Independence

Ione

Ironside

Irrigon

Island City

Jacksonville

Jamieson

Jasper

Jefferson

John Day

Jordan Valley

Joseph

Junction City

Juntura

Keizer

Keno

Kent

Kerby

Kimberly

King City

Kinzua

Klamath Falls

La Grande

La Pine

Lafayette

Lake Grove

Lake Oswego

Lakeside

Lakeview

Langlois

Lawen

Leaburg

Lebanon

Lexington

Lincoln City

Logsden

Lonerock

Long Creek

Lorane

Lostine

Lowell

Lyons

Madras

Malin

Manning

Manzanita

Mapleton

Marcola

Marion

Marylhurst

Maupin

Mayville

McKenzie Bridge

McMinnville

McNary

Meacham

Medford

Medical Springs

Mehama

Merlin

Merrill

Metolius

Midland

Mikkalo

Mill City

Millersburg

Milton Freewater

Milwaukie

Mitchell

Molalla

Monmouth

Monroe

Monument

Moro

Mosier

Mount Angel

Mount Hood Parkdale

Mount Vernon

Mulino

Murphy

Myrtle Creek

Myrtle Point

Nehalem

Neotsu

Neskowin

Netarts

Netarts Bay

New Pine Creek

Newberg

Newport

North Bend

North Plains

North Powder

Norway

Noti

Nyssa

O Brien

Oak Grove

Oakland

Oakridge

Oceanside

Odell

Ontario

Ophir

Oregon City

Oretech

Otis

Otter Rock

Oxbow

Pacific City

Paisley

Paulina

Pendleton

Philomath

Phoenix

Pilot Rock

Pleasant Hill

Plush

Port Orford

Portland

Post

Powell Butte

Powers

Prairie City

Prescott

Princeton

Prineville

Prospect

Rainier

Redmond

Reedsport

Remote

Rhododendron

Richland

Rickreall

Riddle

Riley

Ritter

Riverside

Rockaway Beach

Rogue River

Rose Lodge

Roseburg

Rufus

Saginaw

Saint Benedict

Saint Helens

Saint Paul

Salem

Sandy

Scappoose

Scio

Scotts Mills

Scottsburg

Seal Rock

Seaside

Selma

Seneca

Shady Cove

Shaniko

Shedd

Sheridan

Sherwood

Siletz

Silver Lake

Silverton

Sisters

Sixes

South Beach

South Mountain

Sprague River

Spray

Springfield

Stanfield

Stayton

Sublimity

Summer Lake

Summerville

Sumpter

Sunriver

Sutherlin

Sweet Home

Swisshome

Talent

Tangent

Tenmile

Terrebonne

The Dalles

Tidewater

Tigard

Tillamook

Tiller

Timber

Timberline Lodge

Toledo

Tolovana Park

Trail

Troutdale

Tualatin

Turner

Tygh Valley

Ukiah

Umatilla

Umpqua

Union

Unity

Vale

Veneta

Vernonia

Vida

Waldport

Wallowa

Walterville

Walton

Wamic

Warm Springs

Warren

Warrenton

Wasco

Wedderburn

Welches

West Linn

West Stayton

Westfall

Westfir

Westlake

Weston

Westport

Wheeler

White City

Wilbur

Wilderville

Willamina

Williams

Wilsonville

Winchester

Winchester Bay

Winston

Wolf Creek

Wood Village

Woodburn

Yachats

Yamhill

Yoncalla

Zigzag

| LOOKUP_RESIDENTIAL |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| RESIDENTIAL | Text | 1 | US Postal Service Residential Delivery Indicator code. |
| DESCRIPTION | Text | 255 | Brief description for each code. |

| RESIDENTIAL | DESCRIPTION |
| --- | --- |
| G | Non-residential Census group quarters |
| N | Non-residential |
| R | Residential |

| LOOKUP_STAGE |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| STAGE | Text | 1 | Address lifecycle stage code. |
| DESCRIPTION | Text | 255 | Brief description for each code. |

| STAGE | DESCRIPTION |
| --- | --- |
| C | Current Active |
| F | Future Planned |
| P | Past Historic |
| U | Unknown |

| LOOKUP_STREET_DIRECTION |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| NENA | Text | 20 | NENA fully spelled pre or post street direction. |
| USPS | Text | 2 | US Postal Service direction abbreviation. |

| NENA | USPS |
| --- | --- |
| East | E |
| North | N |
| Northeast | NE |
| Northwest | NW |
| South | S |
| Southeast | SE |
| Southwest | SW |
| West | W |

| LOOKUP_STREET_PRE_SEPARATOR |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| NENA | Text | 20 | NENA fully spelled pre separator. |

NENA

at

de

de la

de las

del

des

in the

of

of the

on the

to

to the

| LOOKUP_STREET_TYPE |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| NENA | Text | 20 | NENA fully spelled pre or post street type. |
| USPS | Text | 4 | US Postal Service street post type abbreviation. |
| WVCC | Text | 2 | Willamette Valley Comm. Center street type abbreviation. |

| NENA | USPS | WVCC |
| --- | --- | --- |
| Alley | ALY | AL |
| Annex | ANX |  |
| Arcade | ARC |  |
| Avenida |  |  |
| Avenue | AVE | AV |
| Bayou | BYU |  |
| Beach | BCH |  |
| Bend | BND |  |
| Bluff | BLF |  |
| Bluffs | BLFS |  |
| Bottom | BTM |  |
| Boulevard | BLVD | BV |
| Branch | BR |  |
| Bridge | BRG |  |
| Brook | BRK |  |
| Brooks | BRKS |  |
| Burg | BG |  |
| Burgs | BGS |  |
| Bypass | BYP |  |
| Calle |  |  |
| Camino |  |  |
| Camp | CP |  |
| Canyon | CYN |  |
| Cape | CPE |  |
| Causeway | CSWY |  |
| Center | CTR |  |
| Centers | CTRS |  |
| Circle | CIR | CR |
| Circles | CIRS |  |
| Circus | CRCS |  |
| Cliff | CLF |  |
| Cliffs | CLFS |  |
| Club | CLB |  |
| Common | CMN |  |
| Commons | CMNS |  |
| Corner | COR |  |
| Corners | CORS |  |
| Course | CRSE |  |
| Court | CT | CT |
| Courts | CTS |  |
| Cove | CV |  |
| Coves | CVS |  |
| Creek | CRK |  |
| Crescent | CRES | CS |
| Crest | CRST |  |
| Crossing | XING |  |
| Crossroad | XRD |  |
| Crossroads | XRDS |  |
| Curve | CURV |  |
| Dale | DL |  |
| Dam | DM |  |
| Divide | DV |  |
| Drive | DR | DR |
| Drives | DRS |  |
| Estate | EST |  |
| Estates | ESTS |  |
| Expressway | EXPY |  |
| Extension | EXT |  |
| Extensions | EXTS |  |
| Fall | FALL |  |
| Falls | FLS |  |
| Ferry | FRY |  |
| Field | FLD |  |
| Fields | FLDS |  |
| Flat | FLT |  |
| Flats | FLTS |  |
| Ford | FRD |  |
| Fords | FRDS |  |
| Forest | FRST |  |
| Forge | FRG |  |
| Forges | FRGS |  |
| Fork | FRK |  |
| Forks | FRKS |  |
| Fort | FT |  |
| Freeway | FWY |  |
| Garden | GDN |  |
| Gardens | GDNS |  |
| Gateway | GTWY |  |
| Glen | GLN |  |
| Glens | GLNS |  |
| Green | GRN |  |
| Greens | GRNS |  |
| Grove | GRV |  |
| Groves | GRVS |  |
| Harbor | HBR |  |
| Harbors | HBRS |  |
| Haven | HVN |  |
| Heights | HTS | HT |
| Highway | HWY | HW |
| Hill | HL |  |
| Hills | HLS |  |
| Hollow | HOLW |  |
| Inlet | INLT |  |
| Island | IS |  |
| Islands | ISS |  |
| Isle | ISLE |  |
| Junction | JCT |  |
| Junctions | JCTS |  |
| Key | KY |  |
| Keys | KYS |  |
| Knoll | KNL |  |
| Knolls | KNLS |  |
| Lake | LK |  |
| Lakes | LKS |  |
| Land | LAND |  |
| Landing | LNDG |  |
| Lane | LN | LN |
| Light | LGT |  |
| Lights | LGTS |  |
| Loaf | LF |  |
| Lock | LCK |  |
| Locks | LCKS |  |
| Lodge | LDG |  |
| Loop | LOOP | LP |
| Mall | MALL |  |
| Manor | MNR |  |
| Manors | MNRS |  |
| Meadow | MDW |  |
| Meadows | MDWS |  |
| Mews | MEWS |  |
| Mill | ML |  |
| Mills | MLS |  |
| Mission | MSN |  |
| Motorway | MTWY |  |
| Mount | MT |  |
| Mountain | MTN |  |
| Mountains | MTNS |  |
| Neck | NCK |  |
| Orchard | ORCH |  |
| Oval | OVAL |  |
| Overpass | OPAS |  |
| Park | PARK |  |
| Parks | PARK |  |
| Parkway | PKWY |  |
| Parkways | PKWY |  |
| Pass | PASS |  |
| Passage | PSGE |  |
| Path | PATH |  |
| Pike | PIKE |  |
| Pine | PNE |  |
| Pines | PNES |  |
| Place | PL | PL |
| Plain | PLN |  |
| Plains | PLNS |  |
| Plaza | PLZ |  |
| Point | PT |  |
| Points | PTS |  |
| Port | PRT |  |
| Ports | PRTS |  |
| Prairie | PR |  |
| Radial | RADL |  |
| Ramp | RAMP |  |
| Ranch | RNCH |  |
| Rapid | RPD |  |
| Rapids | RPDS |  |
| Rest | RST |  |
| Ridge | RDG |  |
| Ridges | RDGS |  |
| River | RIV |  |
| Road | RD | RD |
| Roads | RDS |  |
| Route | RTE |  |
| Row | ROW |  |
| Rue | RUE |  |
| Run | RUN |  |
| Shoal | SHL |  |
| Shoals | SHLS |  |
| Shore | SHR |  |
| Shores | SHRS |  |
| Skyway | SKWY |  |
| Spring | SPG |  |
| Springs | SPGS |  |
| Spur | SPUR |  |
| Spurs | SPUR |  |
| Square | SQ | SQ |
| Squares | SQS |  |
| Station | STA |  |
| Stravenue | STRA |  |
| Stream | STRM |  |
| Street | ST | ST |
| Streets | STS |  |
| Summit | SMT |  |
| Terrace | TER |  |
| Throughway | TRWY |  |
| Trace | TRCE |  |
| Track | TRAK |  |
| Trafficway | TRFY |  |
| Trail | TRL | TR |
| Trailer | TRLR |  |
| Tunnel | TUNL |  |
| Turnpike | TPKE |  |
| Underpass | UPAS |  |
| Union | UN |  |
| Unions | UNS |  |
| Valley | VLY |  |
| Valleys | VLYS |  |
| Via |  |  |
| Viaduct | VIA |  |
| View | VW |  |
| Views | VWS |  |
| Village | VLG |  |
| Villages | VLGS |  |
| Ville | VL |  |
| Vista | VIS |  |
| Walk | WALK |  |
| Walks | WALK |  |
| Wall | WALL |  |
| Way | WAY | WY |
| Ways | WAYS |  |
| Well | WL |  |
| Wells | WLS |  |

| LOOKUP_SUBADDRESS_TYPE |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| NENA | Text | 20 | NENA fully spelled subaddress type. |
| USPS | Text | 2 | US Postal Service subaddress abbreviation. |

| NENA | USPS |
| --- | --- |
| # | # |
| Apartment | APT |
| Basement | BSMT |
| Building | BLDG |
| Department | DEPT |
| Floor | FL |
| Front | FRNT |
| Hanger | HNGR |
| Key | KEY |
| Lobby | LBBY |
| Lot | LOT |
| Lower | LOWR |
| Office | OFC |
| Penthouse | PH |
| Pier | PIER |
| Rear | REAR |
| Room | RM |
| Side | SIDE |
| Slip | SLIP |
| Space | SPC |
| Stop | STOP |
| Suite | STE |
| Trailer | TRLR |
| Unit | UNIT |
| Upper | UPPR |

| LOOKUP_UNINCORPORATED_COMMUNITY |  |  |  |
| --- | --- | --- | --- |
| NAME | TYPE | WIDTH | DESCRIPTION |
| STATE_FIPS | Text | 2 | Census state code. |
| PLACE_FIPS | Text | 3 | Census place code. |
| GEOID | Text | 7 | Census unique combined state and place code. |
| Uninc_Comm | Text | 100 | Census Designated Place (CDP) name. |

| STATE_FIPS | PLACE_FIPS | GEOID | Uninc_Comm |
| --- | --- | --- | --- |
| 41 | 01650 | 4101650 | Aloha |
| 41 | 01700 | 4101700 | Alpine |
| 41 | 01800 | 4101800 | Alsea |
| 41 | 01850 | 4101850 | Altamont |
| 41 | 02200 | 4102200 | Annex |
| 41 | 04125 | 4104125 | Barnesdale |
| 41 | 04400 | 4104400 | Barview |
| 41 | 04850 | 4104850 | Bayshore |
| 41 | 04925 | 4104925 | Bayside Gardens |
| 41 | 05150 | 4105150 | Beatty |
| 41 | 05200 | 4105200 | Beaver |
| 41 | 05250 | 4105250 | Beavercreek |
| 41 | 05300 | 4105300 | Beaver Marsh |
| 41 | 05700 | 4105700 | Bellfountain |
| 41 | 05950 | 4105950 | Bethany |
| 41 | 06400 | 4106400 | Biggs Junction |
| 41 | 06650 | 4106650 | Black Butte Ranch |
| 41 | 06950 | 4106950 | Blodgett |
| 41 | 07150 | 4107150 | Bly |
| 41 | 07550 | 4107550 | Boring |
| 41 | 08600 | 4108600 | Brogan |
| 41 | 08750 | 4108750 | Brooks |
| 41 | 09535 | 4109535 | Bull Mountain |
| 41 | 09600 | 4109600 | Bunker Hill |
| 41 | 10100 | 4110100 | Butteville |
| 41 | 10550 | 4110550 | Camp Sherman |
| 41 | 11050 | 4111050 | Cape Meares |
| 41 | 11700 | 4111700 | Cascadia |
| 41 | 11900 | 4111900 | Cayuse |
| 41 | 12050 | 4112050 | Cedar Hills |
| 41 | 12150 | 4112150 | Cedar Mill |
| 41 | 12750 | 4112750 | Chemult |
| 41 | 12800 | 4112800 | Chenoweth |
| 41 | 12850 | 4112850 | Cherry Grove |
| 41 | 13000 | 4113000 | Cheshire |
| 41 | 14200 | 4114200 | Cloverdale |
| 41 | 16350 | 4116350 | Crabtree |
| 41 | 16450 | 4116450 | Crane |
| 41 | 16650 | 4116650 | Crawfordsville |
| 41 | 16750 | 4116750 | Crescent |
| 41 | 16800 | 4116800 | Crescent Lake |
| 41 | 16975 | 4116975 | Crooked River Ranch |
| 41 | 17800 | 4117800 | Damascus |
| 41 | 18150 | 4118150 | Days Creek |
| 41 | 18450 | 4118450 | Deer Island |
| 41 | 19020 | 4119020 | Deschutes River Woods |
| 41 | 19250 | 4119250 | Dexter |
| 41 | 19600 | 4119600 | Dillard |
| 41 | 19650 | 4119650 | Dilley |
| 41 | 21215 | 4121215 | Dunthorpe |
| 41 | 21450 | 4121450 | Eagle Crest |
| 41 | 23000 | 4123000 | Elmira |
| 41 | 23550 | 4123550 | Eola |
| 41 | 24100 | 4124100 | Fair Oaks |
| 41 | 24300 | 4124300 | Fairview |
| 41 | 24350 | 4124350 | Falcon Heights |
| 41 | 26165 | 4126165 | Foots Creek |
| 41 | 26400 | 4126400 | Fort Hill |
| 41 | 26450 | 4126450 | Fort Klamath |
| 41 | 26750 | 4126750 | Four Corners |
| 41 | 27825 | 4127825 | Garden Home-Whitford |
| 41 | 27850 | 4127850 | Gardiner |
| 41 | 28800 | 4128800 | Gilchrist |
| 41 | 29100 | 4129100 | Glasgow |
| 41 | 29750 | 4129750 | Glide |
| 41 | 30165 | 4130165 | Gopher Flats |
| 41 | 30250 | 4130250 | Government Camp |
| 41 | 30300 | 4130300 | Grand Ronde |
| 41 | 30750 | 4130750 | Green |
| 41 | 31115 | 4131115 | Green Meadows |
| 41 | 32100 | 4132100 | Harbor |
| 41 | 32400 | 4132400 | Harper |
| 41 | 32850 | 4132850 | Hayesville |
| 41 | 33100 | 4133100 | Hebo |
| 41 | 33150 | 4133150 | Heceta Beach |
| 41 | 34650 | 4134650 | Holley |
| 41 | 35850 | 4135850 | Idaville |
| 41 | 37150 | 4137150 | Jasper |
| 41 | 37202 | 4137202 | Jeffers Gardens |
| 41 | 37400 | 4137400 | Jennings Lodge |
| 41 | 38075 | 4138075 | Juniper Canyon |
| 41 | 38150 | 4138150 | Juntura |
| 41 | 38750 | 4138750 | Keno |
| 41 | 38900 | 4138900 | Kerby |
| 41 | 39350 | 4139350 | Kings Valley |
| 41 | 39566 | 4139566 | Kirkpatrick |
| 41 | 39850 | 4139850 | Knappa |
| 41 | 40050 | 4140050 | Labish Village |
| 41 | 40150 | 4140150 | Lacomb |
| 41 | 40950 | 4140950 | Langlois |
| 41 | 42550 | 4142550 | Lincoln Beach |
| 41 | 43650 | 4143650 | Lookingglass |
| 41 | 44625 | 4144625 | McKay |
| 41 | 45750 | 4145750 | Mapleton |
| 41 | 45850 | 4145850 | Marcola |
| 41 | 45900 | 4145900 | Marion |
| 41 | 46100 | 4146100 | Marlene Village |
| 41 | 46800 | 4146800 | Meacham |
| 41 | 47300 | 4147300 | Mehama |
| 41 | 47350 | 4147350 | Melrose |
| 41 | 47650 | 4147650 | Merlin |
| 41 | 47800 | 4147800 | Metzger |
| 41 | 49000 | 4149000 | Mission |
| 41 | 50200 | 4150200 | Mount Hood |
| 41 | 50236 | 4150236 | Mount Hood Villages |
| 41 | 50450 | 4150450 | Mulino |
| 41 | 51450 | 4151450 | Neahkahnie |
| 41 | 51900 | 4151900 | Neotsu |
| 41 | 51950 | 4151950 | Nesika Beach |
| 41 | 52000 | 4152000 | Neskowin |
| 41 | 52050 | 4152050 | Netarts |
| 41 | 52275 | 4152275 | New Hope |
| 41 | 52400 | 4152400 | New Pine Creek |
| 41 | 53900 | 4153900 | Oak Grove |
| 41 | 53988 | 4153988 | Oak Hills |
| 41 | 54325 | 4154325 | Oatfield |
| 41 | 54350 | 4154350 | O Brien |
| 41 | 54450 | 4154450 | Oceanside |
| 41 | 54527 | 4154527 | Ochoco West |
| 41 | 54550 | 4154550 | Odell |
| 41 | 55268 | 4155268 | Oregon Shores |
| 41 | 55500 | 4155500 | Orient |
| 41 | 56150 | 4156150 | Pacific City |
| 41 | 56500 | 4156500 | Parkdale |
| 41 | 57300 | 4157300 | Peoria |
| 41 | 57850 | 4157850 | Pine Grove |
| 41 | 57875 | 4157875 | Pine Hollow |
| 41 | 58050 | 4158050 | Pistol River |
| 41 | 58500 | 4158500 | Pleasant Valley |
| 41 | 58550 | 4158550 | Plush |
| 41 | 59916 | 4159916 | Prineville Lake Acres |
| 41 | 60070 | 4160070 | Pronghorn |
| 41 | 60100 | 4160100 | Prospect |
| 41 | 60900 | 4160900 | Raleigh Hills |
| 41 | 61250 | 4161250 | Redwood |
| 41 | 61550 | 4161550 | Rhododendron |
| 41 | 61800 | 4161800 | Rickreall |
| 41 | 62292 | 4162292 | River Point |
| 41 | 62300 | 4162300 | River Road |
| 41 | 62412 | 4162412 | Riverside |
| 41 | 63010 | 4163010 | Rockcreek |
| 41 | 63250 | 4163250 | Rocky Point |
| 41 | 63660 | 4163660 | Roseburg North |
| 41 | 63800 | 4163800 | Rose Lodge |
| 41 | 63950 | 4163950 | Rowena |
| 41 | 64150 | 4164150 | Ruch |
| 41 | 64282 | 4164282 | Running Y Ranch |
| 41 | 65300 | 4165300 | San Marine |
| 41 | 65400 | 4165400 | Santa Clara |
| 41 | 65450 | 4165450 | Saunders Lake |
| 41 | 66150 | 4166150 | Selma |
| 41 | 66385 | 4166385 | Seventh Mountain |
| 41 | 66900 | 4166900 | Shedd |
| 41 | 67600 | 4167600 | Silver Lake |
| 41 | 68875 | 4168875 | South Lebanon |
| 41 | 69387 | 4169387 | Sportsmans Park |
| 41 | 69400 | 4169400 | Sprague River |
| 41 | 69800 | 4169800 | Stafford |
| 41 | 70870 | 4170870 | Summit |
| 41 | 71250 | 4171250 | Sunriver |
| 41 | 71800 | 4171800 | Svensen |
| 41 | 72400 | 4172400 | Takilma |
| 41 | 72800 | 4172800 | Terrebonne |
| 41 | 72820 | 4172820 | Tetherow |
| 41 | 73225 | 4173225 | Three Rivers |
| 41 | 74400 | 4174400 | Trail |
| 41 | 74550 | 4174550 | Trent |
| 41 | 74650 | 4174650 | Tri-City |
| 41 | 75050 | 4175050 | Tumalo |
| 41 | 75185 | 4175185 | Tutuilla |
| 41 | 75500 | 4175500 | Tygh Valley |
| 41 | 75600 | 4175600 | Umapine |
| 41 | 78160 | 4178160 | Wallowa Lake |
| 41 | 78300 | 4178300 | Wamic |
| 41 | 78600 | 4178600 | Warm Springs |
| 41 | 78800 | 4178800 | Warren |
| 41 | 79500 | 4179500 | Wedderburn |
| 41 | 80025 | 4180025 | West Haven-Sylvan |
| 41 | 80400 | 4180400 | Westport |
| 41 | 80750 | 4180750 | West Scio |
| 41 | 80900 | 4180900 | West Slope |
| 41 | 81450 | 4181450 | White City |
| 41 | 82450 | 4182450 | Williams |
| 41 | 82850 | 4182850 | Wimer |
| 41 | 83050 | 4183050 | Winchester Bay |