# Oregon

Current status of elevation data for Oregon

The Oregon Department of Geology and Mineral Industries (DOGAMI) has been supervising and coordinating the collection of high-resolution, high-accuracy lidar data in the Pacific Northwest since 2006. In 2007, DOGAMI created the Oregon Lidar Consortium (OLC) for the purpose of acquiring lidar over the entire state of Oregon. To date, the OLC has ordered 72 lidar flights, supported by funding from more than 80 different federal, tribal, state, city, and private partners. Taken together with data collected by other agencies and stakeholders, 49% of Oregon (48,220 square miles) has lidar coverage, including 98% of Oregon’s populated areas. Bathymetric lidar coverage in Oregon is limited in scope and lags behind the pace of terrestrial lidar collection. Riverine and lake bathymetry data only exist for small tributaries or reservoirs while nearshore bathymetry is collected exclusively by the National Coastal Mapping Program on a five-year cycle.

The Oregon Geospatial Enterprise Office (GEO) assists in coordinating data standardization and stewardship for lidar collected in Oregon. Since 2007, Oregon lidar data standards and procedures have been amended to reflect changes in lidar sensor technology and to maintain the most stringent specifications for lidar accuracy. Oregon State Price Agreement 7525 Amendment 8 (https://www.oregongeology.org/lidar/DAS-Quantum_7525Amendment8.pdf) provides specifications for Quality Level 1 (QL1) terrestrial and bathymetric 3DEP lidar collection in Oregon. The Oregon Elevation Framework Implementation Team (Elevation-FIT), an elevation workgroup within GEO, developed a Lidar Prioritization Plan (https://www.oregon.gov/geo/FIT%20Documents/2014%20Oregon%20Lidar%20Prioritization%20Plan.pdf) that prioritizes watersheds based on need or use. The Lidar Prioritization Plan is used as a guideline for developing and funding future lidar collections in Oregon.

Importance of elevation data to Oregon

Oregon’s dynamic topography comprises various geologic terranes and diverse ecosystems. Uses for lidar and bathymetry elevation data vary across Oregon, but lidar data products are needed for a broad range of business and scientific and academic applications. Elevation data products are used by governments at all levels for the assessment, monitoring, and management of infrastructure, agriculture, and emergency management. High-resolution lidar data products are vital for natural resource management within forestry, geology, and watersheds. Underpinning the importance of lidar for managing natural and man-made resources is the need to mitigate risk of natural hazards.

The dynamic nature of the state’s geology and Oregon’s population growth makes natural hazard mitigation a top priority. High-density, highly accurate elevation data are critical tools for understanding natural hazards and developing resiliency in Oregon:

- Tsunami - The Cascadia Subduction Zone, a 960-kilometer-long (600 mile) earthquake fault zone sits off the Pacific Northwest coast. High-density, high-resolution terrestrial lidar and nearshore bathymetry lidar data have been vital for tsunami inundation modeling and developing evacuation routes for coastal Oregon communities that can be affected by tsunamis. https://www.oregongeology.org/tsuclearinghouse/beatthewave.htm

- Flooding - Flooding is a common and costly natural hazard along rivers and the coast in Oregon. The National Flood Insurance Program (NFIP) identifies 251 communities in Oregon as flood prone, including locations in all 36 counties, 212 cities, and 3 tribal nations. Elevation data are being used to improve our understanding of flood inundation locations. https://www.oregongeology.org/flood/default.htm

- Landslide - Landslides are a common, chronic problem in Oregon, especially in places with or near moderate to steep slopes, during times of heavy precipitation, and during earthquakes. Landslides can be catastrophic. High-density lidar is changing our fundamental understanding of landslide hazards through the discovery of landslide deposits and the tracking of movement. Understanding past landslide hazards is key to predicting future landslides. https://www.oregongeology.org/slido/index.htm

- Earthquake - Earthquake hazards have been recognized as one of the major natural hazards in Oregon. The generation of bare ground lidar products assists with the discovery and analysis of faults in Oregon. First return lidar data also help identify assets that are at risk, such as the exact locations of buildings and other structures. https://www.oregongeology.org/earthquakes/earthquakehome.htm

- Forest Fire - With over 28 million acres of forested land in Oregon, wildfire management is a major resource investment. Lidar data are helpful for wildfire prevention as well as wildfire containment. As with other hazards, lidar helps identify infrastructure and assets at risk. https://www.oregon.gov/ODF/Fire/Pages/default.aspx

High-level summary of elevation data requirements

As part of the Oregon GEO mission to coordinate data standardization in Oregon, the Oregon Lidar Consortium has worked with Elevation-FIT) (https://www.oregon.gov/geo/Pages/elevframe.aspx) to develop elevation data standards. E-FIT is composed of representatives from federal, state, local, tribal, and private organizations. This group of stakeholders for Oregon elevation data has determined that QL1 inland topography and QL1B inland bathymetry is the preferred data standard across the entire state of Oregon.

QL1B nearshore bathymetry data are required along coastal Oregon due to the development of coastal communities and their vulnerability. The combination of ocean warming, sea level rise, and natural disasters requires updated, highly accurate data for modeling. Order 1a offshore bathymetry is required to support shipping channels and monitor the health of the ocean, which serves the commercial fishing and recreational economies of coastal Oregon.

High-level summary of benefits that would come from higher resolution elevation data

One part of Oregon’s strategic mission for lidar data collection is developing plans for recollection of older lidar datasets. Most of western Oregon including coastal communities, Oregon coastal mountain ranges, and the Willamette Valley have lidar coverage that is eight to ten years old. Recollecting these areas with improved lidar technology will result in better data for environmental analysis. Recollecting western Oregon with higher resolution lidar data is a major goal for the Oregon Lidar Consortium over the next five to ten years. Studies in Oregon using lidar differencing show great promise for understanding small to large changes to the earth’s surface including landslides and sediment erosion evaluations. These two processes affect human and environmental risk. In order to evaluate these processes accurately, higher resolution elevation data are fundamental.

Additional information:

The West Coast was the focus for a seafloor data prioritization effort in 2019:

https://coastalscience.noaa.gov/project/prioritizing-areas-for-future-seafloor-mapping-research-and-exploration-offshore-of-california-oregon-and-washington/

Results of that work are posted online at:

https://gis.ngdc.noaa.gov/arcgis/rest/services/nccos/BiogeographicAssessments_WestCoastPrioritizationResults/MapServer

https://www.arcgis.com/home/webmap/viewer.html?url=https%3A%2F%2Fgis.ngdc.noaa.gov%2Farcgis%2Frest%2Fservices%2Fnccos%2FBiogeographicAssessments_WestCoastPrioritizationResults%2FMapServer&source=sd

The data show a high priority need for offshore bathymetry/backscatter data in an area about 50-60 kilometers west of Lincoln City and Depoe Bay. Also an area 20-50 kilometers west of Newport is designated as high priority as well. Another area of high priority need is about 80-90 kilometers offshore from the shoreline between Yachats and Newport. Finally, the region about 25-40 kilometers offshore from the mouth of the Columbia River is deemed high priority.

Preparations are underway by JALBTCX to collect high-resolution topobathymetric data for Oregon’s coastline along the Pacific Ocean (Columbia River to the California border) in 2020. This effort is a part of the five-year cycle associated with the National Coastal Mapping Program. The data extent will be from 500 meters onshore to 1000 meters offshore.

Some areas of topobathymetric data priorities appear to be around Coos Bay and Yaquina Bay based on interactions with staff at the State of Oregon and interactions with stakeholders (e.g., the South Slough National Estuarine Research Reserve).

From State website:

[The Oregon State University Active Tectonics and Seafloor Mapping Lab maintains an Oregon State Waters Mapping Program, which is an outgrowth of a Seafloor Mapping Task Force formed at Oregon State University in 2006. This group met on numerous occasions through 2007 and 2008 to discuss the needs for nearshore mapping with state legislators, biologists, geologists and members of coastal communities to develop the proposal. Presentations were made to the Joint Committee on Emergency Preparedness and Ocean Policy on four occasions, and to the State Land Board. Further discussions were held with the Department of Land Conservation and Development, DOGAMI, Oregon Department of Fish and Wildlife, and others about the needs and application to the missions of these agencies. Discussion and questions about tsunami modeling, wave energy siting, and marine reserves and the needs for basic seafloor data were addressed at these meetings.

In 2008, detailed bottom mapping existed for only about 5% of the area of the Oregon Territorial Sea, which extends three nautical miles from the coast and comprises approximately 950 square nautical miles. Effective decisions concerning the management and conservation of ocean resources in accordance with Department of State Lands Strategic and Asset Management Plans depend upon better knowledge of nearshore waters. To address this problem the Oregon Department of State Lands initiated a seafloor mapping program within Oregon’s Territorial Sea.

The State Waters Mapping Program Downloads Page hosts numerous data products related to the seafloor off the coast of Oregon, including bathymetric maps, backscatter/sidescan sonar images, and seafloor habitat maps.

That site is located here:

http://activetectonics.coas.oregonstate.edu/state_waters_download.htm

The State of Oregon has identified Business Uses and Mission Critical Activities that rely on elevation data and would benefit from enhanced elevation data. Summarized details of elevation data requirements and benefits received from the enhanced elevation data are provided in the following pages.

# Summary Table

| MCA Description |  |  |  | Requirements |  |  | Future Operational Benefits | Future Customer Service Benefits | Future Societal Benefits |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Primary Business Use | Agency/ Organization Name | MCA No. | Mission Critical Activity | Data Type | QL/Order | Update Frequency | Total Estimated Annual Operational Benefits | Total Estimated Annual Customer Service Benefits | Education or Outreach | Environ- mental | Public Safety |
| BU 01 – Water Supply and Quality | Portland State University, Oregon | 21630 | Oregon Lakes and Reservoirs Research and Data Provisioning | Inland Bathy | Coarser bathymetric data satisfies my needs | Event driven | $2,000 | $7,000 | Moderate | I don't know | I don't know |
| BU 01 – Water Supply and Quality | State of Oregon | 60349 | Water Supply and Quality | Inland Bathy | QL1B | 6-10 years | $2,327,748 | $243,259 | Moderate | Major | Moderate |
|  |  |  |  | Nearshore Bathy | QL1B | 4-5 years | $8,886 | $14,639 | Moderate | Moderate | Moderate |
| BU 02 – Riverine Ecosystem Management | State of Oregon | 60350 | Riverine Ecosystem Management | Inland Topo | QL1 HD | 6-10 years | $148,538 | $14,836 | Major | Major | Major |
|  |  |  |  | Inland Bathy | QL0B | 4-5 years | $79,072 | $5,575 | Major | Major | Major |
|  |  |  |  | Nearshore Bathy | QL0B | 2-3 years | $26,950 | $20,594 | Moderate | Moderate | Moderate |
| BU 03 – Coastal Zone Management | Oregon Coastal Management Program | 21606 | Coastal Community Resiliency | Inland Topo | QL1 | 4-5 years | $937,584 | $163,909 | Moderate | Major | Major |
|  |  |  |  | Nearshore Bathy | QL1B | 2-3 years | $30,174 | $51,044 | Major | Major | Major |
| BU 03 – Coastal Zone Management | Oregon Parks and Recreation Department | 21670 | Ocean Shore Management for Protection Structure Permitting and Recreation Access | Inland Topo | QL1 | 4-5 years | $23,762 | $4,139 | I don't know | Minor | I don't know |
|  |  |  |  | Nearshore Bathy | QL4B | 4-5 years | $30,472 | $51,081 | I don't know | Minor | I don't know |
| BU 04 – Forest Resource Management | Oregon Department of Forestry | 1360 | Statewide Wildfire Management and Forest Management | Inland Topo | QL0 HD | 6-10 years | $506,878 | $69,103 | Minor | Moderate | Moderate |
| BU 06 – Natural Resource Management | State of Oregon | 60351 | Natural Resources Conservation | Inland Topo | QL1 HD | 4-5 years | $3,806,356 | $13,978,100 | Major | Major | Major |
|  |  |  |  | Inland Bathy | QL0B | 6-10 years | $515,908 | $23,225 | Major | Major | Major |
| BU 07 – Wildlife and Habitat Management | State of Oregon | 60352 | Wildlife and Habitat Management | Inland Topo | QL2 | 4-5 years | $23,116 | $66,767 | Moderate | Major | Moderate |
|  |  |  |  | Inland Bathy | QL2B | 4-5 years | $185,259 | $111,979 | Moderate | Moderate | Moderate |
|  |  |  |  | Nearshore Bathy | QL0B | 4-5 years | $266 | $15 | Moderate | Major | Moderate |
| BU 08 – Agriculture | State of Oregon | 60353 | Agriculture and Precision Farming | Inland Topo | QL2 | 2-3 years | $112,356 | $89,072 | Major | Major | Minor |
|  |  |  |  | Inland Bathy | QL4B | 4-5 years | $196,849 | $89,072 | Moderate | Major | Major |
| BU 09 – Fisheries Management and Aquaculture | State of Oregon | 60354 | Fisheries Management and Aquaculture | Inland Bathy | QL0B | 6-10 years | $194,176 | $88,377 | Major | Major | Moderate |
|  |  |  |  | Nearshore Bathy | QL0B | 6-10 years | $42,329 | $3,108 | Moderate | Major | Minor |
|  |  |  |  | Offshore Bathy | Special Order | 6-10 years | Unable to quantify | Unable to quantify | Moderate | Moderate | Minor |
| BU 10 – Geologic Assessment | Oregon Dept. Geology and Mineral Industries | 1184 | Geohazard Mitigation and Geologic Resource Identification and Mapping | Inland Topo | (a) QL0 (b) QL2 | (a) 4-5 years (b) 6-10 years | $10,494,296 | $218,572 | Moderate | Major | Major |
|  |  |  |  | Inland Bathy | QL1B | 6-10 years | $1,569,296 | $1,108,442 | Minor | Moderate | Major |
|  |  |  |  | Nearshore Bathy | QL2B | 6-10 years | $112,663 | $100,000 | Major | Moderate | Major |
| BU 10 – Geologic Assessment | Governor's Office, State of Oregon | 1246 | State Infrastructure Resiliency | Inland Topo | QL2 | 4-5 years | $10,242,526 | $179,807 | Major | Major | Major |
|  |  |  |  | Nearshore Bathy | QL4B | 4-5 years | $10,390 | $16,718 | Major | Major | Major |
|  |  |  |  | Offshore Bathy | Order 1 | 4-5 years | Unable to quantify | Unable to quantify | Major | Major | Major |
| BU 11 – Geologic Resource Extraction | State of Oregon | 60355 | Geologic Resource Mining and Extraction | Inland Topo | QL1 HD | 2-3 years | $23,705 | $15,724 | Minor | Major | Major |
| BU 12 – Renewable Energy Resources | State of Oregon | 60356 | Renewable Energy Resources | Inland Topo | QL2 | 6-10 years | Unable to quantify | Unable to quantify | Minor | Minor | None |
|  |  |  |  | Inland Bathy | I don't know | I don't know | Unable to quantify | Unable to quantify | I don't know | I don't know | I don't know |
|  |  |  |  | Nearshore Bathy | I don't know | I don't know | Unable to quantify | Unable to quantify | I don't know | I don't know | I don't know |
|  |  |  |  | Offshore Bathy | I don't know | I don't know | Unable to quantify | Unable to quantify | I don't know | I don't know | I don't know |
| BU 14 – Cultural Resource Management | Oregon Parks and Recreation Department | 21672 | Statewide Cultural Resource Protection and Preservation | Inland Topo | QL1 | 4-5 years | $17,787 | $15,551 | Minor | None | None |
|  |  |  |  | Inland Bathy | QL0B | 6-10 years | $42,655 | Unable to quantify | Moderate | Minor | Minor |
|  |  |  |  | Nearshore Bathy | QL3B | 6-10 years | $771 | Unable to quantify | I don't know | I don't know | I don't know |
|  |  |  |  | Offshore Bathy | Order 1a | 6-10 years | $2,529 | Unable to quantify | Minor | Moderate | Minor |
| BU 15 – Flood Risk Management | Oregon Water Resources Dept. | 1306 | Dam Safety Analysis, Flood Risk Modeling, and Mapping | Inland Topo | QL0 | As long as data is an accurate representation of the site, it is acceptable. | Unable to quantify | Unable to quantify | None | None | Major |
|  |  |  |  | Inland Bathy | QL0B | Only if the bathymetry has changed. | $7,236 | Unable to quantify | None | None | Major |
| BU 16 – Sea Level Rise and Subsidence | State of Oregon | 60357 | Sea Level Rise and Subsidence | Inland Topo | QL1 HD | 4-5 years | $5,109,129 | Unable to quantify | Moderate | Moderate | Moderate |
|  |  |  |  | Inland Bathy | QL1B | 4-5 years | Unable to quantify | Unable to quantify | Major | Major | Major |
|  |  |  |  | Nearshore Bathy | QL2B | 2-3 years | Unable to quantify | Unable to quantify | Major | Major | Minor |
| BU 17 – Wildfire Management | State of Oregon | 60358 | Wildfire Management, Planning, and Response | Inland Topo | QL1 | 4-5 years | $521,404 | Unable to quantify | Major | Major | Major |
| BU 18 – Homeland Security | State of Oregon | 60359 | Homeland Security, Law Enforcement, Disaster Response, and Emergency Management | Inland Topo | QL2 | 4-5 years | $2,862,820 | $50,599 | Major | Moderate | Major |
|  |  |  |  | Inland Bathy | QL0B | 4-5 years | $806,243 | $51,439 | Major | Moderate | Major |
|  |  |  |  | Nearshore Bathy | QL1B | 6-10 years | $2,482 | $71 | Moderate | Moderate | Major |
| BU 19 – Land Navigation | Oregon Department of Transportation | 21870 | Transportation Engineering | Inland Topo | QL1 | 2-3 years | $1,257,147 | $101,520 | Minor | Major | Major |
|  |  |  |  | Inland Bathy | QL0B | 4-5 years | $53,622 | $554 | I don't know | Major | Major |
|  |  |  |  | Nearshore Bathy | QL0B | 4-5 years | Unable to quantify | Unable to quantify | Minor | Minor | Minor |
| BU 22 – Infrastructure Management | Oregon Parks and Recreation Department | 21664 | Engineering and Facilities Management | Inland Topo | QL1 | 4-5 years | $61,708 | Unable to quantify | None | None | None |
| BU 23 – Urban and Regional Planning | Oregon Councils of Government | 21615 | Regional GIS and Data Services | Inland Topo | (a) QL1 HD (b) QL2 | (a) 2 years (b) 2-4 years | $14,304,192 | $1,651,425 | Minor | Moderate | I don't know |
|  |  |  |  | Inland Bathy | QL1B | 2-3 years | $582,584 | $737,019 | Minor | Moderate | Moderate |
|  |  |  |  | Nearshore Bathy | QL1B | 2-3 years | $25,870 | $27,291 | I don't know | I don't know | I don't know |

# MCA Title: Oregon Lakes and Reservoirs Research and Data Provisioning

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type |  | Required |  |  |
| Geographic Area Requirements |  | One or more states, territories, or counties |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Manage bathymetric data as part of Oregon GIS Framework |
| MCA Title | Oregon Lakes and Reservoirs Research and Data Provisioning |
| MCA ID | 21630 |
| Organization Type | Academic or Not-for-Profit |
| Organization Name | Portland State University, Oregon |
| Sub-Agency or Division |  |
| Organization Mission | Public service research on lake water quality, consulting limnology, manage bathymetric data for Oregon |
| Program Name | Oregon GIS Framework, Elevation |
| Total Annual Program Budget |  |
| Primary Business Use | BU 01 - Water Supply and Quality |
| Secondary Business Use | BU 06 - Natural Resources Conservation |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Highly desirable |
| Tops of buildings, structures, objects | Not required |
| Tops of vegetation | Not required |
| Tops of submerged structures, objects | Highly desirable |
| Tops of submerged vegetation | Highly desirable |
| Subcanopy of vegetation/understory | Not required |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Highly desirable |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 1 sq mi - 49 sq mi |
| Smallest 3D features needed | Large features |
| Description of smallest 3D features | Limnological research on lakes, typically one lake |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Not required |
| Rivers and Streams |  |
| Less than 10 ft | Not required |
| 10 - 50 ft | Not required |
| 51 - 100 ft | Not required |
| 101 - 500 ft | Not required |
| 501 - 2,500 ft | Not required |
| Greater than 2,500 ft | Not required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Nice to have |
| ½ - 1 acre | Nice to have |
| 1.1 – 2 acres | Nice to have |
| 2.1 – 5 acres | Nice to have |
| 5.1 – 10 acres | Highly desirable |
| Greater than 10 acres | Required |
| Other | Required |
| Other description | Major tributaries especially those with dams |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order |  | Coarser bathymetric data satisfies my needs |  |  |
| Update Frequency |  | Event driven only – Data need to coincide with a specific event. |  |  |
| Event type(s) |  | Dam removal, changes to levee network, a flooding event, or a landslide that affects a river. |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error |  | Up to 10 meters |  |  |
| Acceptable Vertical Error |  | Up to 1 meter |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  | Partial |  |  |
| Cross section/transect requirement |  | Most of our lake bathymetry is collected with 50M+ transects with single beam sonar. with vertical accuracy of plus or minus a meter and a sampling frequency of 2 M |  |  |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season |  | Highly desirable |  |  |  |  |
| Entire AOI under same environmental conditions |  | Highly desirable |  |  |  |  |
| Other |  | Highly desirable |  |  |  |  |
| Other description |  | We collect the best available |  |  |  |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless |  | Not required |  |  |  |  |
| DEM for entire AOI needs to be seamless |  | Not required |  |  |  |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness |  |  |  |  |  |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM |  | Not required |  |  |
| DTM |  | Highly desirable |  |  |
| DEM |  | Highly desirable |  |  |
| Raw point cloud data |  | Not required |  |  |
| Classified point cloud |  | Highly desirable |  |  |
| Edited/cube XYZ |  | Not required |  |  |
| Full waveform |  | Highly desirable |  |  |
| Bathymetric Attributed Grid (BAG) |  | Required |  |  |
| Breaklines required for standard hydro-flattening |  | Not required |  |  |
| Additional breaklines for hydro-enforcement of culverts |  |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery |  | Not required |  |  |
| Ground control/ground truthing |  | Highly desirable |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery |  | Not required |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data |  | Highly desirable |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates |  | Highly desirable |  |  |
| Land use/land cover |  | Highly desirable |  |  |
| Wetlands |  | Highly desirable |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features |  | Required |  |  |
| Bridges/culverts |  | Nice to have |  |  |
| Landmark features |  | Nice to have |  |  |
| Cultural resources |  | Nice to have |  |  |
| Coastal and riverine structures |  | Highly desirable |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings |  |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  | We are collecting and publishing the best bathymetric data available for Oregon lakes and reservoirs for the elevation element of Oregon's GIS framework. We are partnering with the Portland Office of USACE but I assume they will address their interests. We are collecting the best bathymetry available and attempting to provide useful metadata for legacy surveys. |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Other |  | Yes |  |  |
| Other description |  | Various public and private surveys |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings |  | Moderate |  |  |
| Cost savings/cost reduction |  | Inland bathy data not available |  |  |
| Cost avoidance |  | Inland bathy data not available |  |  |
| Increased revenues |  | Inland bathy data not available |  |  |
| Mission-driven performance improvements |  | Inland bathy data not available |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services |  | Minor |  |  |
| Improved response or timeliness |  | Major |  |  |
| Improved customer experience |  | Moderate |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach |  | Moderate |  |  |
| Environmental |  | Moderate |  |  |
| Public safety, including life and property |  | Minor |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings |  |  |  | I don't know | Unable to provide |  |  |  |  |  |  |  |
| Cost savings/cost reduction |  |  |  | I don't know | Unable to provide |  |  |  |  |  |  |  |
| Cost avoidance |  |  |  | I don't know | Unable to provide |  |  |  |  |  |  |  |
| Increased revenues |  |  |  | Moderate | Annual dollars saved/realized | $1,000 |  |  |  |  |  |  |
| Mission-driven performance improvements |  |  |  | Major | Annual percent improvement | 22% |  |  |  |  |  |  |
| Other operational benefits |  |  |  | Moderate | Annual dollars saved/realized | $1,000 |  |  |  |  |  |  |
| Other operational benefits description |  |  |  |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services |  |  |  | I don't know | Unable to provide |  |  |  |  |  |  |  |
| Improved response or timeliness |  |  |  | I don't know | Unable to provide |  |  |  |  |  |  |  |
| Improved customer experience |  |  |  | Moderate | Annual dollars saved/realized | $2,000 |  |  |  |  |  |  |
| Other customer service benefits |  |  |  | Major | Annual dollars saved/realized | $5,000 |  |  |  |  |  |  |
| Other customer service benefits description |  |  |  | Provide data on web. |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach |  |  |  | Moderate |  |  |  |  |  |  |  |  |
| Education or outreach description |  |  |  | We use in teaching, research, and public service |  |  |  |  |  |  |  |  |
| Environmental |  |  |  | I don't know |  |  |  |  |  |  |  |  |
| Public safety, including life and property |  |  |  | I don't know |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  | Yes |  |  |
| Contours |  | Yes |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  | Yes |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  | Yes |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  | Yes |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 3 |
| Vertical accuracy | 2 |
| Update frequency | 1 |

# MCA Title: Water Supply and Quality

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type |  | Required | Required |  |
| Geographic Area Requirements |  | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Fate and transport of contaminants. Pollution risk mitigation. Runoff and sedimentation analyses. Point- or non-point source pollution modeling. Management of contaminants and marine debris - point, non-point, vessel, and atmospheric pollution; spills; trash. |
| MCA Title | Water Supply and Quality |
| MCA ID | 60349 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 01 - Water Supply and Quality |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Highly desirable |
| Tops of buildings, structures, objects | Nice to have |
| Tops of vegetation | Nice to have |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Highly desirable |
| Rivers and Streams |  |
| Less than 10 ft | Highly desirable |
| 10 - 50 ft | Highly desirable |
| 51 - 100 ft | Highly desirable |
| 101 - 500 ft | Highly desirable |
| 501 - 2,500 ft | Highly desirable |
| Greater than 2,500 ft | Highly desirable |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Highly desirable |
| 1.1 – 2 acres | Highly desirable |
| 2.1 – 5 acres | Highly desirable |
| 5.1 – 10 acres | Highly desirable |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order |  | QL1B | QL1B |  |
| Update Frequency |  | 6-10 years | 4-5 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error |  | Up to 2 meters | Up to 2 meters |  |
| Acceptable Vertical Error |  | Up to 30 cm | Up to 30 cm |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season |  | Nice to have | Nice to have |  |  | Nice to have |
| Entire AOI under same environmental conditions |  | Highly desirable | Highly desirable |  |  | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless |  | Highly desirable | Nice to have |  |  | Nice to have |
| DEM for entire AOI needs to be seamless |  | Required | Highly desirable |  |  | Highly desirable |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM |  | Highly desirable | Nice to have |  |
| DTM |  | Highly desirable | Nice to have |  |
| DEM |  | Required | Required |  |
| Raw point cloud data |  | Nice to have | Nice to have |  |
| Classified point cloud |  | Nice to have | Nice to have |  |
| Edited/cube XYZ |  | Nice to have | Nice to have |  |
| Full waveform |  | Nice to have | Nice to have |  |
| Bathymetric Attributed Grid (BAG) |  | Nice to have | Nice to have |  |
| Breaklines required for standard hydro-flattening |  | Highly desirable |  |  |
| Additional breaklines for hydro-enforcement of culverts |  |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Nice to have |  |
| Tide Predictions |  |  | Nice to have |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Nice to have |  |
| Intensity imagery/sidescan imagery |  | Nice to have | Nice to have |  |
| Ground control/ground truthing |  | Highly desirable | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Highly desirable |  |
| Nautical and/or navigation charts |  |  | Nice to have |  |
| Acoustic imagery of the seafloor |  |  | Nice to have |  |
| Aerial and/or satellite imagery |  | Highly desirable | Highly desirable |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Nice to have |  |
| Bottom type |  |  | Nice to have |  |
| Submerged features |  |  | Nice to have |  |
| Subbottom characteristics |  |  | Nice to have |  |
| Geologic and seismic data |  | Nice to have | Nice to have |  |
| Water column properties - Physical |  |  | Nice to have |  |
| Water column properties - Chemical |  |  | Nice to have |  |
| Water column properties - Biological |  |  | Nice to have |  |
| Currents |  |  | Nice to have |  |
| Tide/wave heights |  |  | Nice to have |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Nice to have |  |
| Boundaries |  |  | Nice to have |  |
| Routes |  |  | Nice to have |  |
| Offshore cadastral |  |  | Nice to have |  |
| Lease areas |  |  | Nice to have |  |
| Fixed obstructions |  |  | Nice to have |  |
| Floating observation/navigation systems |  |  | Nice to have |  |
| Shorelines – current, historic, change rates |  | Highly desirable | Highly desirable |  |
| Land use/land cover |  | Highly desirable | Highly desirable |  |
| Wetlands |  | Highly desirable | Highly desirable |  |
| Estuaries |  |  | Required |  |
| Inland surface water features |  | Required | Highly desirable |  |
| Bridges/culverts |  | Highly desirable |  |  |
| Landmark features |  | Nice to have | Nice to have |  |
| Cultural resources |  | Nice to have | Nice to have |  |
| Coastal and riverine structures |  | Highly desirable | Highly desirable |  |
| Overhead structures |  |  | Nice to have |  |
| Lowest Floor Elevation of Buildings |  |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings |  | Moderate | Moderate |  |
| Cost savings/cost reduction |  | Minor | Moderate |  |
| Cost avoidance |  | Minor | Minor |  |
| Increased revenues |  | None | Minor |  |
| Mission-driven performance improvements |  | Minor | Minor |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services |  | Minor | Moderate |  |
| Improved response or timeliness |  | Minor | Moderate |  |
| Improved customer experience |  | Minor | Moderate |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach |  | Minor | Minor |  |
| Environmental |  | Moderate | Moderate |  |
| Public safety, including life and property |  | Minor | Moderate |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings |  |  |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost savings/cost reduction |  |  |  | Major | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Cost avoidance |  |  |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Increased revenues |  |  |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  |  |  |  | Annual dollars saved/ realized | $2,327,748 |  | Annual dollars saved/ realized | $8,886 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services |  |  |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved response or timeliness |  |  |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved customer experience |  |  |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  |  |  |  | Annual dollars saved/ realized | $243,259 |  | Annual dollars saved/ realized | $14,639 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach |  |  |  | Moderate |  |  | Moderate |  |  |  |  |  |
| Environmental |  |  |  | Major |  |  | Moderate |  |  |  |  |  |
| Public safety, including life and property |  |  |  | Moderate |  |  | Moderate |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Riverine Ecosystem Management

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Stream channel analysis and mapping. Stream bank erosion analysis. Aquatic and terrestrial species habitat management. Environmental management. |
| MCA Title | Riverine Ecosystem Management |
| MCA ID | 60350 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 02 - Riverine Ecosystem Management |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Nice to have |
| Tops of vegetation | Nice to have |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Highly desirable |
| Rivers and Streams |  |
| Less than 10 ft | Nice to have |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Nice to have |
| Greater than 2,500 ft | Nice to have |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Highly desirable |
| 1.1 – 2 acres | Highly desirable |
| 2.1 – 5 acres | Highly desirable |
| 5.1 – 10 acres | Highly desirable |
| Greater than 10 acres | Highly desirable |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 HD | QL0B | QL0B |  |
| Update Frequency | 6-10 years | 4-5 years | 2-3 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | Up to 2 meters | Up to 2 meters |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 30 cm | Up to 30 cm |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Required |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Highly desirable |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have | Nice to have | Highly desirable |  | Nice to have | Highly desirable |
| Entire AOI under same environmental conditions | Highly desirable | Highly desirable | Required |  | Highly desirable | Required |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Nice to have |
| DEM for entire AOI needs to be seamless | Highly desirable | Required | Required |  | Highly desirable | Required |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | I don't know | I don't know |  | I don't know | I don't know |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required | Highly desirable | Highly desirable |  |
| DTM | Required | Highly desirable | Required |  |
| DEM | Required | Required | Required |  |
| Raw point cloud data | Nice to have | Nice to have | Nice to have |  |
| Classified point cloud | Required | Highly desirable | Highly desirable |  |
| Edited/cube XYZ |  | Not required | Nice to have |  |
| Full waveform | Nice to have | Nice to have | Highly desirable |  |
| Bathymetric Attributed Grid (BAG) |  | Not required | Nice to have |  |
| Breaklines required for standard hydro-flattening | Highly desirable | Nice to have |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Nice to have |  |
| Tide Predictions |  |  | Not required |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Not required |  |
| Intensity imagery/sidescan imagery | Highly desirable | Nice to have | Nice to have |  |
| Ground control/ground truthing | Highly desirable | Highly desirable | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Required |  |
| Nautical and/or navigation charts |  |  | Not required |  |
| Acoustic imagery of the seafloor |  |  | Highly desirable |  |
| Aerial and/or satellite imagery | Required | Highly desirable | Required |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Highly desirable |  |
| Bottom type |  |  | Required |  |
| Submerged features |  |  | Highly desirable |  |
| Subbottom characteristics |  |  | Highly desirable |  |
| Geologic and seismic data | Nice to have | Nice to have | Not required |  |
| Water column properties - Physical |  |  | Nice to have |  |
| Water column properties - Chemical |  |  | Nice to have |  |
| Water column properties - Biological |  |  | Nice to have |  |
| Currents |  |  | Nice to have |  |
| Tide/wave heights |  |  | Not required |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Nice to have |  |
| Boundaries |  |  | Not required |  |
| Routes |  |  | Not required |  |
| Offshore cadastral |  |  | Not required |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Not required |  |
| Floating observation/navigation systems |  |  | Not required |  |
| Shorelines – current, historic, change rates | Nice to have | Nice to have | Highly desirable |  |
| Land use/land cover | Highly desirable | Nice to have | Required |  |
| Wetlands | Highly desirable | Highly desirable | Required |  |
| Estuaries |  |  | Highly desirable |  |
| Inland surface water features | Required | Required | Required |  |
| Bridges/culverts | Required | Highly desirable |  |  |
| Landmark features | Highly desirable | Nice to have | Not required |  |
| Cultural resources | Nice to have | Nice to have | Not required |  |
| Coastal and riverine structures | Highly desirable | Highly desirable | Highly desirable |  |
| Overhead structures |  |  | Not required |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Major | Moderate |  |
| Cost savings/cost reduction | Moderate | Minor | Moderate |  |
| Cost avoidance | Moderate | None | None |  |
| Increased revenues | None | None | None |  |
| Mission-driven performance improvements | Major | None | Minor |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | Minor | Minor |  |
| Improved response or timeliness | Moderate | Moderate | Minor |  |
| Improved customer experience | Major | Minor | Minor |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major | Moderate | Minor |  |
| Environmental | Major | Major | Moderate |  |
| Public safety, including life and property | Major | Moderate | Moderate |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Moderate | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost savings/cost reduction | Moderate | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Increased revenues | Minor | Unable to provide |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $148,538 |  | Annual dollars saved/ realized | $79,072 |  | Annual dollars saved/ realized | $26,950 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $14,836 |  | Annual dollars saved/ realized | $5,575 |  | Annual dollars saved/ realized | $20,594 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Major |  |  | Major |  |  | Moderate |  |  |  |  |  |
| Environmental | Major |  |  | Major |  |  | Moderate |  |  |  |  |  |
| Public safety, including life and property | Major |  |  | Major |  |  | Moderate |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Coastal Community Resiliency

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  | Required |  |
| Geographic Area Requirements | One or more Hydrologic Units (HUC4s) |  | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Coastal hazards mitigation planning through identification of coastal erosion rates and hazard zones, flood hazard zones, landslide hazard zones and tsunami inundation hazard zones. |
| MCA Title | Coastal Community Resiliency |
| MCA ID | 21606 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Coastal Management Program |
| Sub-Agency or Division |  |
| Organization Mission | The mission of the Oregon Coastal Management Program is to work in partnership with coastal local governments, state and federal agencies, and other stakeholders to ensure that Oregon's coastal and ocean resources are managed, conserved, and developed consistent with statewide planning goals and the coastal zone management act. |
| Program Name | Natural Hazards program |
| Total Annual Program Budget |  |
| Primary Business Use | BU 03 - Coastal Zone Management |
| Secondary Business Use | BU 10 - Geologic Assessment and Hazard Mitigation |
| Tertiary Business Use | BU 16 - Sea Level Rise and Subsidence |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Nice to have |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Highly desirable |
| River/lake bottom | Highly desirable |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Required |
| Ocean/sea bottom (>10 m deep) | Required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 1 sq mi - 49 sq mi |
| Smallest 3D features needed | Small features |
| Description of smallest 3D features | Tidegates and culverts in estuarine systems. |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 |  | QL1B |  |
| Update Frequency | 4-5 years |  | 2-3 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter |  | The best horizontal accuracy achievable for the vertical accuracy I need |  |
| Acceptable Vertical Error | Up to 20 cm |  | Up to 30 cm |  |
| How far onshore needed |  |  | To cover the coastal uplands |  |
| How far down the beach profile needed | Below MLLW |  | Below MLLW |  |
| Tide correction requirement |  |  | No requirement for tide correction |  |
| Cross sections and/or transects meet needs | Partial |  | Partial |  |
| Cross section/transect requirement | Cross-shore coastal transects (for beaches and rocky shores), 10cm vertical accuracy, 1m sampling density along the transect, and no more than 100m between transects. |  | 50m spacing. |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Not required |
| Hydro-enforcement | Not required |
| Hydro-conditioning | Not required |
| No Treatment | Required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable |  | Highly desirable |  |  | Nice to have |
| Entire AOI under same environmental conditions | Highly desirable |  | Highly desirable |  |  | Nice to have |
| Other | Required |  |  |  |  |  |
| Other description | Prefer that tidal areas are collected during low tide windows |  |  |  |  |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required |  | Highly desirable |  |  | Nice to have |
| DEM for entire AOI needs to be seamless | Required |  | Highly desirable |  |  | Nice to have |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to triple the required TVU at the 95% confidence level |  | Whatever it takes to achieve seamlessness, including changes to the older, previously accepted dataset if it is proven to be less accurate than the newer |  |  | Whatever it takes to achieve seamlessness, including changes to the older, previously accepted dataset if it is proven to be less accurate than the newer |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Nice to have |  | Not required |  |
| DTM | Not required |  | Not required |  |
| DEM | Required |  | Required |  |
| Raw point cloud data | Not required |  | Not required |  |
| Classified point cloud | Nice to have |  | Nice to have |  |
| Edited/cube XYZ |  |  | Not required |  |
| Full waveform | Not required |  | Not required |  |
| Bathymetric Attributed Grid (BAG) |  |  | Not required |  |
| Breaklines required for standard hydro-flattening | Nice to have |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Required |  |
| Tide Predictions |  |  | Required |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Nice to have |  |
| Intensity imagery/sidescan imagery | Highly desirable |  | Not required |  |
| Ground control/ground truthing | Highly desirable |  | Nice to have |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Not required |  |
| Nautical and/or navigation charts |  |  | Not required |  |
| Acoustic imagery of the seafloor |  |  | Not required |  |
| Aerial and/or satellite imagery | Highly desirable |  | Required |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Not required |  |
| Bottom type |  |  | Nice to have |  |
| Submerged features |  |  | Not required |  |
| Subbottom characteristics |  |  | Not required |  |
| Geologic and seismic data | Not required |  | Not required |  |
| Water column properties - Physical |  |  | Not required |  |
| Water column properties - Chemical |  |  | Not required |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  | Not required |  |
| Tide/wave heights |  |  | Highly desirable |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Nice to have |  |
| Boundaries |  |  | Nice to have |  |
| Routes |  |  | Not required |  |
| Offshore cadastral |  |  | Not required |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Not required |  |
| Floating observation/navigation systems |  |  | Not required |  |
| Shorelines – current, historic, change rates | Highly desirable |  | Highly desirable |  |
| Land use/land cover | Not required |  | Nice to have |  |
| Wetlands | Highly desirable |  | Nice to have |  |
| Estuaries |  |  | Nice to have |  |
| Inland surface water features | Highly desirable |  | Nice to have |  |
| Bridges/culverts | Highly desirable |  |  |  |
| Landmark features | Not required |  | Not required |  |
| Cultural resources | Not required |  | Not required |  |
| Coastal and riverine structures | Highly desirable |  | Nice to have |  |
| Overhead structures |  |  | Not required |  |
| Lowest Floor Elevation of Buildings | Not required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Oregon Lidar Consortium data circa 2008-2009, JALBTCX data from 2010, 2014, and USGS data from 2016 |  | JALBTCX data from 2010, 2014 |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast | Yes |  | Yes |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  | Yes |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major |  | Moderate |  |
| Cost savings/cost reduction | Major |  | Major |  |
| Cost avoidance | Major |  | Major |  |
| Increased revenues | None |  | Moderate |  |
| Mission-driven performance improvements | Major |  | Major |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major |  | Moderate |  |
| Improved response or timeliness | Major |  | Moderate |  |
| Improved customer experience | Major |  | Moderate |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major |  | Minor |  |
| Environmental | I don't know |  | Major |  |
| Public safety, including life and property | Major |  | Moderate |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Cost savings/cost reduction | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Increased revenues | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $937,584 |  |  |  |  | Annual dollars saved/ realized | $30,174 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $163,909 |  |  |  |  | Annual dollars saved/ realized | $51,044 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Moderate |  |  |  |  |  | Major |  |  |  |  |  |
| Environmental | Major |  |  |  |  |  | Major |  |  |  |  |  |
| Public safety, including life and property | Major |  |  |  |  |  | Major |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) | Yes |  | Yes |  |
| Contours |  |  | Yes |  |
| Hillshades | Yes |  | Yes |  |
| Slope maps | Yes |  | Yes |  |
| Aspect maps | Yes |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  | Yes |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) | Yes |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) | Yes |  |  |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 1 |
| Vertical accuracy | 2 |
| Update frequency | 3 |

# MCA Title: Ocean Shore Management for Protection Structure Permitting and Recreation Access

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  | Highly desirable |  |
| Geographic Area Requirements | Custom description |  | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements | 1000-meter swath along Oregon's shoreline |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Ocean shore management for protection structure permitting and recreation access. Shore management includes permitting for structures on the beach such as armoring (rip rap, other ways to protect property). Property owners have to apply for a permit. Recreation access – the public has a right to access the entire state shoreline, lidar helps manage access and areas that require access. A swath of data both onshore and nearshore is needed. Topographic data is required, topobathy is highly desirable. |
| MCA Title | Ocean Shore Management for Protection Structure Permitting and Recreation Access |
| MCA ID | 21670 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Parks and Recreation Department |
| Sub-Agency or Division |  |
| Organization Mission | The mission of the Oregon Parks and Recreation Department is to provide and protect outstanding natural, scenic, cultural, historic and recreational sites for the enjoyment and education of present and future generations |
| Program Name | Oregon Parks and Recreation Department Stewardship Division |
| Total Annual Program Budget |  |
| Primary Business Use | BU 03 - Coastal Zone Management |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Not required |
| Nearshore elevation (<10 m deep) | Highly desirable |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 50 sq mi - 999 sq mi (e.g. small county or County Equivalent, District of Columbia, etc.) |
| Smallest 3D features needed | Survey-level features |
| Description of smallest 3D features | Rock used for shoreline protection structures, approximately 30cm-100cm |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 |  | QL4B |  |
| Update Frequency | 4-5 years |  | 4-5 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter |  | Up to 2 meters |  |
| Acceptable Vertical Error | Up to 20 cm |  | Up to 30 cm |  |
| How far onshore needed |  |  | To cover the beach slope |  |
| How far down the beach profile needed | To MLLW |  | To MLLW |  |
| Tide correction requirement |  |  | No requirement for tide correction |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Nice to have |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have |  | Nice to have |  |  | Highly desirable |
| Entire AOI under same environmental conditions | Nice to have |  | Nice to have |  |  | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable |  | Highly desirable |  |  | Nice to have |
| DEM for entire AOI needs to be seamless | Highly desirable |  | Highly desirable |  |  | Highly desirable |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | >10cm manipulation |  | I don't know |  |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required |  | Required |  |
| DTM | Required |  | Nice to have |  |
| DEM | Required |  | Required |  |
| Raw point cloud data | Not required |  | Not required |  |
| Classified point cloud | Required |  | Required |  |
| Edited/cube XYZ |  |  | Not required |  |
| Full waveform | Nice to have |  | Not required |  |
| Bathymetric Attributed Grid (BAG) |  |  | Not required |  |
| Breaklines required for standard hydro-flattening | Nice to have |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Nice to have |  |
| Tide Predictions |  |  | Not required |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Not required |  |
| Intensity imagery/sidescan imagery | Required |  | Required |  |
| Ground control/ground truthing | Required |  | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Not required |  |
| Nautical and/or navigation charts |  |  | Not required |  |
| Acoustic imagery of the seafloor |  |  | Not required |  |
| Aerial and/or satellite imagery | Not required |  | Highly desirable |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Not required |  |
| Bottom type |  |  | Not required |  |
| Submerged features |  |  | Not required |  |
| Subbottom characteristics |  |  | Not required |  |
| Geologic and seismic data | Nice to have |  | Nice to have |  |
| Water column properties - Physical |  |  | Not required |  |
| Water column properties - Chemical |  |  | Not required |  |
| Water column properties - Biological |  |  | Not required |  |
| Currents |  |  | Not required |  |
| Tide/wave heights |  |  | Nice to have |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Not required |  |
| Boundaries |  |  | Not required |  |
| Routes |  |  | Not required |  |
| Offshore cadastral |  |  | Not required |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Not required |  |
| Floating observation/navigation systems |  |  | Not required |  |
| Shorelines – current, historic, change rates | Required |  | Required |  |
| Land use/land cover | Nice to have |  | Nice to have |  |
| Wetlands | Nice to have |  | Not required |  |
| Estuaries |  |  | Not required |  |
| Inland surface water features | Not required |  | Not required |  |
| Bridges/culverts | Not required |  |  |  |
| Landmark features | Not required |  | Not required |  |
| Cultural resources | Not required |  | Not required |  |
| Coastal and riverine structures | Required |  | Required |  |
| Overhead structures |  |  | Nice to have |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Mix of USGS, state datasets, best available data |  | Bathymetric data from Oregon State University, ESRI |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map | Yes |  |  |  |
| Digital Coast | Yes |  | Yes |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories | Yes |  |  |  |
| State repositories used | Oregon |  |  |  |
| Other | Yes |  | Yes |  |
| Other description | Oregon State University, ESRI, State Data Center |  | Oregon State University, ESRI |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major |  | Moderate |  |
| Cost savings/cost reduction | Major |  | Minor |  |
| Cost avoidance | Major |  | Moderate |  |
| Increased revenues | None |  | None |  |
| Mission-driven performance improvements | None |  | None |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Minor |  | None |  |
| Improved response or timeliness | Major |  | Minor |  |
| Improved customer experience | Major |  | None |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | I don't know |  | I don't know |  |
| Environmental | Minor |  | Minor |  |
| Public safety, including life and property | Minor |  | Minor |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Moderate | Annual dollars saved/realized | $14,472 |  |  |  | Moderate | Annual dollars saved/realized | $14,472 |  |  |  |
| Time savings description | 10 hrs/mo reduced data development time = 120 hrs/yr. 10 hrs/mo planning savings = 120 hrs/yr. 20 hrs/mo reduced field visit time = 240 hrs/yr. |  |  |  |  |  | 10 hrs/mo reduced data development time = 120 hrs/yr. 20 hrs/mo reduced field visit time = 480 hrs/yr. 10 hrs/mo planning savings = 120 hrs/yr. |  |  |  |  |  |
| Cost savings/cost reduction | Minor | Annual dollars saved/realized | $5,000 |  |  |  | Minor | Annual dollars saved/realized | $5,000 |  |  |  |
| Cost savings/cost reduction description | Potential data acquisition savings. |  |  |  |  |  | Potential data acquisition savings. |  |  |  |  |  |
| Cost avoidance | Minor | Unable to provide |  |  |  |  | Minor | Unable to provide |  |  |  |  |
| Increased revenues | I don't know | Unable to provide |  |  |  |  | I don't know | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Minor | Unable to provide |  |  |  |  | Minor | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $4,290 |  |  |  |  | Annual dollars saved/ realized | $11,000 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Minor | Unable to provide |  |  |  |  | Minor | Unable to provide |  |  |  |  |
| Improved response or timeliness | Minor | Annual dollars saved/realized | $1,809 |  |  |  | Minor | Annual dollars saved/realized | $1,809 |  |  |  |
| Improved response or timeliness description | 5 hrs/mo in time savings = 60 hrs/yr. |  |  |  |  |  | 5 hrs/mo in time savings = 60 hrs/yr. |  |  |  |  |  |
| Improved customer experience | Minor | Unable to provide |  |  |  |  | Minor | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $2,330 |  |  |  |  | Annual dollars saved/ realized | $49,272 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | I don't know |  |  |  |  |  | I don't know |  |  |  |  |  |
| Environmental | Minor |  |  |  |  |  | Minor |  |  |  |  |  |
| Environmental description | Possible reduction of shoreline armoring or increase in green infrastructure |  |  |  |  |  | Possible reduction of shoreline armoring or increase in green infrastructure |  |  |  |  |  |
| Public safety, including life and property | I don't know |  |  |  |  |  | I don't know |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades | Yes |  | Yes |  |
| Slope maps | Yes |  | Yes |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps | Yes |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) | Yes |  | Yes |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) | Yes |  | Yes |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 1 |
| Vertical accuracy | 2 |
| Update frequency | 3 |

# MCA Title: Statewide Wildfire Management and Forest Management

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  |  |  |
| Geographic Area Requirements | One or more states, territories, or counties |  |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | All aspects of forestry and the supporting activities. Forest resources management; Timber harvests; Wildfire management; Homeland security, law enforcement, disaster response, and emergency management; Recreation; and Telecommunications (used for siting radio repeater sites across the state for self and other agencies. Topo is used for line of sight and coverage analysis). For forestry, Topographic Position Index (TPI) analysis is run on DEMs. This is a neighborhood analysis used to see the relationship of a cell to its neighbors; negative indicates a valley, positive is a ridge, neutral is flat. TPI is used to identify stream channels, valleys, draws, etc. It is used for fire documentation and enforcement of Oregon Forest Practice Act. The regulations are based on proximity to streams. High quality lidar data is critical for all aspects of our mission. |
| MCA Title | Statewide Wildfire Management and Forest Management |
| MCA ID | 1360 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Department of Forestry |
| Sub-Agency or Division |  |
| Organization Mission | Forest Management and Fire Protection |
| Program Name | Enforcement of the Oregon Forest Practices Act |
| Total Annual Program Budget |  |
| Primary Business Use | BU 04 - Forest Resources Management |
| Secondary Business Use | BU 17 - Wildfire Management, Planning, and Response |
| Tertiary Business Use | BU 06 - Natural Resources Conservation |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Required |
| River/lake bottom | Nice to have |
| Nearshore elevation (<10 m deep) | Nice to have |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 75,000 sq mi - 199,999 sq mi (e.g. large state or medium multi-state region) |
| Smallest 3D features needed | Small features |
| Description of smallest 3D features | Tree seedlings, small ditches, and small waterfalls |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL0 HD |  |  |  |
| Update Frequency | 6-10 years |  |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 30 cm |  |  |  |
| Acceptable Vertical Error | Up to 10 cm |  |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed | To MHW |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs | Partial |  |  |  |
| Cross section/transect requirement | Accurate positional locations of roads and streams. 5cm vertical. |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable |  |  |  |  |  |
| Entire AOI under same environmental conditions | Nice to have |  |  |  |  |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required |  |  |  |  |  |
| DEM for entire AOI needs to be seamless | Required |  |  |  |  |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to double the required TVU at the 95% confidence level |  |  |  |  |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required |  |  |  |
| DTM | Required |  |  |  |
| DEM | Required |  |  |  |
| Raw point cloud data | Nice to have |  |  |  |
| Classified point cloud | Highly desirable |  |  |  |
| Edited/cube XYZ |  |  |  |  |
| Full waveform | Nice to have |  |  |  |
| Bathymetric Attributed Grid (BAG) |  |  |  |  |
| Breaklines required for standard hydro-flattening | Nice to have |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Nice to have |  |  |  |
| Ground control/ground truthing | Nice to have |  |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Highly desirable |  |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Not required |  |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Nice to have |  |  |  |
| Land use/land cover | Nice to have |  |  |  |
| Wetlands | Nice to have |  |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Highly desirable |  |  |  |
| Bridges/culverts | Highly desirable |  |  |  |
| Landmark features | Nice to have |  |  |  |
| Cultural resources | Nice to have |  |  |  |
| Coastal and riverine structures | Highly desirable |  |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Stream channel locations are critical for enforcement of the Oregon Forest Practices Act. Best available from USGS, 10m DEMs, state, other partners |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map | Yes |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories | Yes |  |  |  |
| State repositories used | Oregon |  |  |  |
| Other | Yes |  |  |  |
| Other description | Partners |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major |  |  |  |
| Cost savings/cost reduction | Major |  |  |  |
| Cost avoidance | Moderate |  |  |  |
| Increased revenues | Minor |  |  |  |
| Mission-driven performance improvements | Moderate |  |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate |  |  |  |
| Improved response or timeliness | Moderate |  |  |  |
| Improved customer experience | Moderate |  |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Moderate |  |  |  |
| Environmental | Moderate |  |  |  |
| Public safety, including life and property | Moderate |  |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Annual dollars saved/realized | $256,878 |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Moderate | Annual dollars saved/realized | $10,000 |  |  |  |  |  |  |  |  |  |
| Cost avoidance | I don't know | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Increased revenues | Major | Annual dollars saved/realized | $240,000 |  |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Moderate | Annual percent improvement | 6% |  |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements description | High quality lidar data is critical for all aspects of our mission. |  |  |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Moderate | Annual dollars saved/realized | $14,472 |  |  |  |  |  |  |  |  |  |
| Improved response or timeliness | Minor | Annual dollars saved/realized | $43,416 |  |  |  |  |  |  |  |  |  |
| Improved customer experience | Minor | Annual dollars saved/realized | $11,216 |  |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Minor |  |  |  |  |  |  |  |  |  |  |  |
| Education or outreach description | Tree farmer education |  |  |  |  |  |  |  |  |  |  |  |
| Environmental | Moderate |  |  |  |  |  |  |  |  |  |  |  |
| Environmental description | Regulation enforcement |  |  |  |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Moderate |  |  |  |  |  |  |  |  |  |  |  |
| Public safety, including life and property description | Fire protection |  |  |  |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours | Yes |  |  |  |
| Hillshades | Yes |  |  |  |
| Slope maps | Yes |  |  |  |
| Aspect maps | Yes |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections | Yes |  |  |  |
| Height-Above-Ground maps | Yes |  |  |  |
| Viewshed maps | Yes |  |  |  |
| Hydrologic Flow Direction Grids | Yes |  |  |  |
| Hydrologic Flow Accumulation Grids | Yes |  |  |  |
| Hydrologic networks (e.g. streams, lakes) | Yes |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) | Yes |  |  |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness | Yes |  |  |  |
| Other (please specify) | Yes |  |  |  |
| Other description | Topographic Position Index |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 3 |
| Vertical accuracy | 2 |
| Update frequency | 1 |

# MCA Title: Natural Resources Conservation

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required |  |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Conservation engineering. Soils and wetlands mapping and characterization. Modeling of biological and ecological systems. Erosion control. Rainfall penetration studies, impervious surfaces. Assessment of blue carbon stocks. |
| MCA Title | Natural Resources Conservation |
| MCA ID | 60351 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 06 - Natural Resources Conservation |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Highly desirable |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Highly desirable |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Nice to have |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Required |
| Rivers and Streams |  |
| Less than 10 ft | Required |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Highly desirable |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Required |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 HD | QL0B |  |  |
| Update Frequency | 4-5 years | 6-10 years |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | Up to 2 meters |  |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 30 cm |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Required |
| Hydro-enforcement | Required |
| Hydro-conditioning | Highly desirable |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Required |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Required | Highly desirable |  |  | Required |  |
| Entire AOI under same environmental conditions | Required | Required |  |  | Required |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required | Required |  |  | Required |  |
| DEM for entire AOI needs to be seamless | Required | Required |  |  | Required |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required | Highly desirable |  |  |
| DTM | Required | Required |  |  |
| DEM | Required | Required |  |  |
| Raw point cloud data | Required | Required |  |  |
| Classified point cloud | Required | Required |  |  |
| Edited/cube XYZ |  | Nice to have |  |  |
| Full waveform | Nice to have | Nice to have |  |  |
| Bathymetric Attributed Grid (BAG) |  | Highly desirable |  |  |
| Breaklines required for standard hydro-flattening | Required | Required |  |  |
| Additional breaklines for hydro-enforcement of culverts | Required |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Highly desirable | Highly desirable |  |  |
| Ground control/ground truthing | Required | Required |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Required | Required |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Nice to have | Nice to have |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Highly desirable | Required |  |  |
| Land use/land cover | Required | Required |  |  |
| Wetlands | Required | Required |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Required | Required |  |  |
| Bridges/culverts | Required | Required |  |  |
| Landmark features | Highly desirable | Nice to have |  |  |
| Cultural resources | Nice to have | Nice to have |  |  |
| Coastal and riverine structures | Highly desirable | Required |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Major |  |  |
| Cost savings/cost reduction | Major | Major |  |  |
| Cost avoidance | Major | Moderate |  |  |
| Increased revenues | Minor | Minor |  |  |
| Mission-driven performance improvements | Major | Major |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | Major |  |  |
| Improved response or timeliness | Major | Major |  |  |
| Improved customer experience | Major | Moderate |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major | Moderate |  |  |
| Environmental | Major | Major |  |  |
| Public safety, including life and property | Major | Moderate |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Increased revenues | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $3,806,356 |  | Annual dollars saved/ realized | $515,908 |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $13,978,100 |  | Annual dollars saved/ realized | $23,225 |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Major |  |  | Major |  |  |  |  |  |  |  |  |
| Environmental | Major |  |  | Major |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Major |  |  | Major |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Wildlife and Habitat Management

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Conservation planning for wildlife refuges and marine sanctuaries. Conservation of critical habitats. Management of diverse migratory bird habitats, coral reef and coral communities, marine mammals, protected fish species, and trust resources. |
| MCA Title | Wildlife and Habitat Management |
| MCA ID | 60352 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 07 - Wildlife and Habitat Management |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Nice to have |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Highly desirable |
| Subcanopy of vegetation/understory | Highly desirable |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Nice to have |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Highly desirable |
| Rivers and Streams |  |
| Less than 10 ft | Highly desirable |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Nice to have |
| ½ - 1 acre | Highly desirable |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL2 | QL2B | QL0B |  |
| Update Frequency | 4-5 years | 4-5 years | 4-5 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | Up to 5 meters | Up to 2 meters |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 40 cm | Up to 30 cm |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Nice to have |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Entire AOI under same environmental conditions | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| DEM for entire AOI needs to be seamless | Required | Highly desirable | Highly desirable |  | Highly desirable | Required |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Highly desirable | Highly desirable | Highly desirable |  |
| DTM | Highly desirable | Highly desirable | Highly desirable |  |
| DEM | Required | Required | Required |  |
| Raw point cloud data | Highly desirable | Nice to have | Nice to have |  |
| Classified point cloud | Highly desirable | Nice to have | Nice to have |  |
| Edited/cube XYZ |  | Nice to have | Nice to have |  |
| Full waveform | Not required | Not required | Not required |  |
| Bathymetric Attributed Grid (BAG) |  | Nice to have | Nice to have |  |
| Breaklines required for standard hydro-flattening | Nice to have | Nice to have |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Nice to have |  |
| Tide Predictions |  |  | Nice to have |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Nice to have |  |
| Intensity imagery/sidescan imagery | Nice to have | Nice to have | Nice to have |  |
| Ground control/ground truthing | Highly desirable | Highly desirable | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Highly desirable |  |
| Nautical and/or navigation charts |  |  | Nice to have |  |
| Acoustic imagery of the seafloor |  |  | Nice to have |  |
| Aerial and/or satellite imagery | Required | Required | Required |  |
| Underwater videography |  |  | Nice to have |  |
| Bottom texture |  |  | Highly desirable |  |
| Bottom type |  |  | Highly desirable |  |
| Submerged features |  |  | Highly desirable |  |
| Subbottom characteristics |  |  | Nice to have |  |
| Geologic and seismic data | Nice to have | Nice to have | Nice to have |  |
| Water column properties - Physical |  |  | Nice to have |  |
| Water column properties - Chemical |  |  | Nice to have |  |
| Water column properties - Biological |  |  | Nice to have |  |
| Currents |  |  | Nice to have |  |
| Tide/wave heights |  |  | Nice to have |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Highly desirable |  |
| Boundaries |  |  | Highly desirable |  |
| Routes |  |  | Nice to have |  |
| Offshore cadastral |  |  | Not required |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Nice to have |  |
| Floating observation/navigation systems |  |  | Nice to have |  |
| Shorelines – current, historic, change rates | Highly desirable | Highly desirable | Highly desirable |  |
| Land use/land cover | Highly desirable | Highly desirable | Highly desirable |  |
| Wetlands | Highly desirable | Highly desirable | Highly desirable |  |
| Estuaries |  |  | Highly desirable |  |
| Inland surface water features | Required | Required | Highly desirable |  |
| Bridges/culverts | Nice to have | Nice to have |  |  |
| Landmark features | Nice to have | Highly desirable | Nice to have |  |
| Cultural resources | Nice to have | Nice to have | Nice to have |  |
| Coastal and riverine structures | Nice to have | Highly desirable | Highly desirable |  |
| Overhead structures |  |  | Nice to have |  |
| Lowest Floor Elevation of Buildings | Not required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Moderate | Minor |  |
| Cost savings/cost reduction | Major | Minor | Minor |  |
| Cost avoidance | Major | Minor | Moderate |  |
| Increased revenues | None | Minor | None |  |
| Mission-driven performance improvements | Moderate | Moderate | Moderate |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate | Moderate | Minor |  |
| Improved response or timeliness | Minor | Minor | Minor |  |
| Improved customer experience | Minor | Moderate | Minor |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Minor | Moderate | Minor |  |
| Environmental | Moderate | Moderate | Moderate |  |
| Public safety, including life and property | Minor | Minor | Moderate |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Cost savings/cost reduction | Minor | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Increased revenues | Minor | Unable to provide |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $23,116 |  | Annual dollars saved/ realized | $185,259 |  | Annual dollars saved/ realized | $266 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Moderate | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved customer experience | Moderate | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $66,767 |  | Annual dollars saved/ realized | $111,979 |  | Annual dollars saved/ realized | $15 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Moderate |  |  | Moderate |  |  | Moderate |  |  |  |  |  |
| Environmental | Major |  |  | Moderate |  |  | Major |  |  |  |  |  |
| Public safety, including life and property | Moderate |  |  | Moderate |  |  | Moderate |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Agriculture and Precision Farming

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required |  |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Farm pond design. Irrigation system design. Detailed site analysis to support precision farming. Analysis of farm sedimentation and runoff. Calibration of fertilizer application, fertilizer management, and irrigation planning. Optimized terraforming. |
| MCA Title | Agriculture and Precision Farming |
| MCA ID | 60353 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 08 - Agriculture and Precision Farming |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Highly desirable |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Not required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Not required |
| Rivers and Streams |  |
| Less than 10 ft | Highly desirable |
| 10 - 50 ft | Highly desirable |
| 51 - 100 ft | Nice to have |
| 101 - 500 ft | Not required |
| 501 - 2,500 ft | Not required |
| Greater than 2,500 ft | Not required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Nice to have |
| ½ - 1 acre | Highly desirable |
| 1.1 – 2 acres | Highly desirable |
| 2.1 – 5 acres | Nice to have |
| 5.1 – 10 acres | Nice to have |
| Greater than 10 acres | Nice to have |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL2 | QL4B |  |  |
| Update Frequency | 2-3 years | 4-5 years |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | Up to 10 meters |  |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 50 cm |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Required |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable | Not required |  |  | Not required |  |
| Entire AOI under same environmental conditions | Highly desirable | Highly desirable |  |  | Nice to have |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable | Highly desirable |  |  | Not required |  |
| DEM for entire AOI needs to be seamless | Highly desirable | Highly desirable |  |  | Not required |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to double the required TVU at the 95% confidence level | Up to double the required TVU at the 95% confidence level |  |  | Up to double the required TVU at the 95% confidence level |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Highly desirable | Required |  |  |
| DTM | Required | Required |  |  |
| DEM | Required | Required |  |  |
| Raw point cloud data | Nice to have | Nice to have |  |  |
| Classified point cloud | Nice to have | Nice to have |  |  |
| Edited/cube XYZ |  | Highly desirable |  |  |
| Full waveform | Nice to have | Highly desirable |  |  |
| Bathymetric Attributed Grid (BAG) |  | Highly desirable |  |  |
| Breaklines required for standard hydro-flattening | Required | Nice to have |  |  |
| Additional breaklines for hydro-enforcement of culverts | Required |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Nice to have | Nice to have |  |  |
| Ground control/ground truthing | Highly desirable | Highly desirable |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Required | Required |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Nice to have | Highly desirable |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Highly desirable | Highly desirable |  |  |
| Land use/land cover | Highly desirable | Required |  |  |
| Wetlands | Required | Required |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Required | Required |  |  |
| Bridges/culverts | Required | Required |  |  |
| Landmark features | Nice to have | Required |  |  |
| Cultural resources | Nice to have | Highly desirable |  |  |
| Coastal and riverine structures | Highly desirable | Required |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Not required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map | Yes |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Moderate | None |  |  |
| Cost savings/cost reduction | Major | I don't know |  |  |
| Cost avoidance | Major | Moderate |  |  |
| Increased revenues | None | I don't know |  |  |
| Mission-driven performance improvements | Moderate | None |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | None |  |  |
| Improved response or timeliness | Moderate | None |  |  |
| Improved customer experience | Moderate | None |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Moderate | None |  |  |
| Environmental | Major | I don't know |  |  |
| Public safety, including life and property | Moderate | I don't know |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |  |  |  |
| Cost avoidance | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Increased revenues | Minor | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $112,356 |  | Annual dollars saved/ realized | $196,849 |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $89,072 |  | Annual dollars saved/ realized | $89,072 |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Major |  |  | Moderate |  |  |  |  |  |  |  |  |
| Environmental | Major |  |  | Major |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Minor |  |  | Major |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Fisheries Management and Aquaculture

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type |  | Required | Required | Required |
| Geographic Area Requirements |  | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) | Waters offshore off one or more states (including Great Lakes states), territories, or counties |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Management of fisheries. Sustainable aquaculture. |
| MCA Title | Fisheries Management and Aquaculture |
| MCA ID | 60354 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 09 - Fisheries Management and Aquaculture |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Nice to have |
| Tops of buildings, structures, objects | Not required |
| Tops of vegetation | Nice to have |
| Tops of submerged structures, objects | Highly desirable |
| Tops of submerged vegetation | Highly desirable |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Nice to have |
| Rivers and Streams |  |
| Less than 10 ft | Nice to have |
| 10 - 50 ft | Highly desirable |
| 51 - 100 ft | Highly desirable |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Highly desirable |
| Greater than 2,500 ft | Highly desirable |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Required |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order |  | QL0B | QL0B | Special Order |
| Update Frequency |  | 6-10 years | 6-10 years | 6-10 years |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error |  | Up to 2 meters | Up to 2 meters | Up to 2 meters |
| Acceptable Vertical Error |  | Up to 30 cm | Up to 30 cm | Less than 1 meter |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season |  | Highly desirable | Nice to have | Nice to have |  | Nice to have |
| Entire AOI under same environmental conditions |  | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless |  | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable |
| DEM for entire AOI needs to be seamless |  | Required | Highly desirable | Highly desirable |  | Highly desirable |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM |  | Highly desirable | Highly desirable | Required |
| DTM |  | Highly desirable | Highly desirable | Nice to have |
| DEM |  | Required | Required | Required |
| Raw point cloud data |  | Nice to have | Highly desirable | Nice to have |
| Classified point cloud |  | Nice to have | Highly desirable |  |
| Edited/cube XYZ |  | Nice to have | Nice to have | Nice to have |
| Full waveform |  | Nice to have | Nice to have | Nice to have |
| Bathymetric Attributed Grid (BAG) |  | Nice to have | Highly desirable | Nice to have |
| Breaklines required for standard hydro-flattening |  | Highly desirable |  |  |
| Additional breaklines for hydro-enforcement of culverts |  |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Highly desirable | Nice to have |
| Tide Predictions |  |  | Not required | Nice to have |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Nice to have | Nice to have |
| Intensity imagery/sidescan imagery |  | Nice to have | Highly desirable | Required |
| Ground control/ground truthing |  | Highly desirable | Required | Required |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Required | Required |
| Nautical and/or navigation charts |  |  | Nice to have | Required |
| Acoustic imagery of the seafloor |  |  | Highly desirable | Required |
| Aerial and/or satellite imagery |  | Highly desirable | Highly desirable | Required |
| Underwater videography |  |  | Nice to have | Highly desirable |
| Bottom texture |  |  | Nice to have | Required |
| Bottom type |  |  | Highly desirable | Required |
| Submerged features |  |  | Highly desirable | Required |
| Subbottom characteristics |  |  | Nice to have | Nice to have |
| Geologic and seismic data |  | Nice to have | Nice to have | Nice to have |
| Water column properties - Physical |  |  | Nice to have | Highly desirable |
| Water column properties - Chemical |  |  | Nice to have | Highly desirable |
| Water column properties - Biological |  |  | Nice to have | Highly desirable |
| Currents |  |  | Nice to have | Highly desirable |
| Tide/wave heights |  |  | Nice to have | Nice to have |
| Sea ice conditions |  |  | Not required | Not required |
| Habitat distribution and classification |  |  | Highly desirable | Nice to have |
| Boundaries |  |  | Nice to have | Nice to have |
| Routes |  |  | Nice to have | Nice to have |
| Offshore cadastral |  |  | Nice to have | Nice to have |
| Lease areas |  |  | Nice to have | Nice to have |
| Fixed obstructions |  |  | Nice to have | Required |
| Floating observation/navigation systems |  |  | Nice to have | Required |
| Shorelines – current, historic, change rates |  | Highly desirable | Highly desirable |  |
| Land use/land cover |  | Highly desirable | Nice to have | Nice to have |
| Wetlands |  | Highly desirable | Highly desirable | Highly desirable |
| Estuaries |  |  | Required | Highly desirable |
| Inland surface water features |  | Required | Nice to have |  |
| Bridges/culverts |  | Highly desirable |  |  |
| Landmark features |  | Nice to have | Nice to have |  |
| Cultural resources |  | Nice to have | Not required |  |
| Coastal and riverine structures |  | Highly desirable | Required |  |
| Overhead structures |  |  | Nice to have |  |
| Lowest Floor Elevation of Buildings |  |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  | Yes | Yes |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings |  | Major | Moderate | Moderate |
| Cost savings/cost reduction |  | Moderate | Major | Minor |
| Cost avoidance |  | Moderate | Minor | Minor |
| Increased revenues |  | Moderate | Minor | None |
| Mission-driven performance improvements |  | Major | Moderate | Minor |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services |  | Major | Minor | Moderate |
| Improved response or timeliness |  | Major | Moderate | Minor |
| Improved customer experience |  | Major | Minor | Minor |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach |  | Minor | Moderate | Moderate |
| Environmental |  | Moderate | Moderate | Minor |
| Public safety, including life and property |  | Minor | Minor | None |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  | Minor | Unable to provide |  |
| Cost savings/cost reduction |  |  |  | Major | Unable to provide |  | Moderate | Unable to provide |  | None |  |  |
| Cost avoidance |  |  |  | Major | Unable to provide |  | Moderate | Unable to provide |  | None |  |  |
| Increased revenues |  |  |  | None |  |  | Minor | Unable to provide |  | Minor | Unable to provide |  |
| Mission-driven performance improvements |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  | Minor | Unable to provide |  |
| Potential future annual operational benefits added during validation |  |  |  |  | Annual dollars saved/ realized | $194,176 |  | Annual dollars saved/ realized | $42,329 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services |  |  |  | Major | Unable to provide |  | Minor | Unable to provide |  | Minor | Unable to provide |  |
| Improved response or timeliness |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  | Minor | Unable to provide |  |
| Improved customer experience |  |  |  | Major | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |
| Potential future annual customer service benefits added during validation |  |  |  |  | Annual dollars saved/ realized | $88,377 |  | Annual dollars saved/ realized | $3,108 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach |  |  |  | Major |  |  | Moderate |  |  | Moderate |  |  |
| Environmental |  |  |  | Major |  |  | Major |  |  | Moderate |  |  |
| Public safety, including life and property |  |  |  | Moderate |  |  | Minor |  |  | Minor |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Geohazard Mitigation and Geologic Resource Identification and Mapping

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | Area split by varying quality level or update frequency | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements | Oregon |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Oregon Department of Geology and Mineral Industries operates the Oregon Lidar Consortium. The Oregon Lidar Consortium mission critical activity is to acquire, organize and distribute high resolution lidar and other remotely sensed imagery for the state. Geologic assessment and hazard mitigation, Geologic resource mining and extraction. Geologic mapping and resource identification. Bare earth and imagery together yield more accurate maps of bedrock and surficial geology. Elevation data are used to identify faults, tighten up GIS layers to follow contacts between formations, for geohazard mapping (e.g. landslide, liquefaction, floodplain delineation, channel migration zone mapping, etc.) Lidar are used to update the Mineral Information Layer (MILO) to show occurrences, prospects, and mine locations. The data are also used to locate historic mine sites, develop site descriptions, perform change detection and site characterization for active mines. |
| MCA Title | Geohazard Mitigation and Geologic Resource Identification and Mapping |
| MCA ID | 1184 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Dept. Geology and Mineral Industries |
| Sub-Agency or Division |  |
| Organization Mission | Oregon Department of Geology and Mineral Industries provides earth science information and regulation to make Oregon safe and prosperous. |
| Program Name | Geological Survey and Services Program |
| Total Annual Program Budget |  |
| Primary Business Use | BU 10 - Geologic Assessment and Hazard Mitigation |
| Secondary Business Use | BU 11 - Geologic Resource Mining and Extraction |
| Tertiary Business Use | BU 23 - Urban and Regional Planning |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Highly desirable |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Highly desirable |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Nice to have |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 50 sq mi - 999 sq mi (e.g. small county or County Equivalent, District of Columbia, etc.) |
| Smallest 3D features needed | Large features |
| Description of smallest 3D features | Rocks, geologic contacts, 1-meter size objects |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Required |
| Rivers and Streams |  |
| Less than 10 ft | Nice to have |
| 10 - 50 ft | Highly desirable |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Nice to have |
| ½ - 1 acre | Highly desirable |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | (a) QL0 (b) QL2 | QL1B | QL2B |  |
| Update Frequency | (a) 4-5 years (b) 6-10 years | 6-10 years | 6-10 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI | Lower QL and update frequency for large portions of Eastern Oregon Southeastern OR quadrant could use QL2 at 6-10 years |  |  |  |
| Acceptable Horizontal Error | Less than 20 cm | Less than 50 cm | Up to 1 meter |  |
| Acceptable Vertical Error | Up to 10 cm | Up to 30 cm | Up to 40 cm |  |
| How far onshore needed |  |  | To the fall line |  |
| How far down the beach profile needed | To MHW |  | To MHW |  |
| Tide correction requirement |  |  | MSL |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Highly desirable |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Required | Required | Required |  | Highly desirable | Highly desirable |
| Entire AOI under same environmental conditions | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required | Required | Required |  | Required | Required |
| DEM for entire AOI needs to be seamless | Required | Required | Required |  | Required | Required |
| Other | Required |  |  |  |  |  |
| Other description | Digital Surface Model |  |  |  |  |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required | Required | Required |  |
| DTM | Required | Required | Highly desirable |  |
| DEM | Required | Required | Required |  |
| Raw point cloud data | Not required | Not required | Not required |  |
| Classified point cloud | Required | Required | Nice to have |  |
| Edited/cube XYZ |  | Nice to have | Nice to have |  |
| Full waveform | Nice to have | Nice to have | Nice to have |  |
| Bathymetric Attributed Grid (BAG) |  | Highly desirable | Required |  |
| Breaklines required for standard hydro-flattening | Highly desirable | Highly desirable |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Required |  |
| Tide Predictions |  |  | Highly desirable |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Highly desirable |  |
| Intensity imagery/sidescan imagery | Required | Highly desirable | Highly desirable |  |
| Ground control/ground truthing | Required | Required | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Required |  |
| Nautical and/or navigation charts |  |  | Nice to have |  |
| Acoustic imagery of the seafloor |  |  | Nice to have |  |
| Aerial and/or satellite imagery | Highly desirable | Highly desirable | Not required |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Nice to have |  |
| Bottom type |  |  | Highly desirable |  |
| Submerged features |  |  | Nice to have |  |
| Subbottom characteristics |  |  | Nice to have |  |
| Geologic and seismic data | Required | Required | Highly desirable |  |
| Water column properties - Physical |  |  | Not required |  |
| Water column properties - Chemical |  |  | Not required |  |
| Water column properties - Biological |  |  | Not required |  |
| Currents |  |  | Nice to have |  |
| Tide/wave heights |  |  | Highly desirable |  |
| Sea ice conditions |  |  | Nice to have |  |
| Habitat distribution and classification |  |  | Not required |  |
| Boundaries |  |  | Highly desirable |  |
| Routes |  |  | Not required |  |
| Offshore cadastral |  |  | Nice to have |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Not required |  |
| Floating observation/navigation systems |  |  | Nice to have |  |
| Shorelines – current, historic, change rates | Required | Required | Highly desirable |  |
| Land use/land cover | Highly desirable | Required | Not required |  |
| Wetlands | Highly desirable | Highly desirable | Not required |  |
| Estuaries |  |  | Not required |  |
| Inland surface water features | Required | Required |  |  |
| Bridges/culverts | Highly desirable | Highly desirable |  |  |
| Landmark features | Highly desirable | Nice to have | Highly desirable |  |
| Cultural resources | Highly desirable | Nice to have | Nice to have |  |
| Coastal and riverine structures | Required | Highly desirable | Highly desirable |  |
| Overhead structures |  |  | Highly desirable |  |
| Lowest Floor Elevation of Buildings | Required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Currently we acquire lidar through the Oregon Lidar Consortium. The Oregon Lidar Consortium collects data at QL 0. The data is collected piecemeal throughout Oregon which is due to lack of dedicated funding source for lidar. | Bathymetry data is collected by DOGAMI through the Oregon Lidar Consortium. This data is typically paid for by outside funds that tend to be federal in nature. Bathymetry data is also provided to DOGAMI from other agencies that collected bathymetry data in Oregon. The data is normally QL2B. | NOAA and USACE JALBTCX |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  | Yes |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  | Yes |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories | Yes | Yes |  |  |
| State repositories used | Oregon Lidar Consortium - http://www.oregongeology.org/lidar/index.htm | Oregon Lidar Consortium |  |  |
| Other |  |  | Yes |  |
| Other description |  |  | USACE |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Moderate | Major |  |
| Cost savings/cost reduction | Major | Moderate | Major |  |
| Cost avoidance | Major | Moderate | Major |  |
| Increased revenues | Moderate | Minor | None |  |
| Mission-driven performance improvements | Moderate | Minor | Major |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate | Minor | Major |  |
| Improved response or timeliness | Major | Minor | Major |  |
| Improved customer experience | Moderate | Minor | Major |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Moderate | Minor | Major |  |
| Environmental | Major | Major | Minor |  |
| Public safety, including life and property | Major | Major | Major |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Annual dollars saved/realized | $19,296 | Major | Annual dollars saved/realized | $19,296 | Major | Annual dollars saved/realized | $12,663 |  |  |  |
| Time savings description | Less time needed in field to walk transects and map faults and landslide areas. Better, more reliable modeling at a state level instead of sub-county level. Better quality data to use for assessing mining permits and mining regulation. Less office time tweaking lidar data for analysis. |  |  | Recollection of all waterbodies with the same projection/datum would greatly improve analysis work. Improved modeling and analysis with full bathymetric coverage of water bodies. Coastal community benefits would be enormous. Minimal improvement. |  |  | Less field surveying. |  |  |  |  |  |
| Cost savings/cost reduction | Major | Annual dollars saved/realized | $150,000 | Major | Annual dollars saved/realized | $1,500,000 | Moderate | Annual dollars saved/realized | $100,000 |  |  |  |
| Cost savings/cost reduction description | Funding lidar collections and building consortium projects. Less time in the field for surveying. |  |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance | Major | Annual dollars saved/realized | $10,125,000 | Moderate | Annual dollars saved/realized | $50,000 | Major | Unable to provide |  |  |  |  |
| Cost avoidance description | Less time in field for quality control surveying. Lidar based landslide mapping will assist in reducing potential loss of life and property. |  |  | Potential for reduced loss from flooding but cannot provide an annual estimate for the reduction of a rare event. This would be minimal. NHD updates and fixes would be reduced with this dataset. |  |  |  |  |  |  |  |  |
| Increased revenues | Major | Annual dollars saved/realized | $200,000 | None |  |  | None |  |  |  |  |  |
| Increased revenues description | Additional lidar coverage will allow us to conduct more hazard mapping and analysis. |  |  |  |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Major | Annual percent improvement | 20% | Moderate | Annual percent improvement | 8% | Major | Annual percent improvement | 14% |  |  |  |
| Mission-driven performance improvements description | Hazard mitigation would greatly benefit from additional lidar coverage in Oregon. Standardized products, no need to adjust between datums or projections. |  |  |  |  |  | More data to feed into already produced modeling. Better model will provide better information to coastal communities who make land use decisions. Expanded coverage of hazard mapping to all coastal communities. |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Annual dollars saved/realized | $6,030 | Major | Annual dollars saved/realized | $1,000,000 | Major | Annual dollars saved/realized | $100,000 |  |  |  |
| Value added to products or services description | Basically can produce more products and services where Oregon currently doesn't have lidar coverage. These products would only have minimal benefit to our agency needs. |  |  | Increased efficiency in flood hazard mapping, flood warnings and improved accuracy of levy construction. |  |  |  |  |  |  |  |  |
| Improved response or timeliness | Major | Annual dollars saved/realized | $206,030 | Major | Annual dollars saved/realized | $106,030 | Major | Unable to provide |  |  |  |  |
| Improved response or timeliness description | See above. Improved mapping for fire response and any other natural disaster. New data will improve resiliency and mitigation of natural disasters as well. Oregon would make data available to public to download, allowing all users to benefit from lightly accurate elevation information covering Oregon. |  |  | Better modeling and consistency would improve current modeling and allow for more modeling coverage in the state. Cheaper to produce from lidar then aerial imagery or by other means. |  |  |  |  |  |  |  |  |
| Improved customer experience | Moderate | Annual dollars saved/realized | $6,512 | Minor | Annual dollars saved/realized | $2,412 | Moderate | Unable to provide |  |  |  |  |
| Improved customer experience description | Less time working on developing lidar collections and more time creating ways to make the lidar data more useful. |  |  | Easier to distribute to other state agencies and the public. |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Moderate |  |  | Minor |  |  | Major |  |  |  |  |  |
| Education or outreach description | Provide new images and maps showing lidar usefulness and importance in keeping Oregonians safe. STEM marketing. |  |  | Minimal impact to improving education and outreach |  |  |  |  |  |  |  |  |
| Environmental | Major |  |  | Moderate |  |  | Moderate |  |  |  |  |  |
| Environmental description | Hazard mitigation, beach and costal community health, forest fire prevention |  |  | Reduced water waste and improve understanding of fluvial geomorphology |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Major |  |  | Major |  |  | Major |  |  |  |  |  |
| Public safety, including life and property description | Improved tsunami modeling along coast and improved earthquake vulnerability assessment for all Oregonian communities. |  |  | Improved flood studies will greatly improve public safety |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours | Yes | Yes | Yes |  |
| Hillshades | Yes | Yes | Yes |  |
| Slope maps | Yes | Yes | Yes |  |
| Aspect maps | Yes |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps | Yes |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  | Yes | Yes |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  | Yes | Yes |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  | Yes | Yes |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 2 |
| Vertical accuracy | 3 |
| Update frequency | 1 |

# MCA Title: State Infrastructure Resiliency

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  | Required | Required |
| Geographic Area Requirements | One or more states, territories, or counties |  | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) | Custom description |
| Sub Area Requirements |  |  |  | Out to subduction zone (75-100 mi offshore) |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Increasing Oregon's level of preparedness and improving the resilience of Oregon's built infrastructure, bridges, highways, and slopes so that we make the state more resilient and ready to recover for the expected 9.0 Cascadia earthquake and ensuing tsunami. |
| MCA Title | State Infrastructure Resiliency |
| MCA ID | 1246 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Governor's Office, State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission | To protect, safeguard and enhance the lives of all Oregonians. |
| Program Name | Office of the State Resilience Officer |
| Total Annual Program Budget |  |
| Primary Business Use | BU 10 - Geologic Assessment and Hazard Mitigation |
| Secondary Business Use | BU 24 - Health and Human Services |
| Tertiary Business Use | BU 18 - Homeland Security, Law Enforcement, Disaster Response, and Emergency Management |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Highly desirable |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Nice to have |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Highly desirable |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 75,000 sq mi - 199,999 sq mi (e.g. large state or medium multi-state region) |
| Smallest 3D features needed | Large features |
| Description of smallest 3D features | Infrastructure is the main focus. Fiber optic cables on ocean floor. |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL2 |  | QL4B | Order 1 |
| Update Frequency | 4-5 years |  | 4-5 years | 4-5 years |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter |  | Up to 5 meters | Up to 5 meters |
| Acceptable Vertical Error | Up to 20 cm |  | Up to 1 meter | Up to 1 meter |
| How far onshore needed |  |  | >1 kilometer inland |  |
| How far down the beach profile needed | To MHHW |  | To MHHW |  |
| Tide correction requirement |  |  | I don't know | I don't know |
| Cross sections and/or transects meet needs |  |  | Partial |  |
| Cross section/transect requirement |  |  | Unsure, this is not my subject matter |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Required |
| Hydro-conditioning | Required |
| No Treatment | Nice to have |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable |  | Required | Required |  | Required |
| Entire AOI under same environmental conditions | Required |  | Highly desirable | Highly desirable |  | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required |  | Highly desirable | Highly desirable |  | Highly desirable |
| DEM for entire AOI needs to be seamless | Required |  | Required | Required |  | Required |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | I don't know |  | I don't know | I don't know |  | I don't know |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required |  | Required | Required |
| DTM | Required |  | Required | Required |
| DEM | Required |  | Required | Required |
| Raw point cloud data | Highly desirable |  | Highly desirable | Highly desirable |
| Classified point cloud | Highly desirable |  | Highly desirable |  |
| Edited/cube XYZ |  |  | Highly desirable | Highly desirable |
| Full waveform | Nice to have |  | Highly desirable | Highly desirable |
| Bathymetric Attributed Grid (BAG) |  |  | Highly desirable | Highly desirable |
| Breaklines required for standard hydro-flattening | Nice to have |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Highly desirable | Highly desirable |
| Tide Predictions |  |  | Required | Required |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Highly desirable | Highly desirable |
| Intensity imagery/sidescan imagery | Nice to have |  | Required | Required |
| Ground control/ground truthing | Highly desirable |  | Highly desirable | Highly desirable |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Required | Required |
| Nautical and/or navigation charts |  |  | Highly desirable | Highly desirable |
| Acoustic imagery of the seafloor |  |  | Highly desirable | Highly desirable |
| Aerial and/or satellite imagery | Required |  | Required | Required |
| Underwater videography |  |  | Required | Required |
| Bottom texture |  |  | Highly desirable | Highly desirable |
| Bottom type |  |  | Highly desirable | Highly desirable |
| Submerged features |  |  | Nice to have | Nice to have |
| Subbottom characteristics |  |  | Nice to have | Nice to have |
| Geologic and seismic data | Required |  | Required | Required |
| Water column properties - Physical |  |  | Highly desirable | Highly desirable |
| Water column properties - Chemical |  |  | Highly desirable | Highly desirable |
| Water column properties - Biological |  |  | Highly desirable | Highly desirable |
| Currents |  |  | Highly desirable | Highly desirable |
| Tide/wave heights |  |  | Required | Required |
| Sea ice conditions |  |  | Nice to have | Nice to have |
| Habitat distribution and classification |  |  | Highly desirable | Highly desirable |
| Boundaries |  |  | Required | Required |
| Routes |  |  | Highly desirable | Highly desirable |
| Offshore cadastral |  |  | Required | Required |
| Lease areas |  |  | Required | Required |
| Fixed obstructions |  |  | Highly desirable | Highly desirable |
| Floating observation/navigation systems |  |  | Highly desirable | Highly desirable |
| Shorelines – current, historic, change rates | Required |  | Required |  |
| Land use/land cover | Required |  | Highly desirable | Highly desirable |
| Wetlands | Required |  | Required | Required |
| Estuaries |  |  | Required | Required |
| Inland surface water features | Required |  | Required |  |
| Bridges/culverts | Highly desirable |  |  |  |
| Landmark features | Highly desirable |  | Highly desirable |  |
| Cultural resources | Highly desirable |  | Required |  |
| Coastal and riverine structures | Required |  | Required |  |
| Overhead structures |  |  | Highly desirable |  |
| Lowest Floor Elevation of Buildings | Highly desirable |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Unknown, I do not have any data on this |  | Not able to respond, not my subject matter. Best available from NOAA | Best available from NOAA |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  | Yes | Yes |
| NCEI | Yes |  | Yes | Yes |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  | Yes | Yes |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories | Yes |  |  |  |
| State repositories used | Oregon - DOGAMI |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major |  | Moderate | Moderate |
| Cost savings/cost reduction | Major |  | Moderate | Moderate |
| Cost avoidance | Moderate |  | Minor | Minor |
| Increased revenues | I don't know |  | I don't know | I don't know |
| Mission-driven performance improvements | Moderate |  | Major | Major |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate |  | Moderate | Moderate |
| Improved response or timeliness | Major |  | Major | Major |
| Improved customer experience | Moderate |  | Moderate | Moderate |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major |  | Major | Major |
| Environmental | Major |  | Major | Major |
| Public safety, including life and property | Major |  | Major | Major |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | I don't know | Unable to provide |  |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  |
| Cost savings/cost reduction | Major | Unable to provide |  |  |  |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Cost avoidance | Major | Unable to provide |  |  |  |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Increased revenues | Moderate | Unable to provide |  |  |  |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Mission-driven performance improvements | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  |
| Mission-driven performance improvements description | Better data means better hazard mitigation plans and community engagement. |  |  |  |  |  | Better data means better hazard mitigation plans and community engagement. |  |  | Better data means better hazard mitigation plans and community engagement. |  |  |
| Other operational benefits | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $10,242,526 |  |  |  |  | Annual dollars saved/ realized | $10,390 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  |
| Improved response or timeliness | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  |
| Improved response or timeliness description | Better preparation for natural disasters such as earthquake or tsunami. |  |  |  |  |  | Better preparation for natural disasters such as earthquake or tsunami. |  |  | Better preparation for natural disasters such as earthquake or tsunami. |  |  |
| Improved customer experience | Major | Unable to provide |  |  |  |  | Major | Unable to provide |  | Major | Unable to provide |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $179,807 |  |  |  |  | Annual dollars saved/ realized | $16,718 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Major |  |  |  |  |  | Major |  |  | Major |  |  |
| Education or outreach description | Better data tells a better story, encourages action |  |  |  |  |  | Better data tells a better story, encourages action |  |  | Better data tells a better story, encourages action |  |  |
| Environmental | Major |  |  |  |  |  | Major |  |  | Major |  |  |
| Public safety, including life and property | Major |  |  |  |  |  | Major |  |  | Major |  |  |
| Public safety, including life and property description | Better preparation for natural disasters such as earthquake or tsunami |  |  |  |  |  | Better preparation for natural disasters such as earthquake or tsunami |  |  | Better preparation for natural disasters such as earthquake or tsunami |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours | Yes |  |  |  |
| Hillshades | Yes |  |  |  |
| Slope maps | Yes |  |  |  |
| Aspect maps | Yes |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  | Yes | Yes |
| Height-Above-Ground maps |  |  | Yes | Yes |
| Viewshed maps | Yes |  |  |  |
| Hydrologic Flow Direction Grids |  |  | Yes | Yes |
| Hydrologic Flow Accumulation Grids |  |  | Yes | Yes |
| Hydrologic networks (e.g. streams, lakes) |  |  | Yes | Yes |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  | Yes | Yes |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness | Yes |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 3 |
| Vertical accuracy | 1 |
| Update frequency | 2 |

# MCA Title: Geologic Resource Mining and Extraction

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  |  |  |
| Geographic Area Requirements | One or more states, territories, or counties |  |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Onshore or offshore mineral extraction. Monitoring sand as a local resource. Seabed resources. Open mine volume computations. Stockpile analysis. Environmental impact assessment and site restoration. |
| MCA Title | Geologic Resource Mining and Extraction |
| MCA ID | 60355 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 11 - Geologic Resource Mining and Extraction |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Nice to have |
| Tops of vegetation | Highly desirable |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Nice to have |
| Nearshore elevation (<10 m deep) | Not required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 HD |  |  |  |
| Update Frequency | 2-3 years |  |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter |  |  |  |
| Acceptable Vertical Error | Up to 20 cm |  |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have |  |  |  |  |  |
| Entire AOI under same environmental conditions | Highly desirable |  |  |  |  |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable |  |  |  |  |  |
| DEM for entire AOI needs to be seamless | Highly desirable |  |  |  |  |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  |  |  |  |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Nice to have |  |  |  |
| DTM | Required |  |  |  |
| DEM | Required |  |  |  |
| Raw point cloud data | Nice to have |  |  |  |
| Classified point cloud | Highly desirable |  |  |  |
| Edited/cube XYZ |  |  |  |  |
| Full waveform | Not required |  |  |  |
| Bathymetric Attributed Grid (BAG) |  |  |  |  |
| Breaklines required for standard hydro-flattening | Highly desirable |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Nice to have |  |  |  |
| Ground control/ground truthing | Nice to have |  |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Required |  |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Nice to have |  |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Not required |  |  |  |
| Land use/land cover | Nice to have |  |  |  |
| Wetlands | Nice to have |  |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Highly desirable |  |  |  |
| Bridges/culverts | Nice to have |  |  |  |
| Landmark features | Nice to have |  |  |  |
| Cultural resources | Nice to have |  |  |  |
| Coastal and riverine structures | Nice to have |  |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Not required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map | Yes |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major |  |  |  |
| Cost savings/cost reduction | Moderate |  |  |  |
| Cost avoidance | Moderate |  |  |  |
| Increased revenues | None |  |  |  |
| Mission-driven performance improvements | Moderate |  |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate |  |  |  |
| Improved response or timeliness | Moderate |  |  |  |
| Improved customer experience | Moderate |  |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Moderate |  |  |  |
| Environmental | Major |  |  |  |
| Public safety, including life and property | Moderate |  |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Minor | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance | Minor | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Increased revenues | None |  |  |  |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $23,705 |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Minor | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Improved customer experience | Moderate | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $15,724 |  |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Minor |  |  |  |  |  |  |  |  |  |  |  |
| Environmental | Major |  |  |  |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Major |  |  |  |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Renewable Energy Resources

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required | Required |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) | Waters offshore off one or more states (including Great Lakes states), territories, or counties |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Alternate energy development – solar, tidal, wind, wave, and ocean current. Assessment of rooftops for solar energy potential. Analysis of wind energy potential and turbine placement. Low head power potential for hydropower. |
| MCA Title | Renewable Energy Resources |
| MCA ID | 60356 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 12 - Renewable Energy Resources |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Highly desirable |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Highly desirable |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | I don't know |
| Rivers and Streams |  |
| Less than 10 ft | I don't know |
| 10 - 50 ft | I don't know |
| 51 - 100 ft | I don't know |
| 101 - 500 ft | I don't know |
| 501 - 2,500 ft | I don't know |
| Greater than 2,500 ft | I don't know |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | I don't know |
| ½ - 1 acre | I don't know |
| 1.1 – 2 acres | I don't know |
| 2.1 – 5 acres | I don't know |
| 5.1 – 10 acres | I don't know |
| Greater than 10 acres | I don't know |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL2 | I don't know | I don't know | I don't know |
| Update Frequency | 6-10 years | I don't know | I don't know | I don't know |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | I don't know | I don't know | I don't know |
| Acceptable Vertical Error | Up to 20 cm | I don't know | I don't know | I don't know |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Nice to have |
| Hydro-enforcement | Nice to have |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | I don't know |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable | I don't know | I don't know | I don't know | I don't know | I don't know |
| Entire AOI under same environmental conditions | Highly desirable | I don't know | I don't know | I don't know | I don't know | I don't know |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable | I don't know | I don't know | I don't know | I don't know | I don't know |
| DEM for entire AOI needs to be seamless | Highly desirable | I don't know | I don't know | I don't know | I don't know | I don't know |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | I don't know | I don't know | I don't know | I don't know | I don't know |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required | I don't know | I don't know | I don't know |
| DTM | Required | I don't know | I don't know | I don't know |
| DEM | Required | I don't know | I don't know | I don't know |
| Raw point cloud data | Highly desirable | I don't know | I don't know | I don't know |
| Classified point cloud | Highly desirable | I don't know | I don't know |  |
| Edited/cube XYZ |  | I don't know | I don't know | I don't know |
| Full waveform | Nice to have | I don't know | I don't know | I don't know |
| Bathymetric Attributed Grid (BAG) |  | I don't know | I don't know | I don't know |
| Breaklines required for standard hydro-flattening | Nice to have | I don't know |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | I don't know | I don't know |
| Tide Predictions |  |  | I don't know | I don't know |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | I don't know | I don't know |
| Intensity imagery/sidescan imagery | Not required | I don't know | I don't know | I don't know |
| Ground control/ground truthing | Highly desirable | I don't know | I don't know | I don't know |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | I don't know | I don't know |
| Nautical and/or navigation charts |  |  | I don't know | I don't know |
| Acoustic imagery of the seafloor |  |  | I don't know | I don't know |
| Aerial and/or satellite imagery | Highly desirable | I don't know | I don't know | I don't know |
| Underwater videography |  |  | I don't know | I don't know |
| Bottom texture |  |  | I don't know | I don't know |
| Bottom type |  |  | I don't know | I don't know |
| Submerged features |  |  | I don't know | I don't know |
| Subbottom characteristics |  |  | I don't know | I don't know |
| Geologic and seismic data | Not required | I don't know | I don't know | I don't know |
| Water column properties - Physical |  |  | I don't know | I don't know |
| Water column properties - Chemical |  |  | I don't know | I don't know |
| Water column properties - Biological |  |  | I don't know | I don't know |
| Currents |  |  | I don't know | I don't know |
| Tide/wave heights |  |  | I don't know | I don't know |
| Sea ice conditions |  |  | I don't know | I don't know |
| Habitat distribution and classification |  |  | I don't know | I don't know |
| Boundaries |  |  | I don't know | I don't know |
| Routes |  |  | I don't know | I don't know |
| Offshore cadastral |  |  | I don't know | I don't know |
| Lease areas |  |  | I don't know | I don't know |
| Fixed obstructions |  |  | I don't know | I don't know |
| Floating observation/navigation systems |  |  | I don't know | I don't know |
| Shorelines – current, historic, change rates | Nice to have | I don't know | I don't know |  |
| Land use/land cover | Highly desirable | I don't know | I don't know | I don't know |
| Wetlands | Nice to have | I don't know | I don't know | I don't know |
| Estuaries |  |  | I don't know | I don't know |
| Inland surface water features | Nice to have | I don't know | I don't know |  |
| Bridges/culverts | Nice to have | I don't know |  |  |
| Landmark features | Nice to have | I don't know | I don't know |  |
| Cultural resources | Not required | I don't know | I don't know |  |
| Coastal and riverine structures | Nice to have | I don't know | I don't know |  |
| Overhead structures |  |  | I don't know |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Moderate | I don't know | I don't know | I don't know |
| Cost savings/cost reduction | Moderate | I don't know | I don't know | I don't know |
| Cost avoidance | Moderate | I don't know | I don't know | I don't know |
| Increased revenues | None | I don't know | I don't know | I don't know |
| Mission-driven performance improvements | Minor | I don't know | I don't know | I don't know |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Minor | I don't know | I don't know | I don't know |
| Improved response or timeliness | Minor | I don't know | I don't know | I don't know |
| Improved customer experience | Minor | I don't know | I don't know | I don't know |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Minor | I don't know | I don't know | I don't know |
| Environmental | Minor | I don't know | I don't know | I don't know |
| Public safety, including life and property | Minor | I don't know | I don't know | I don't know |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Cost savings/cost reduction | Moderate | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Cost avoidance | Major | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Increased revenues | Moderate | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Mission-driven performance improvements | Major | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Improved response or timeliness | Moderate | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Improved customer experience | Minor | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  | I don't know | Unable to provide |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Minor |  |  | I don't know |  |  | I don't know |  |  | I don't know |  |  |
| Environmental | Minor |  |  | I don't know |  |  | I don't know |  |  | I don't know |  |  |
| Public safety, including life and property | None |  |  | I don't know |  |  | I don't know |  |  | I don't know |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Statewide Cultural Resource Protection and Preservation

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required | Required |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) | Waters offshore off one or more states (including Great Lakes states), territories, or counties |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Statewide cultural resource protection and preservation. Elevation data are used by archaeologists for review of projects done by others and for conducting in-house projects. |
| MCA Title | Statewide Cultural Resource Protection and Preservation |
| MCA ID | 21672 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Parks and Recreation Department |
| Sub-Agency or Division |  |
| Organization Mission | The mission of the Oregon Parks and Recreation Department is to provide and protect outstanding natural, scenic, cultural, historic and recreational sites for the enjoyment and education of present and future generations |
| Program Name | State Historic Preservation Office |
| Total Annual Program Budget |  |
| Primary Business Use | BU 14 - Cultural Resources Preservation and Management |
| Secondary Business Use | BU 27 - Recreation |
| Tertiary Business Use | BU 26 - Education K12 and Beyond, Basic Research |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Not required |
| Tops of submerged structures, objects | Required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Not required |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Highly desirable |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 75,000 sq mi - 199,999 sq mi (e.g. large state or medium multi-state region) |
| Smallest 3D features needed | Survey-level features |
| Description of smallest 3D features | Site level cultural features <100cm |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Required |
| Rivers and Streams |  |
| Less than 10 ft | Highly desirable |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Required |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 | QL0B | QL3B | Order 1a |
| Update Frequency | 4-5 years | 6-10 years | 6-10 years | 6-10 years |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Less than 20 cm | Up to 2 meters | The best horizontal accuracy achievable for the vertical accuracy I need | Up to 5 meters |
| Acceptable Vertical Error | Up to 10 cm | Up to 30 cm | Up to 40 cm | Less than 1 meter |
| How far onshore needed |  |  | To cover the beach slope |  |
| How far down the beach profile needed | To MHW |  | To MHHW |  |
| Tide correction requirement |  |  | No requirement for tide correction |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Not required |
| Hydro-enforcement | Not required |
| Hydro-conditioning | Not required |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Not required | Nice to have | Not required | Nice to have | Nice to have | Nice to have |
| Entire AOI under same environmental conditions | Not required | Nice to have | Not required | Nice to have | Nice to have | Nice to have |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Nice to have | Not required | Not required | Nice to have | Nice to have | Nice to have |
| DEM for entire AOI needs to be seamless | Nice to have | Nice to have | Not required | Nice to have | Nice to have | Nice to have |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | I don't know | Up to triple the required TVU at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to double the required TVU at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required | Required | Required | Required |
| DTM | Not required | Highly desirable | Nice to have | Highly desirable |
| DEM | Required | Required | Required | Required |
| Raw point cloud data | Not required | Not required | Not required | Not required |
| Classified point cloud | Required | Nice to have | Required |  |
| Edited/cube XYZ |  | Nice to have | Not required | Nice to have |
| Full waveform | Not required | Not required | Not required | Not required |
| Bathymetric Attributed Grid (BAG) |  | Nice to have | Not required | Highly desirable |
| Breaklines required for standard hydro-flattening | Not required | Not required |  |  |
| Additional breaklines for hydro-enforcement of culverts | Not required |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Not required | Nice to have |
| Tide Predictions |  |  | Not required | Highly desirable |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Not required | Highly desirable |
| Intensity imagery/sidescan imagery | Required | Highly desirable | Required | Highly desirable |
| Ground control/ground truthing | Highly desirable | Highly desirable | Highly desirable | Highly desirable |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Not required | Required |
| Nautical and/or navigation charts |  |  | Not required | Required |
| Acoustic imagery of the seafloor |  |  | Not required | Required |
| Aerial and/or satellite imagery | Highly desirable | Highly desirable | Highly desirable | Nice to have |
| Underwater videography |  |  | Not required | Not required |
| Bottom texture |  |  | Not required | Highly desirable |
| Bottom type |  |  | Not required | Highly desirable |
| Submerged features |  |  | Required | Required |
| Subbottom characteristics |  |  | Not required | Nice to have |
| Geologic and seismic data | Not required | Highly desirable | Not required | Highly desirable |
| Water column properties - Physical |  |  | Not required | Highly desirable |
| Water column properties - Chemical |  |  | Not required | Not required |
| Water column properties - Biological |  |  | Not required | Not required |
| Currents |  |  | Not required | Nice to have |
| Tide/wave heights |  |  | Not required | Nice to have |
| Sea ice conditions |  |  | Not required | Not required |
| Habitat distribution and classification |  |  | Not required | Nice to have |
| Boundaries |  |  | Nice to have | Highly desirable |
| Routes |  |  | Nice to have | Nice to have |
| Offshore cadastral |  |  | Not required | Nice to have |
| Lease areas |  |  | Nice to have | Nice to have |
| Fixed obstructions |  |  | Nice to have | Required |
| Floating observation/navigation systems |  |  | Not required | Required |
| Shorelines – current, historic, change rates | Nice to have | Highly desirable | Highly desirable |  |
| Land use/land cover | Not required | Highly desirable | Not required | Highly desirable |
| Wetlands | Not required | Highly desirable | Not required | Required |
| Estuaries |  |  | Not required | Highly desirable |
| Inland surface water features | Nice to have | Highly desirable | Nice to have |  |
| Bridges/culverts | Nice to have | Highly desirable |  |  |
| Landmark features | Nice to have | Highly desirable | Nice to have |  |
| Cultural resources | Nice to have | Highly desirable | Required |  |
| Coastal and riverine structures | Nice to have | Highly desirable | Nice to have |  |
| Overhead structures |  |  | Not required |  |
| Lowest Floor Elevation of Buildings | Not required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Cooperatively acquired QL1 data, various dates. |  | Oregon State University, ESRI |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast | Yes |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Other | Yes |  | Yes |  |
| Other description | Cooperative contracts |  | Oregon State University, ESRI |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | I don't know | Major | I don't know | Moderate |
| Cost savings/cost reduction | None | Minor | I don't know | Minor |
| Cost avoidance | I don't know | Minor | I don't know | Minor |
| Increased revenues | None | Minor | None | None |
| Mission-driven performance improvements | Moderate | Moderate | Moderate | Minor |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | None | Minor | None | Minor |
| Improved response or timeliness | I don't know | Major | I don't know | Moderate |
| Improved customer experience | I don't know | Moderate | I don't know | Moderate |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Minor | Minor | Minor | Minor |
| Environmental | I don't know | Moderate | I don't know | Moderate |
| Public safety, including life and property | I don't know | Moderate | I don't know | Minor |
| Other Current Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Other | Moderate |  |  |  |
| Other description | Cultural resource protection/preservation |  |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Moderate | Unable to provide |  | Minor | Unable to provide |  | I don't know | Unable to provide |  | Moderate | Unable to provide |  |
| Time savings description | Fairly significant benefits can be realized by modeling or analyzing data with higher accuracy and resolution. |  |  |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction | None |  |  | Moderate | Unable to provide |  | I don't know | Unable to provide |  | Minor | Unable to provide |  |
| Cost avoidance | Moderate | Unable to provide |  | Moderate | Unable to provide |  | I don't know | Unable to provide |  | Major | Unable to provide |  |
| Cost avoidance description | Improved data can reduce costs associated with resource conflicts, difficult to quantify. |  |  |  |  |  |  |  |  |  |  |  |
| Increased revenues | None |  |  | None |  |  | I don't know | Unable to provide |  | None |  |  |
| Mission-driven performance improvements | Moderate | Unable to provide |  | Minor | Unable to provide |  | I don't know | Unable to provide |  | Major | Unable to provide |  |
| Mission-driven performance improvements description | Improvement in ability to inventory and assess cultural resources. |  |  |  |  |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $17,787 |  | Annual dollars saved/ realized | $42,655 |  | Annual dollars saved/ realized | $771 |  | Annual dollars saved/ realized | $2,529 |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Moderate | Unable to provide |  | Major | Unable to provide |  | I don't know | Unable to provide |  | Minor | Unable to provide |  |
| Improved response or timeliness | Moderate | Unable to provide |  | Minor | Unable to provide |  | I don't know | Unable to provide |  | Moderate | Unable to provide |  |
| Improved customer experience | None |  |  | Minor | Unable to provide |  | I don't know | Unable to provide |  | Minor | Unable to provide |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $15,551 |  |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Minor |  |  | Moderate |  |  | I don't know |  |  | Minor |  |  |
| Environmental | None |  |  | Minor |  |  | I don't know |  |  | Moderate |  |  |
| Public safety, including life and property | None |  |  | Minor |  |  | I don't know |  |  | Minor |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades | Yes |  | Yes |  |
| Slope maps | Yes |  | Yes |  |
| Aspect maps | Yes |  | Yes |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 1 |
| Vertical accuracy | 2 |
| Update frequency | 3 |

# MCA Title: Dam Safety Analysis, Flood Risk Modeling, and Mapping

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required |  |  |
| Geographic Area Requirements | One or more Hydrologic Units (HUC8s) | One or more Hydrologic Units (HUC8s) |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Dam safety analysis, flood risk modeling and mapping for dam safety (not FIRMs). Identify structures at risk and save lives. Elevation data are used for H&H modeling. Hydroelectric dam licensing is covered by another group. |
| MCA Title | Dam Safety Analysis, Flood Risk Modeling, and Mapping |
| MCA ID | 1306 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Water Resources Dept. |
| Sub-Agency or Division |  |
| Organization Mission | OWRD 's mission is to serve the public by practicing and promoting responsible water management through two key goals: 1) to directly address Oregon's water supply needs; and, 2) to restore and protect streamflows and watersheds in order to ensure the long-term sustainability of Oregon's ecosystems, economy, and quality of life. |
| Program Name | Oregon Dam Safety Program |
| Total Annual Program Budget |  |
| Primary Business Use | BU 15 - Flood Risk Management |
| Secondary Business Use | BU 01 - Water Supply and Quality |
| Tertiary Business Use | BU 10 - Geologic Assessment and Hazard Mitigation |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Nice to have |
| Tops of submerged structures, objects | Highly desirable |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Not required |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Nice to have |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Nice to have |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 1 sq mi - 49 sq mi |
| Smallest 3D features needed | Survey-level features |
| Description of smallest 3D features | Edges/corners of structures Identify houses in the path of dam failure. |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Highly desirable |
| Rivers and Streams |  |
| Less than 10 ft | Required |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Highly desirable |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Required |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL0 | QL0B |  |  |
| Update Frequency | As long as data is an accurate representation of the site, it is acceptable. | Only if the bathymetry has changed. |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI | Areas of rapid development would need more recent data. Channel changes would alter flow. |  |  |  |
| Acceptable Horizontal Error | Less than 20 cm | Less than 50 cm |  |  |
| Acceptable Vertical Error | Less than 5 cm | Less than 10 cm |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed | To MLLW |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Required |
| Hydro-enforcement | Required |
| Hydro-conditioning | Required |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Required |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have | Nice to have |  |  | Nice to have |  |
| Entire AOI under same environmental conditions | Nice to have | Nice to have |  |  | Nice to have |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required | Required |  |  | Required |  |
| DEM for entire AOI needs to be seamless | Required | Required |  |  | Required |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Whatever it takes to achieve seamlessness, including changes to the older, previously accepted dataset if it is proven to be less accurate than the newer | Whatever it takes to achieve seamlessness, including changes to the older, previously accepted dataset if it is proven to be less accurate than the newer |  |  | Whatever it takes to achieve seamlessness, including changes to the older, previously accepted dataset if it is proven to be less accurate than the newer |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Nice to have | Highly desirable |  |  |
| DTM | Required | Required |  |  |
| DEM | Required | Required |  |  |
| Raw point cloud data | Not required | Highly desirable |  |  |
| Classified point cloud | Not required | Highly desirable |  |  |
| Edited/cube XYZ |  | Highly desirable |  |  |
| Full waveform | Not required | Highly desirable |  |  |
| Bathymetric Attributed Grid (BAG) |  | Highly desirable |  |  |
| Breaklines required for standard hydro-flattening | Nice to have | Highly desirable |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Nice to have | Highly desirable |  |  |
| Ground control/ground truthing | Required | Required |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Required | Required |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Not required | Required |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Not required | Required |  |  |
| Land use/land cover | Required | Required |  |  |
| Wetlands | Not required | Required |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Required | Required |  |  |
| Bridges/culverts | Required | Required |  |  |
| Landmark features | Required | Required |  |  |
| Cultural resources | Nice to have | Not required |  |  |
| Coastal and riverine structures | Required | Required |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Primarily using Oregon Lidar Consortium (OLC) project data which conforms to QL0 (RMSEz ≤ 5 cm and aggregate nominal pulse density ≥8 points/square meter). | We collect bathymetric data for reservoirs when time permits. Data are included in permits from applicants. Independent verification is done for high hazard dams. |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  | Yes |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | None |  |  |
| Cost savings/cost reduction | Major | None |  |  |
| Cost avoidance | Major | None |  |  |
| Increased revenues | Major | None |  |  |
| Mission-driven performance improvements | None | None |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | None |  |  |
| Improved response or timeliness | None | None |  |  |
| Improved customer experience | I don't know | None |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major | None |  |  |
| Environmental | Major | None |  |  |
| Public safety, including life and property | None | None |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Major | Annual dollars saved/realized | $7,236 |  |  |  |  |  |  |
| Cost savings/cost reduction | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Cost avoidance | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Increased revenues | I don't know | Unable to provide |  | None |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Other operational benefits |  |  |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Other operational benefits description |  |  |  |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  | None |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | None |  |  | None |  |  |  |  |  |  |  |  |
| Environmental | None |  |  | None |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Major |  |  | Major |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours | Yes | Yes |  |  |
| Hillshades | Yes | Yes |  |  |
| Slope maps | Yes | Yes |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections | Yes | Yes |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids | Yes | Yes |  |  |
| Hydrologic Flow Accumulation Grids | Yes | Yes |  |  |
| Hydrologic networks (e.g. streams, lakes) | Yes | Yes |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) | Yes | Yes |  |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement | Yes |  |  |  |
| Rugosity/Surface Roughness | Yes |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 2 |
| Vertical accuracy | 1 |
| Update frequency | 3 |

# MCA Title: Sea Level Rise and Subsidence

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Modeling and mapping the effects of sea level rise or subsidence. Population and economic vulnerability assessments. Coastal inundation and infrastructure assessment. |
| MCA Title | Sea Level Rise and Subsidence |
| MCA ID | 60357 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 16 - Sea Level Rise and Subsidence |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Highly desirable |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Required |
| Ocean/sea bottom (>10 m deep) | Highly desirable |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Required |
| Rivers and Streams |  |
| Less than 10 ft | Required |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Highly desirable |
| Greater than 2,500 ft | Nice to have |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Required |
| ½ - 1 acre | Required |
| 1.1 – 2 acres | Required |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 HD | QL1B | QL2B |  |
| Update Frequency | 4-5 years | 4-5 years | 2-3 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | Up to 2 meters | Up to 5 meters |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 30 cm | Up to 40 cm |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Highly desirable |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Required |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have | Required | Required |  | Highly desirable | Required |
| Entire AOI under same environmental conditions | Highly desirable | Required | Required |  | Required | Required |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required | Nice to have | Required |  | Highly desirable | Required |
| DEM for entire AOI needs to be seamless | Required | Required | Required |  | Required | Required |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Highly desirable | Nice to have | Highly desirable |  |
| DTM | Required | Required | Highly desirable |  |
| DEM | Required | Required | Required |  |
| Raw point cloud data | Highly desirable | Highly desirable | Nice to have |  |
| Classified point cloud | Required | Nice to have | Highly desirable |  |
| Edited/cube XYZ |  | Not required | Not required |  |
| Full waveform | Not required | Not required | Not required |  |
| Bathymetric Attributed Grid (BAG) |  | Not required | Not required |  |
| Breaklines required for standard hydro-flattening | Highly desirable | Nice to have |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Required |  |
| Tide Predictions |  |  | Required |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Nice to have |  |
| Intensity imagery/sidescan imagery | Highly desirable | Nice to have | Highly desirable |  |
| Ground control/ground truthing | Required | Highly desirable | Required |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Highly desirable |  |
| Nautical and/or navigation charts |  |  | Highly desirable |  |
| Acoustic imagery of the seafloor |  |  | Nice to have |  |
| Aerial and/or satellite imagery | Required | Required | Required |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Not required |  |
| Bottom type |  |  | Nice to have |  |
| Submerged features |  |  | Nice to have |  |
| Subbottom characteristics |  |  | Nice to have |  |
| Geologic and seismic data | Nice to have | Nice to have | Nice to have |  |
| Water column properties - Physical |  |  | Nice to have |  |
| Water column properties - Chemical |  |  | Nice to have |  |
| Water column properties - Biological |  |  | Not required |  |
| Currents |  |  | Nice to have |  |
| Tide/wave heights |  |  | Required |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Highly desirable |  |
| Boundaries |  |  | Nice to have |  |
| Routes |  |  | Nice to have |  |
| Offshore cadastral |  |  | Not required |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Nice to have |  |
| Floating observation/navigation systems |  |  | Highly desirable |  |
| Shorelines – current, historic, change rates | Required | Required | Highly desirable |  |
| Land use/land cover | Required | Required | Highly desirable |  |
| Wetlands | Required | Required | Required |  |
| Estuaries |  |  | Required |  |
| Inland surface water features | Required | Required | Required |  |
| Bridges/culverts | Required | Required |  |  |
| Landmark features | Highly desirable | Required | Nice to have |  |
| Cultural resources | Nice to have | Nice to have | Nice to have |  |
| Coastal and riverine structures | Required | Required | Required |  |
| Overhead structures |  |  | Required |  |
| Lowest Floor Elevation of Buildings | Required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast | Yes | Yes | Yes |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  | Yes |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Moderate | Major |  |
| Cost savings/cost reduction | Major | Major | Major |  |
| Cost avoidance | Major | Major | Major |  |
| Increased revenues | None | Minor | Minor |  |
| Mission-driven performance improvements | Major | Major | Major |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | Major | Major |  |
| Improved response or timeliness | Moderate | Minor | Major |  |
| Improved customer experience | Moderate | Moderate | Major |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major | Moderate | Major |  |
| Environmental | Moderate | Moderate | Major |  |
| Public safety, including life and property | Major | Moderate | Major |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Cost savings/cost reduction | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Increased revenues | Moderate | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $5,109,129 |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Moderate |  |  | Major |  |  | Major |  |  |  |  |  |
| Environmental | Moderate |  |  | Major |  |  | Major |  |  |  |  |  |
| Public safety, including life and property | Moderate |  |  | Major |  |  | Minor |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Wildfire Management, Planning, and Response

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  |  |  |
| Geographic Area Requirements | One or more states, territories, or counties |  |  |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Determination of forest fuel and fire susceptibility. Fire behavior modeling to support wildfire suppression activities. Wildland/urban interface building identification. Post fire analysis to determine landslide prone areas. |
| MCA Title | Wildfire Management, Planning, and Response |
| MCA ID | 60358 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 17 - Wildfire Management, Planning, and Response |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Required |
| River/lake bottom | Not required |
| Nearshore elevation (<10 m deep) | Not required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 |  |  |  |
| Update Frequency | 4-5 years |  |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter |  |  |  |
| Acceptable Vertical Error | Up to 20 cm |  |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Nice to have |
| Hydro-enforcement | Nice to have |
| Hydro-conditioning | Nice to have |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Highly desirable |  |  |  |  |  |
| Entire AOI under same environmental conditions | Required |  |  |  |  |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required |  |  |  |  |  |
| DEM for entire AOI needs to be seamless | Required |  |  |  |  |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  |  |  |  |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required |  |  |  |
| DTM | Required |  |  |  |
| DEM | Required |  |  |  |
| Raw point cloud data | Nice to have |  |  |  |
| Classified point cloud | Highly desirable |  |  |  |
| Edited/cube XYZ |  |  |  |  |
| Full waveform | Not required |  |  |  |
| Bathymetric Attributed Grid (BAG) |  |  |  |  |
| Breaklines required for standard hydro-flattening | Nice to have |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Highly desirable |  |  |  |
| Ground control/ground truthing | Highly desirable |  |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Required |  |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Nice to have |  |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Not required |  |  |  |
| Land use/land cover | Highly desirable |  |  |  |
| Wetlands | Nice to have |  |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Highly desirable |  |  |  |
| Bridges/culverts | Nice to have |  |  |  |
| Landmark features | Nice to have |  |  |  |
| Cultural resources | Nice to have |  |  |  |
| Coastal and riverine structures | Nice to have |  |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Not required |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map | Yes |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Moderate |  |  |  |
| Cost savings/cost reduction | Major |  |  |  |
| Cost avoidance | Major |  |  |  |
| Increased revenues | Minor |  |  |  |
| Mission-driven performance improvements | Major |  |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major |  |  |  |
| Improved response or timeliness | Major |  |  |  |
| Improved customer experience | Major |  |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Major |  |  |  |
| Environmental | Major |  |  |  |
| Public safety, including life and property | Major |  |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Increased revenues | Minor | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $521,404 |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Major |  |  |  |  |  |  |  |  |  |  |  |
| Environmental | Major |  |  |  |  |  |  |  |  |  |  |  |
| Public safety, including life and property | Major |  |  |  |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Homeland Security, Law Enforcement, Disaster Response, and Emergency Management

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Infrastructure and border protection. Coastal search and rescue. Population dynamics. Emergency fuel supply and movement. Line of sight analysis in urban areas. Disaster response. Flood risk analysis resulting from acts of terrorism. |
| MCA Title | Homeland Security, Law Enforcement, Disaster Response, and Emergency Management |
| MCA ID | 60359 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | State of Oregon |
| Sub-Agency or Division |  |
| Organization Mission |  |
| Program Name |  |
| Total Annual Program Budget |  |
| Primary Business Use | BU 18 - Homeland Security, Law Enforcement, Disaster Response, and Emergency Management |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Highly desirable |
| Tops of submerged structures, objects | Nice to have |
| Tops of submerged vegetation | Nice to have |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Nice to have |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area |  |
| Smallest 3D features needed |  |
| Description of smallest 3D features |  |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Highly desirable |
| Rivers and Streams |  |
| Less than 10 ft | Highly desirable |
| 10 - 50 ft | Required |
| 51 - 100 ft | Required |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Highly desirable |
| ½ - 1 acre | Highly desirable |
| 1.1 – 2 acres | Highly desirable |
| 2.1 – 5 acres | Required |
| 5.1 – 10 acres | Required |
| Greater than 10 acres | Required |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL2 | QL0B | QL1B |  |
| Update Frequency | 4-5 years | 4-5 years | 6-10 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Up to 1 meter | Up to 2 meters | Up to 2 meters |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 30 cm | Up to 30 cm |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed |  |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Required |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Highly desirable |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Entire AOI under same environmental conditions | Required | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required | Required | Highly desirable |  | Highly desirable | Highly desirable |
| DEM for entire AOI needs to be seamless | Required | Required | Highly desirable |  | Required | Highly desirable |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Highly desirable | Highly desirable | Required |  |
| DTM | Required | Highly desirable | Highly desirable |  |
| DEM | Required | Required | Required |  |
| Raw point cloud data | Nice to have | Highly desirable | Highly desirable |  |
| Classified point cloud | Required | Required | Required |  |
| Edited/cube XYZ |  | Highly desirable | Highly desirable |  |
| Full waveform | Nice to have | Nice to have | Highly desirable |  |
| Bathymetric Attributed Grid (BAG) |  | Nice to have | Highly desirable |  |
| Breaklines required for standard hydro-flattening | Highly desirable | Highly desirable |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Highly desirable |  |
| Tide Predictions |  |  | Highly desirable |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Highly desirable |  |
| Intensity imagery/sidescan imagery | Nice to have | Nice to have | Highly desirable |  |
| Ground control/ground truthing | Required | Highly desirable | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Required |  |
| Nautical and/or navigation charts |  |  | Required |  |
| Acoustic imagery of the seafloor |  |  | Nice to have |  |
| Aerial and/or satellite imagery | Required | Required | Required |  |
| Underwater videography |  |  | Nice to have |  |
| Bottom texture |  |  | Nice to have |  |
| Bottom type |  |  | Nice to have |  |
| Submerged features |  |  | Highly desirable |  |
| Subbottom characteristics |  |  | Nice to have |  |
| Geologic and seismic data | Nice to have | Nice to have | Highly desirable |  |
| Water column properties - Physical |  |  | Nice to have |  |
| Water column properties - Chemical |  |  | Nice to have |  |
| Water column properties - Biological |  |  | Nice to have |  |
| Currents |  |  | Highly desirable |  |
| Tide/wave heights |  |  | Highly desirable |  |
| Sea ice conditions |  |  | Nice to have |  |
| Habitat distribution and classification |  |  | Nice to have |  |
| Boundaries |  |  | Highly desirable |  |
| Routes |  |  | Highly desirable |  |
| Offshore cadastral |  |  | Nice to have |  |
| Lease areas |  |  | Nice to have |  |
| Fixed obstructions |  |  | Highly desirable |  |
| Floating observation/navigation systems |  |  | Highly desirable |  |
| Shorelines – current, historic, change rates | Highly desirable | Required | Required |  |
| Land use/land cover | Highly desirable | Highly desirable | Highly desirable |  |
| Wetlands | Highly desirable | Highly desirable | Highly desirable |  |
| Estuaries |  |  | Highly desirable |  |
| Inland surface water features | Required | Required | Highly desirable |  |
| Bridges/culverts | Required | Required |  |  |
| Landmark features | Highly desirable | Highly desirable | Highly desirable |  |
| Cultural resources | Nice to have | Nice to have | Highly desirable |  |
| Coastal and riverine structures | Highly desirable | Required | Required |  |
| Overhead structures |  |  | Required |  |
| Lowest Floor Elevation of Buildings | Highly desirable |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used |  |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Major | Moderate |  |
| Cost savings/cost reduction | Major | Major | Major |  |
| Cost avoidance | Major | Moderate | Moderate |  |
| Increased revenues | None | None | None |  |
| Mission-driven performance improvements | Major | Moderate | Moderate |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | Moderate | Minor |  |
| Improved response or timeliness | Major | Major | Minor |  |
| Improved customer experience | Major | Moderate | Minor |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Moderate | Moderate | Minor |  |
| Environmental | Minor | Moderate | Minor |  |
| Public safety, including life and property | Major | Major | Moderate |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost savings/cost reduction | Major | Unable to provide |  | Major | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Increased revenues | Minor | Unable to provide |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $2,862,820 |  | Annual dollars saved/ realized | $806,243 |  | Annual dollars saved/ realized | $2,482 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Improved customer experience | Major | Unable to provide |  | Major | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $50,599 |  | Annual dollars saved/ realized | $51,439 |  | Annual dollars saved/ realized | $71 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Major |  |  | Major |  |  | Moderate |  |  |  |  |  |
| Environmental | Moderate |  |  | Moderate |  |  | Moderate |  |  |  |  |  |
| Public safety, including life and property | Major |  |  | Major |  |  | Major |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours |  |  |  |  |
| Hillshades |  |  |  |  |
| Slope maps |  |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps |  |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints |  |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage |  |
| Vertical accuracy |  |
| Update frequency |  |

# MCA Title: Transportation Engineering

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | One or more states, territories, or counties | One or more states, territories, or counties | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements |  |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | We have many mission critical activities throughout the transportation engineering process. They cover many of the business uses and MCAs. Stream channel analysis and mapping. Environmental Management. Land use and environmental planning. Geologic mapping and analysis. Identification of geomorphologic units. Landslide hazard mapping and assessment. Site protection and preservation planning. Discovery and analysis of Native American and other historical cultural sites and subsistence activities. Post-fire analysis to determine landslide-prone areas. Road and railroad route selection and maintenance. Slope analysis for autonomous cars. GPS navigation visualization. Bridge design and construction. Engineering and construction of dams, levees, dikes, reservoirs, and coastal structures. Shipyard and port construction. Water, sewer, or power line planning and vegetation analysis. Pump, drain, and well placement. Storm water modeling. Cut and fill analysis for earth-moving. Building site analysis. Road infrastructure. Infrastructure hardening or mitigation for climate change effects Land development and zoning. Delimitation of legal and other coastal boundaries, inland boundaries, and ordinary high water lines (OHWL). BU 20 – Marine and Riverine Navigation and Safety would be an additional Business Use. |
| MCA Title | Transportation Engineering |
| MCA ID | 21870 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Department of Transportation |
| Sub-Agency or Division |  |
| Organization Mission | To provide safe reliable transportation for those traveling within Oregon. |
| Program Name | The program within Oregon is the Oregon Lidar Consortium (OLC). ODOT uses data collected under OLC and specific contracts managed by ODOT. |
| Total Annual Program Budget |  |
| Primary Business Use | BU 19 - Land Navigation and Safety |
| Secondary Business Use | BU 22 - Infrastructure and Construction Management |
| Tertiary Business Use | BU 10 - Geologic Assessment and Hazard Mitigation |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Highly desirable |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Highly desirable |
| Tops of submerged vegetation | Highly desirable |
| Subcanopy of vegetation/understory | Highly desirable |
| River/lake bottom | Required |
| Nearshore elevation (<10 m deep) | Required |
| Sea surface | Nice to have |
| Ocean/sea bottom (>10 m deep) | Nice to have |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 50 sq mi - 999 sq mi (e.g. small county or County Equivalent, District of Columbia, etc.) |
| Smallest 3D features needed | Survey-level features |
| Description of smallest 3D features | Signs, bridge components, curb, striping, individual rocks. |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Required |
| Rivers and Streams |  |
| Less than 10 ft | Highly desirable |
| 10 - 50 ft | Highly desirable |
| 51 - 100 ft | Highly desirable |
| 101 - 500 ft | Required |
| 501 - 2,500 ft | Required |
| Greater than 2,500 ft | Required |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Nice to have |
| ½ - 1 acre | Nice to have |
| 1.1 – 2 acres | Nice to have |
| 2.1 – 5 acres | Nice to have |
| 5.1 – 10 acres | Nice to have |
| Greater than 10 acres | Nice to have |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 | QL0B | QL0B |  |
| Update Frequency | 2-3 years | 4-5 years | 4-5 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Less than 20 cm | Up to 1 meter | Up to 1 meter |  |
| Acceptable Vertical Error | Up to 10 cm | Up to 30 cm | Up to 30 cm |  |
| How far onshore needed |  |  | 1 kilometer inland |  |
| How far down the beach profile needed | To MLLW |  | None |  |
| Tide correction requirement |  |  | I don't know |  |
| Cross sections and/or transects meet needs |  | Partial | Partial |  |
| Cross section/transect requirement |  | Vertical accuracy would be up to 30 cm. Longitudinal sampling should be every 5-8 meters |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Nice to have |
| Hydro-enforcement | Nice to have |
| Hydro-conditioning | Nice to have |
| No Treatment | Nice to have |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Not required | Nice to have | Highly desirable |  | Required | Highly desirable |
| Entire AOI under same environmental conditions | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| DEM for entire AOI needs to be seamless | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | Whatever it takes to achieve seamlessness, including changes to the older, previously accepted dataset if it is proven to be less accurate than the newer | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |  | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level | Up to the required Total Vertical Uncertainty (TVU) at the 95% confidence level |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Highly desirable | Highly desirable | Highly desirable |  |
| DTM | Required | Required | Required |  |
| DEM | Required | Required | Required |  |
| Raw point cloud data | Required | Required | Required |  |
| Classified point cloud | Required | Required | Required |  |
| Edited/cube XYZ |  | Highly desirable | Highly desirable |  |
| Full waveform | Required | Highly desirable | Highly desirable |  |
| Bathymetric Attributed Grid (BAG) |  | Highly desirable | Required |  |
| Breaklines required for standard hydro-flattening | Highly desirable | Required |  |  |
| Additional breaklines for hydro-enforcement of culverts | Highly desirable |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Required |  |
| Tide Predictions |  |  | Required |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Required |  |
| Intensity imagery/sidescan imagery | Required | Required | Required |  |
| Ground control/ground truthing | Required | Required | Required |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Highly desirable |  |
| Nautical and/or navigation charts |  |  | Nice to have |  |
| Acoustic imagery of the seafloor |  |  | Nice to have |  |
| Aerial and/or satellite imagery | Highly desirable | Required | Highly desirable |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Highly desirable |  |
| Bottom type |  |  | Highly desirable |  |
| Submerged features |  |  | Highly desirable |  |
| Subbottom characteristics |  |  | Highly desirable |  |
| Geologic and seismic data | Required | Required | Highly desirable |  |
| Water column properties - Physical |  |  | Highly desirable |  |
| Water column properties - Chemical |  |  | Highly desirable |  |
| Water column properties - Biological |  |  | Highly desirable |  |
| Currents |  |  | Highly desirable |  |
| Tide/wave heights |  |  | Highly desirable |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Highly desirable |  |
| Boundaries |  |  | Highly desirable |  |
| Routes |  |  | Highly desirable |  |
| Offshore cadastral |  |  | Highly desirable |  |
| Lease areas |  |  | Highly desirable |  |
| Fixed obstructions |  |  | Highly desirable |  |
| Floating observation/navigation systems |  |  | Highly desirable |  |
| Shorelines – current, historic, change rates | Highly desirable | Required | Highly desirable |  |
| Land use/land cover | Required | Required | Highly desirable |  |
| Wetlands | Highly desirable | Required | Highly desirable |  |
| Estuaries |  |  | Highly desirable |  |
| Inland surface water features | Highly desirable | Required | Highly desirable |  |
| Bridges/culverts | Required | Required |  |  |
| Landmark features | Required | Highly desirable | Highly desirable |  |
| Cultural resources | Highly desirable | Highly desirable | Highly desirable |  |
| Coastal and riverine structures | Highly desirable | Required |  |  |
| Overhead structures |  |  | Highly desirable |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | A majority of the data we're using is QL1 collected between 2008 and 2017. | QL1 quality data | QL2 – whatever has been provided by USGS and NOAA |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  | Yes | Yes |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories | Yes |  |  |  |
| State repositories used | Oregon Lidar Consortium |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Moderate | Minor |  |
| Cost savings/cost reduction | Moderate | Minor | Minor |  |
| Cost avoidance | Moderate | Minor | Minor |  |
| Increased revenues | I don't know | None | None |  |
| Mission-driven performance improvements | Major | Minor | Minor |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Major | Moderate | Minor |  |
| Improved response or timeliness | Major | Moderate | Minor |  |
| Improved customer experience | Major | Moderate | Minor |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | I don't know | Minor | Minor |  |
| Environmental | Moderate | Moderate | Moderate |  |
| Public safety, including life and property | I don't know | Moderate | Moderate |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Time savings description | It's hard to truly say how much is saved, but the amount is significant. |  |  |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Moderate | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Cost savings/cost reduction description | Our lidar needs are typically project based and are scheduled around budget or an event in which case 3DEP data may or may not be useful. |  |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Increased revenues | None |  |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Mission-driven performance improvements description | Statewide data covering our transportation network could help the effectiveness of highway design and maintenance. Morphological modeling is important in Oregon and aerial lidar is a key component to making better decisions. |  |  |  |  |  |  |  |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $1,257,147 |  | Annual dollars saved/ realized | $53,622 |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Moderate | Unable to provide |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Improved response or timeliness | Major | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved customer experience | Moderate | Unable to provide |  | Minor | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $101,520 |  | Annual dollars saved/ realized | $554 |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Minor |  |  | I don't know |  |  | Minor |  |  |  |  |  |
| Environmental | Major |  |  | Major |  |  | Minor |  |  |  |  |  |
| Public safety, including life and property | Major |  |  | Major |  |  | Minor |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) | Yes | Yes | Yes |  |
| Contours | Yes | Yes | Yes |  |
| Hillshades | Yes | Yes | Yes |  |
| Slope maps | Yes |  |  |  |
| Aspect maps | Yes |  |  |  |
| Curvature maps | Yes |  |  |  |
| Cross sections |  | Yes | Yes |  |
| Height-Above-Ground maps | Yes | Yes |  |  |
| Viewshed maps | Yes | Yes | Yes |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  | Yes | Yes |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement | Yes |  |  |  |
| Rugosity/Surface Roughness | Yes |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 3 |
| Vertical accuracy | 1 |
| Update frequency | 2 |

# MCA Title: Engineering and Facilities Management

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required |  |  |  |
| Geographic Area Requirements | Respondent provided own shapefile, KML, or geodatabase |  |  |  |
| Sub Area Requirements | Oregon State Parks |  |  |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Engineering and facilities management. Site planning and infrastructure development, Engineering and facilities management. Site design and planning for infrastructure projects including utilities (power, water, wastewater), buildings, roads. |
| MCA Title | Engineering and Facilities Management |
| MCA ID | 21664 |
| Organization Type | State or U.S. Territorial government |
| Organization Name | Oregon Parks and Recreation Department |
| Sub-Agency or Division |  |
| Organization Mission | The mission of the Oregon Parks and Recreation Department is to provide and protect outstanding natural, scenic, cultural, historic and recreational sites for the enjoyment and education of present and future generations |
| Program Name | Park Development |
| Total Annual Program Budget |  |
| Primary Business Use | BU 22 - Infrastructure and Construction Management |
| Secondary Business Use |  |
| Tertiary Business Use |  |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Not required |
| Nearshore elevation (<10 m deep) | Not required |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | Less than 1 sq mi |
| Smallest 3D features needed | Survey-level features |
| Description of smallest 3D features | Curbs, manholes, electrical transformers, transformers |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | QL1 |  |  |  |
| Update Frequency | 4-5 years |  |  |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI |  |  |  |  |
| Acceptable Horizontal Error | Less than 20 cm |  |  |  |
| Acceptable Vertical Error | Up to 10 cm |  |  |  |
| How far onshore needed |  |  |  |  |
| How far down the beach profile needed | To MHW |  |  |  |
| Tide correction requirement |  |  |  |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Nice to have |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Not required |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  |  |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have |  |  |  |  |  |
| Entire AOI under same environmental conditions | Nice to have |  |  |  |  |  |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Required |  |  |  |  |  |
| DEM for entire AOI needs to be seamless | Required |  |  |  |  |  |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | I don't know |  |  |  |  |  |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required |  |  |  |
| DTM | Nice to have |  |  |  |
| DEM | Required |  |  |  |
| Raw point cloud data | Not required |  |  |  |
| Classified point cloud | Required |  |  |  |
| Edited/cube XYZ |  |  |  |  |
| Full waveform | Not required |  |  |  |
| Bathymetric Attributed Grid (BAG) |  |  |  |  |
| Breaklines required for standard hydro-flattening | Nice to have |  |  |  |
| Additional breaklines for hydro-enforcement of culverts | Nice to have |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  |  |  |
| Tide Predictions |  |  |  |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  |  |  |
| Intensity imagery/sidescan imagery | Required |  |  |  |
| Ground control/ground truthing | Highly desirable |  |  |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  |  |  |
| Nautical and/or navigation charts |  |  |  |  |
| Acoustic imagery of the seafloor |  |  |  |  |
| Aerial and/or satellite imagery | Highly desirable |  |  |  |
| Underwater videography |  |  |  |  |
| Bottom texture |  |  |  |  |
| Bottom type |  |  |  |  |
| Submerged features |  |  |  |  |
| Subbottom characteristics |  |  |  |  |
| Geologic and seismic data | Nice to have |  |  |  |
| Water column properties - Physical |  |  |  |  |
| Water column properties - Chemical |  |  |  |  |
| Water column properties - Biological |  |  |  |  |
| Currents |  |  |  |  |
| Tide/wave heights |  |  |  |  |
| Sea ice conditions |  |  |  |  |
| Habitat distribution and classification |  |  |  |  |
| Boundaries |  |  |  |  |
| Routes |  |  |  |  |
| Offshore cadastral |  |  |  |  |
| Lease areas |  |  |  |  |
| Fixed obstructions |  |  |  |  |
| Floating observation/navigation systems |  |  |  |  |
| Shorelines – current, historic, change rates | Nice to have |  |  |  |
| Land use/land cover | Nice to have |  |  |  |
| Wetlands | Nice to have |  |  |  |
| Estuaries |  |  |  |  |
| Inland surface water features | Nice to have |  |  |  |
| Bridges/culverts | Highly desirable |  |  |  |
| Landmark features | Nice to have |  |  |  |
| Cultural resources | Nice to have |  |  |  |
| Coastal and riverine structures | Nice to have |  |  |  |
| Overhead structures |  |  |  |  |
| Lowest Floor Elevation of Buildings | Nice to have |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | Existing QL1 lidar, photogrammetric mass points/surfaces (SfM), terrestrial laser scanning data, traditional topographic survey data, Dominant source is QL1 lidar. Secondary sources include mass points from photogrammetry (SfM), terrestrial laser scanning and traditional topographic surveying. |  |  |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast | Yes |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Other | Yes |  |  |  |
| Other description | Contracted, Cooperative acquisition, |  |  |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major |  |  |  |
| Cost savings/cost reduction | Major |  |  |  |
| Cost avoidance | Moderate |  |  |  |
| Increased revenues | None |  |  |  |
| Mission-driven performance improvements | Minor |  |  |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate |  |  |  |
| Improved response or timeliness | Moderate |  |  |  |
| Improved customer experience | None |  |  |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | None |  |  |  |
| Environmental | Moderate |  |  |  |
| Public safety, including life and property | Moderate |  |  |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Moderate | Annual dollars saved/realized | $21,708 |  |  |  |  |  |  |  |  |  |
| Time savings description | 40 hrs/mo reduced data developemnt time = 480 hrs/yr. 20 hrs/mo reduced field visit time = 240 hrs/yr. |  |  |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction | Moderate | Annual dollars saved/realized | $40,000 |  |  |  |  |  |  |  |  |  |
| Cost savings/cost reduction description | Acquisition reduction and/or reduced contracting costs. |  |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance | Minor | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance description | Cost reduction due to using inferior (lower accuracy) data. |  |  |  |  |  |  |  |  |  |  |  |
| Increased revenues | None |  |  |  |  |  |  |  |  |  |  |  |
| Mission-driven performance improvements | Minor | Unable to provide |  |  |  |  |  |  |  |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | None |  |  |  |  |  |  |  |  |  |  |  |
| Improved response or timeliness | None |  |  |  |  |  |  |  |  |  |  |  |
| Improved customer experience | None |  |  |  |  |  |  |  |  |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | None |  |  |  |  |  |  |  |  |  |  |  |
| Environmental | None |  |  |  |  |  |  |  |  |  |  |  |
| Public safety, including life and property | None |  |  |  |  |  |  |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours | Yes |  |  |  |
| Hillshades | Yes |  |  |  |
| Slope maps | Yes |  |  |  |
| Aspect maps |  |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  |  |  |  |
| Height-Above-Ground maps | Yes |  |  |  |
| Viewshed maps |  |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 3 |
| Vertical accuracy | 2 |
| Update frequency | 1 |

# MCA Title: Regional GIS and Data Services

| MCA Area of Interest | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Data Type | Required | Required | Required |  |
| Geographic Area Requirements | Area split by varying quality level or update frequency | Custom description | Nearshore areas along the coast off one or more states, territories, or counties (including Great Lakes states) |  |
| Sub Area Requirements | HUC8s that cover Oregon Council of Governments | HUC8s that cover Oregon Council of Governments | Lane, Lincoln, Coos, and Douglas Counties, OR |  |

| MCA Description | Response |
| --- | --- |
| Mission Critical Activity | Oregon's Councils of Governments provide data acquisition and development, data analysis, and planning services for a diverse set of agencies. The mission critical activities fall primarily into BU 23 (Urban and Regional Planning) and 25 (Real Estate) and related cases. Individual agencies provide data back as well. Elevation data are needed for ecosystem management for electric utilities, drinking water, data sharing consortium, and real estate/property management. Bathymetry in reservoirs is used for river dynamics. |
| MCA Title | Regional GIS and Data Services |
| MCA ID | 21615 |
| Organization Type | Regional, County, City, or other local government |
| Organization Name | Oregon Councils of Government |
| Sub-Agency or Division |  |
| Organization Mission | By direction of member agencies, LCOG identifies future trends and coordinates, promotes, plans and provides efficient intergovernmental and specialized services to local governments, other agencies and citizens. |
| Program Name | Geographic Information Systems and Data Services |
| Total Annual Program Budget |  |
| Primary Business Use | BU 23 - Urban and Regional Planning |
| Secondary Business Use | BU 25 - Real Estate, Banking, Mortgage, and Insurance |
| Tertiary Business Use | BU 02 - Riverine Ecosystem Management |

| What Needs to be Measured in 3D | Response |
| --- | --- |
| Bare earth ground | Required |
| Tops of buildings, structures, objects | Required |
| Tops of vegetation | Required |
| Tops of submerged structures, objects | Not required |
| Tops of submerged vegetation | Not required |
| Subcanopy of vegetation/understory | Nice to have |
| River/lake bottom | Highly desirable |
| Nearshore elevation (<10 m deep) | Highly desirable |
| Sea surface | Not required |
| Ocean/sea bottom (>10 m deep) | Not required |

| General Geographic Area and Size |  |
| --- | --- |
| Average geographic extent of day-to-day area | 1,000 sq mi - 24,999 sq mi (e.g. large county, small state, intrastate region [e.g. a multi-county region such as the San Francisco Bay Area, Tri-County Council, etc.]) |
| Smallest 3D features needed | Survey-level features |
| Description of smallest 3D features | Our municipal and utility partners are interested in curbs, utility access (man holes), power lines, and related structures on the fine end of scale. |

| Inland Bathy Feature Size Requirements | Response |
| --- | --- |
| Navigable channels (as defined by USACE) | Highly desirable |
| Rivers and Streams |  |
| Less than 10 ft | Not required |
| 10 - 50 ft | Highly desirable |
| 51 - 100 ft | Highly desirable |
| 101 - 500 ft | Highly desirable |
| 501 - 2,500 ft | Highly desirable |
| Greater than 2,500 ft | Highly desirable |
| Waterbodies (Reservoirs, lakes, ponds) |  |
| Less than ½ acre | Nice to have |
| ½ - 1 acre | Nice to have |
| 1.1 – 2 acres | Highly desirable |
| 2.1 – 5 acres | Highly desirable |
| 5.1 – 10 acres | Highly desirable |
| Greater than 10 acres | Highly desirable |

| Requirements | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Quality Level/IHO Order | (a) QL1 HD (b) QL2 | QL1B | QL1B |  |
| Update Frequency | (a) 2 years (b) 2-4 years | 2-3 years | 2-3 years |  |
| Event type(s) |  |  |  |  |
| Quality Level and/or update frequency variability across AOI | Urban areas need greater frequency and higher standards than rural areas, in general. Greater metro area of Eugene, Coburg, and Springfield at QL1 HD every 2 years, elsewhere QL2 every 2-4 years. |  | Lane County coastline |  |
| Acceptable Horizontal Error | Up to 30 cm | Up to 1 meter | Up to 1 meter |  |
| Acceptable Vertical Error | Up to 20 cm | Up to 30 cm | Up to 30 cm |  |
| How far onshore needed |  |  | To Mean Lower Low Water (MLLW) |  |
| How far down the beach profile needed | Not applicable |  | None |  |
| Tide correction requirement |  |  | I don't know |  |
| Cross sections and/or transects meet needs |  |  |  |  |
| Cross section/transect requirement |  |  |  |  |

| Hydrologic Processing Required | Response |
| --- | --- |
| Hydro-flattening | Highly desirable |
| Hydro-enforcement | Highly desirable |
| Hydro-conditioning | Highly desirable |
| No Treatment | Not required |

| Seamlessness Requirements | Within Inland Topo Data | Within Inland Bathy Data | Within Nearshore Data | Within Offshore Data | Between Inland Topo and Inland Bathy | Between Topo, Topobathy, and/or Bathy |
| --- | --- | --- | --- | --- | --- | --- |
| Importance of Seamlessness |  |  |  |  | Highly desirable |  |
| Temporal Seamlessness |  |  |  |  |  |  |
| Entire AOI in same acquisition season | Nice to have | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Entire AOI under same environmental conditions | Nice to have | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Spatial Seamlessness |  |  |  |  |  |  |
| Point cloud for entire AOI needs to be seamless | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| DEM for entire AOI needs to be seamless | Highly desirable | Highly desirable | Highly desirable |  | Highly desirable | Highly desirable |
| Amount of acceptable vertical manipulation to achieve spatial seamlessness | I don't know | I don't know | I don't know |  | I don't know | I don't know |

| Importance of 3D Products | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| DSM | Required | Highly desirable | Required |  |
| DTM | Required | Highly desirable | Required |  |
| DEM | Required | Highly desirable | Required |  |
| Raw point cloud data | Required | Nice to have | Highly desirable |  |
| Classified point cloud | Highly desirable | Highly desirable | Highly desirable |  |
| Edited/cube XYZ |  | Not required | Nice to have |  |
| Full waveform | Nice to have | Not required | Not required |  |
| Bathymetric Attributed Grid (BAG) |  | Not required | Not required |  |
| Breaklines required for standard hydro-flattening | Not required | Not required |  |  |
| Additional breaklines for hydro-enforcement of culverts | Not required |  |  |  |
| National Vertical Datum Transformation Tool (V-Datum) |  |  | Nice to have |  |
| Tide Predictions |  |  | Nice to have |  |
| Tidal Constituent And Residual Interpolation (TCARI) |  |  | Not required |  |
| Intensity imagery/sidescan imagery | Highly desirable | Nice to have | Not required |  |
| Ground control/ground truthing | Highly desirable | Highly desirable | Highly desirable |  |

| Importance of Integration with Other Datasets | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Hydrographic survey data |  |  | Not required |  |
| Nautical and/or navigation charts |  |  | Not required |  |
| Acoustic imagery of the seafloor |  |  | Not required |  |
| Aerial and/or satellite imagery | Required | Highly desirable | Highly desirable |  |
| Underwater videography |  |  | Not required |  |
| Bottom texture |  |  | Not required |  |
| Bottom type |  |  | Not required |  |
| Submerged features |  |  | Not required |  |
| Subbottom characteristics |  |  | Not required |  |
| Geologic and seismic data | Highly desirable | Highly desirable | Not required |  |
| Water column properties - Physical |  |  | Not required |  |
| Water column properties - Chemical |  |  | Not required |  |
| Water column properties - Biological |  |  | Not required |  |
| Currents |  |  | Not required |  |
| Tide/wave heights |  |  | Not required |  |
| Sea ice conditions |  |  | Not required |  |
| Habitat distribution and classification |  |  | Not required |  |
| Boundaries |  |  | Not required |  |
| Routes |  |  | Not required |  |
| Offshore cadastral |  |  | Not required |  |
| Lease areas |  |  | Not required |  |
| Fixed obstructions |  |  | Not required |  |
| Floating observation/navigation systems |  |  | Not required |  |
| Shorelines – current, historic, change rates | Highly desirable | Highly desirable | Not required |  |
| Land use/land cover | Highly desirable | Highly desirable | Not required |  |
| Wetlands | Highly desirable | Highly desirable | Not required |  |
| Estuaries |  |  | Not required |  |
| Inland surface water features | Required | Highly desirable | Not required |  |
| Bridges/culverts | Required | Highly desirable |  |  |
| Landmark features | Highly desirable | Nice to have | Not required |  |
| Cultural resources | Nice to have | Nice to have | Not required |  |
| Coastal and riverine structures | Nice to have | Highly desirable | Not required |  |
| Overhead structures |  |  | Not required |  |
| Lowest Floor Elevation of Buildings | Highly desirable |  |  |  |

| Benefits of Currently Used Elevation Data | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Current dataset used | We have a variety of dates available from old USGS to 2016 Lidar information (point cloud and derivative products) through Oregon DOGAMI. | We have limited amounts of bathymetric data on the McKenzie and Middle Fork Willamette. | None at this time. |  |
| Where current elevation data are accessed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| National Map |  |  |  |  |
| Digital Coast |  |  |  |  |
| NCEI |  |  |  |  |
| Open Topography |  |  |  |  |
| NOAA nautical charts |  |  |  |  |
| USACE navigation charts |  |  |  |  |
| USGS Inland Waters server |  |  |  |  |
| USGS data series |  |  |  |  |
| Marine Minerals Program GIS |  |  |  |  |
| State Repositories |  |  |  |  |
| State repositories used |  |  |  |  |
| Other | Yes | Yes | Yes |  |
| Other description | Oregon Lidar Consortium | Contracted flights | None |  |
| Data that meet my needs are not available |  |  |  |  |
| Current Operational Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Time savings | Major | Minor | I don't know |  |
| Cost savings/cost reduction | Major | Minor | I don't know |  |
| Cost avoidance | Moderate | Minor | I don't know |  |
| Increased revenues | Moderate | Minor | I don't know |  |
| Mission-driven performance improvements | Major | Minor | I don't know |  |
| Current Customer Service Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Value added to products or services | Moderate | Minor | I don't know |  |
| Improved response or timeliness | Minor | None | I don't know |  |
| Improved customer experience | Moderate | Minor | I don't know |  |
| Current Societal Benefits | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| Education or outreach | Moderate | I don't know | I don't know |  |
| Environmental | Major | Minor | I don't know |  |
| Public safety, including life and property | Moderate | Minor | I don't know |  |

| Future Benefits if Elevation Data Requirements Are Met | Inland Topo |  |  | Inland Bathy |  |  | Nearshore Bathy |  |  | Offshore Bathy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Future Operational Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Time savings | Major | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Cost savings/cost reduction | Moderate | Annual dollars saved/realized | $5,927,432 | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Cost savings/cost reduction description | $490/square mile acquisition costs saved * COG square miles divided by 2.5 for update frequency. |  |  |  |  |  |  |  |  |  |  |  |
| Cost avoidance | Major | Unable to provide |  | Moderate | Unable to provide |  | Major | Unable to provide |  |  |  |  |
| Increased revenues | None |  |  | None |  |  | None |  |  |  |  |  |
| Mission-driven performance improvements | Major | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Potential future annual operational benefits added during validation |  | Annual dollars saved/ realized | $8,376,760 |  | Annual dollars saved/ realized | $582,584 |  | Annual dollars saved/ realized | $25,870 |  |  |  |
| Future Customer Service Benefits | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount | Benefits | Units | Amount |
| Value added to products or services | Minor | Unable to provide |  | Moderate | Unable to provide |  | Minor | Unable to provide |  |  |  |  |
| Improved response or timeliness | Moderate | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Improved customer experience | Moderate | Unable to provide |  | Moderate | Unable to provide |  | Moderate | Unable to provide |  |  |  |  |
| Potential future annual customer service benefits added during validation |  | Annual dollars saved/ realized | $1,651,425 |  | Annual dollars saved/ realized | $737,019 |  | Annual dollars saved/ realized | $27,291 |  |  |  |
| Societal Benefits | Benefits |  |  | Benefits |  |  | Benefits |  |  | Benefits |  |  |
| Education or outreach | Minor |  |  | Minor |  |  | I don't know |  |  |  |  |  |
| Environmental | Moderate |  |  | Moderate |  |  | I don't know |  |  |  |  |  |
| Public safety, including life and property | I don't know |  |  | Moderate |  |  | I don't know |  |  |  |  |  |

| 3D Derivatives Needed | Inland Topo | Inland Bathy | Nearshore Bathy | Offshore Bathy |
| --- | --- | --- | --- | --- |
| Triangulated Irregular Network (TIN) |  |  |  |  |
| Contours | Yes | Yes |  |  |
| Hillshades | Yes |  |  |  |
| Slope maps | Yes |  |  |  |
| Aspect maps | Yes |  |  |  |
| Curvature maps |  |  |  |  |
| Cross sections |  | Yes | Yes |  |
| Height-Above-Ground maps | Yes |  |  |  |
| Viewshed maps | Yes |  |  |  |
| Hydrologic Flow Direction Grids |  |  |  |  |
| Hydrologic Flow Accumulation Grids |  |  |  |  |
| Hydrologic networks (e.g. streams, lakes) |  |  |  |  |
| Hydrologic Units (Watershed Boundaries) (e.g. surface water drainage to a point) |  |  |  |  |
| Building footprints | Yes |  |  |  |
| Breaklines for road edge-of-pavement |  |  |  |  |
| Rugosity/Surface Roughness |  |  |  |  |

| Importance of 3D Data Requirements | Response |
| --- | --- |
| Geographic coverage | 3 |
| Vertical accuracy | 1 |
| Update frequency | 2 |