Oregon EPSG Codes Background

Sheri Schneider, USGS Geospatial Liaison

The European Petroleum Survey Group (EPSG) Geodetic Parameter Dataset is a structured dataset of Coordinate Reference Systems and Coordinate Transformations, accessible through a data registry.

Nearly all GIS software packages include the EPSG code as one way to identify projection information and some rely on it exclusively. The xml file that accompanies a downloaded Oregon las file from Earth Explorer always contains an EPSG code for the projection information.

There are 6 EPSG codes for Oregon Lambert Projected Coordinate Reference System.

2991 NAD83

2992 NAD83(ft)

2993 NAD83(HARN)

2994 NAD83/(HARN)(ft)

3643 NAD83(NSRS2007)

3644 NAD83(NSRS2007)(ft)

Existing Oregon lidar data, for the most part, is in Oregon Lambert, International Feet, NAD83 (CORS96). New Oregon lidar data will be in Oregon Lambert, International Feet, NAD83 (CORS2011). Two new codes are needed so that there is an EPSG code that is accurate for the Oregon lidar data.

These can be submitted to the EPSG Registry as a Change Request -

- Change requests are accepted from any interested party.

- The change request must include the information tabulated in Guidance Note 7-1 annex A.

- If within scope, the request is allocated a change request number and reviewed by the Subcommittee.

- The proposer may be required to provide supplementary information.

- The Subcommittee aims to process all requests within one release cycle of the dataset. Where new data is added, the code(s) for this are made public only at the time of the next full dataset release.

# Annex A – Essential Elements for Description of CRS and Transformation

In the table below, elements that are essential for the unambiguous description of coordinate reference system or transformation are indicated by a 0.

| Element |  | Geodetic CRS | Projected CRS | Engineering CRS | Vertical CRS | Transformation |
| --- | --- | --- | --- | --- | --- | --- |
| MS Access Table Name | Entity Name |  |  |  |  |  |
| Coordinate Reference System | COORD_REF_SYS_CODE | 0 | 0 | 0 | 0 |  |
| Coordinate Reference System | COORD_REF_SYS_NAME | 0 | 0 | 0 | 0 |  |
| Coordinate Axis Name | COORD_AXIS_NAME | 0 Note 1 | 0 Note 1 | 0 Note 1 | 0 |  |
| Coordinate Axis | COORD_AXIS_ORIENTATION | 0 Note 1 | 0 Note 1 | 0 Note 1 | 0 |  |
| Coordinate Axis | ORDER | 0 Note 1 | 0 Note 1 | 0 Note 1 |  |  |
| Unit of Measure | Axis unit name | 0 Note 1 | 0 Note 1 | 0 Note 1 | 0 |  |
| Datum | DATUM_NAME | 0 | 0 | 0 | 0 |  |
| Ellipsoid | ELLIPSOID_NAME | 0 | 0 |  |  |  |
| Ellipsoid | SEMI_MAJOR_AXIS | 0 | 0 |  |  |  |
| Unit of Measure | Ellipsoid sma unit name | 0 | 0 |  |  |  |
| Ellipsoid | INV_FLATTENING or (1/f calculated from a and b) | 0 Note 2 | 0 Note 2 |  |  |  |
| Prime Meridian | PRIME_MERID_NAME | 0 | 0 |  |  |  |
| Coordinate_Operation | COORD_OP_NAME |  | 0 |  |  | 0 |
| Coordinate_Operation Method | COORD_OP_METHOD_NAME |  | 0 |  |  | 0 |
| Coordinate_Operation Method | REVERSE_OP |  |  |  |  | 0 |
| Coordinate_Operation | UOM_CODE_SOURCE_COORD_DIFF |  |  |  |  | 0 |
| Coordinate_Operation | UOM_CODE_TARGET_COORD_DIFF |  |  |  |  | 0 |
| Coordinate_Operation Parameter | PARAMETER_NAME |  | 0 Note 3 |  |  | 0 Note 4 |
| Coordinate_Operation Parameter Usage | PARAM_SIGN_REVERSAL |  |  |  |  | 0 Note 4 |
| Coordinate_Operation Parameter Value | PARAMETER_VALUE |  | 0 Note 3 |  |  | 0 Notes 4,5 |
| Unit of Measure | Parameter unit name |  | 0 Note 3 |  |  | 0 Notes 4,5 |
| Coordinate_Operation Parameter Value | PARAM_VALUE_FILE_REF |  |  |  |  | 0 Notes 4,5 |

Notes

1. These attributes are required for each axis.

2. When the dataset provides only ellipsoid parameters a and b, 1/f may be calculated from a/(a-b).

3. These attributes are required for each projection parameter.

4. These attributes are required for each transformation parameter.

5. Either Parameter Value with unit name or Parameter Value File Reference is required.